# User manual for PSU00001 test system

Subject: Vision product testers

Author: Jonathan Vanden Berk version; Date; Reviewed by; Approved by. 1v0; 2026-01-14; ...; ....

Revision history

| Version | Date | Revisions |
| --- | --- | --- |
| 1v0 | 2026-01-14 | First version Reviewed by:… Approved by:… |

Table of Contents

Revision history	2

Table of Contents	3

List of Figures	4

List of Tables	5

1	Heading 1	6

1.1	Heading 2	6

1.1.1	Heading 3	6

2	Figures	7

3	Tables	8

Bibliography	10

List of Acronyms	11

Appendix A	Style – Appendix 1	12

A.1	Style appendix 2	12

# Introduction and Safety

- Product description

  - Short description of the product

- ESD

- Thermostreamer

  - Moving head when pressure is released

  - Icing

- The ThermoStream head will drop automatically when pressure is cut off.

- Always wear an ESD wrist strap.

- /!\ When switching between test modes (ex: FT to CHAR, etc.) or changing the database name or location:

  - Close the GUI window (to terminate the script).

  - Relaunch the script and THEN choose your test mode.

# Test system

## Hardware configuration overview

- Refer to tester report?

- Indicate where pin 1 is and that there is a risk to place the component wrongly

- Socket board vs module testing

## Software interface

- How to connect to the test computer

- GUI

  - Guardbands

- GIT, make sure to be inline with latest modifications, perform a GIT pull if necessary.

## Environmental Requirements

- Temperature

  - Either ambient testing or thermostreamer

- Cleanliness

  - Check for dirt or fingerprints on the DUT’s and socket and clean if needed according the procedure described in the PM plan

# Inputs for batch testing

Number of parts to be tested

  - Part ID

Test stage

Guard bands

Temperature points

Check SW binning to HW binning table in test plan specification

Location of storage

Should we create a work order template?

Reference sample(s) to be used

# Outputs for batch testing

- Create unique .db file per day

- Copy .db file and log files to sharepoint polaris WP.9 project

# Procedure for batch testing

Todo:

- Golden samples first, zonder GB???

## Calibration of instruments check

- Verify that all instruments are calibrated in this document: List_Of_Equipment

- Report any non-calibrated equipment to tester owner, if non exist continue with the next step.

## Initialization and power-on of test system

- When temperature testing is required:

  - Open the pressure valve for the Thermo-streamer

  - Place the measurement PCB on the brown insulation foam. Remove the spacers first.

  - Connect the ethernet cable of the thermo-streamer to switch in test rack

- Plug in the socket PCB or the module DUT into the measurement PCB.

- Check the cabling (POWER and DATA):

  - Power (230V) is connected to all instruments (such as PSU, PC, SCOPE…).

  - Ethernet cables are all properly plugged into each instrument.

  - The rack PC is connected via an Ethernet cable to the network, a power source, keyboard/mouse and a display.

  - The connections towards the instruments are described in this document: DCDC Setup Report

  - The STM32 board is correctly wired (SCOPE connection, ethernet, jumpers, …) and makes good contact with the measurement board.

- Power ON:

  - the Thermo-streamer (it will cycle for a couple of minutes for its self-calibration routine) if temperature

  - Rack pc

- Log-in test pc

  - Password: MAGICXX4XX!

- Route the thermocouple probe into the socket, making sure it does not touch the chip (but the tip of the probe must be IN the socket when closed).

- Then cover the whole thing with the second foam layer (the one with the square hole to let the socket go through)

- Set up the ThermoStream nozzle position and height:

  - Two positions are available: TOP and BOTTOM. You can switch between both by pressing the Head button.

  - In the BOTTOM position, ensure a good seal with the foam and NO contact between the socket and the nozzle. You can use shims between the pcb and table if needed.

  - Adjust the height (if needed, for a better seal) using the Stand buttons.

  - Adjust the X and Y position manually (by locking/unlocking the main fixation points of the nozzle arm).

  - The nozzle should be 1–2 mm above the socket cooling fins.

- Power ON all instruments in the rack.

  - Perform “experimate net scan” before enabling the equipment. Only the thermo-streamer should be found at this step.

  - Power on all instruments

  - Disable CH1 on the power supply (it will turn ON automatically when the script runs) and turn ON the output for CH2. (output and channel 2)

- Reset the MCU (simply press the RST button on the STM32 board).

- Perform “experimate net scan” and check that all instruments are found. If not see chapter 6.

- Wait for 60 minutes before starting the measurements to let the instruments warm-up.

## Executing the test on the first part of the batch

- On the rack PC, run the GUI script: “tests/run_gui”

- Fill in all configuration field in the GUI

  - Select the correct test stage: e.g.: CHAR or TRIM or FT, …

  - Change the number of repetitions to e.g. 30 if you want to perform characterization of the test system. For production testing the number of repetitions should be 1!

  - When using the thermo-streamer select the Temperature Control checkbox

  - Select the Add Guard-Bands checkbox for production runs (a must for FT and TRIM test stages), only for GRR runs this box should be no selected.

  - The database path can be kept default most of the time

  - Enter the correct part ID  (unique for each chip!)

- Load the test socket with a DUT. Since this is the first part of the batch this is likely to be a reference sample.

- Click RUN to start the test (if everything is working, the nozzle of the Thermostreamer should go down and start cooling).

## Executing the test of other parts of the batch

- When the test on the previous part is finished all the settings are kept expect the Part-ID field is cleared. Once the previous test is finished, the Thermostreamer head will automatically lift.

- Open the socket, remove the tested chip, store it at the correct HW bin 5.7, and insert a new one.

- Close the socket.

- Enter the new Part ID in the Test Application Window (GUI).

- Click RUN again. There is no need to relaunch the script, you just need to change the Part ID.

## Interpreting results

If the software bin is 1, the part is within the defined test limits for all test parameters. These parts should be stored in the hardware bin Pass trays. Attention should be given in the order of populating the PASS tray, fill in this order: top-left → top-right, row by row, ending bottom-right.

CHAR run incl OTP test stage: will fail on v_ptat parameter since temperature is swept over full range: bin 300 is expected

## Retest

Parts that needed to be retested will fall into bin 100 up to 199. These part must be retested using the correct Part ID number at the end of the batch. One retest is allowed per part.

## Hardware Binning

# Troubleshooting

- If the script gives you an error: ensure ALL instruments (including the ThermoStream and STM32) are visible on the local network:

  - To check that, In the PyCharm console, type: experimate net scan

  - You should be able to see every instrument in the console.

  - IF NOT, power cycle the different instruments and rescan. Note: the uC needs a reset after the reboot of the PSU.

- If no air is coming from the ThermoStream (and the valve is open), there may be a compressor issue: contact Rik.

Idm@F4E #