# Test plan MAG VIS EVK Modules

Subject: Test Plan: MAG VIS EVK Modules - Electrical and Functional Verification

Author: Seppe Servotte version; Date; Reviewed; Approved by. 1v0; 2025-04-23.

Revision history

| Version | Date | Revisions |
| --- | --- | --- |
| 1v0 | 2025-04-23 | First version Reviewed by: Approved by: |

Table of Contents

Revision history	2

Table of Contents	3

List of Figures	5

List of Tables	6

1	Document purpose and scope	7

2	Test strategy	8

2.1	Test conditions	8

2.2	Test stages	8

2.2.1	Visual inspection (VI)	9

2.2.2	Functional verification (VER)	9

2.3	Test flow	9

2.4	Test sequence	10

2.5	Test equipment	11

2.6	Statistical analysis	11

2.6.1	Bin groups	11

3	Measurement test setup	12

3.1	Test hardware	12

3.1.1	Overview	12

3.1.2	DUT information	14

3.1.3	Pin load requirements	14

3.1.4	Power supplies	15

3.2	Test software	15

3.2.1	Overview	15

3.2.2	Park state routine	15

3.2.3	Power cycle routines	15

4	Test procedures	17

4.1	T0001: Visual inspection (VI)	17

4.2	T0002: Power consumption	18

4.3	T0003: CXP Core functionality and video data check	19

4.4	T0004: IMG Core functionality and video data check	21

4.5	T0005: IMG Trigger mode	22

4.6	T0006: MAG-DRV Capabilities (only for MAG-VIS1000x modules)	24

4.7	T0007: MAG-DRV Motor drive capabilities (only for MAG-VIS1002x modules)	26

List of Acronyms	28

Appendix A	MAG-CXP IC Iniatilze SPI and Clock code snippet	30

Appendix B	MAG-DRV IC Reset and clear all	31

List of Figures

Figure 1: MAG VIS EVK Module production test flow diagram	10

Figure 2: Measurement setup overview	12

Figure 3: MAG-VIS1000X Measurement setup overview	13

Figure 4: MAG-VIS1002X Measurement setup overview	13

List of Tables

Table 1: Supply voltage conditions	8

Table 2: Imager modes	8

Table 3: Test stages	8

Table 4: Test sequence per test stage	10

Table 5: Measurement Equipment overview	11

Table 6: Bin table	11

Table 7: Pin list vs equipment matrix	14

Table 8: Power supplies	15

Table 9: Park state pin biasing	15

Table 10: Visual inspection overview	17

Table 11: Power consumption overview	18

Table 12: CXP Core functionality and video data check overview	19

Table 13: IMG Core functionality and video data check overview	21

Table 14: IMG Trigger mode overview	22

Table 15: MAG-DRV Capabilities overview	24

Table 16: MAG-DRV Motor drive capabilities overview	26

# Document purpose and scope

This document is part of the MAG-VIS100xx-N Modules project and defines the test strategy, test setup and the detailed test procedures to perform electrical and functional verification. Radiation testing is out of scope of this test plan, these test are not done on individual MAG-VIS modules.

All the information in this document is intended to describe the verification tests that will be done on the products mentioned above. Any major deviations from this plan will be discussed prior or at latest during the test readiness review (TRR) meeting with the customer. Major changes will be listed in the revision history. Major changes can be (list is not exclusive):

Number of DUT’s

Test conditions

Hardware/equipment changes

Test facility changes

Addition or removal of tests

Any update to a test procedure that changes the test coverage or testing strategy

- Minor changes are kept in the revision history but not explicitly informed to the customer. These changes are limited to:

- Small changes in a test procedure, e.g. additional SPI commands, increasing wait times

- Correction of typo’s

Any changes (major and minor) after the TRR meeting will be discussed with the customer.

# Test strategy

This chapter describes the test methods used for the verification and validation of the MAG-VIS100xx-N modules. The test strategy aims at maximal test coverage of the product.

Functional testing and electrical parametric testing of the test structures is used to verify the performance.

## Test conditions

During functional verification testing the temperature and supply voltage of the DUTs will be varied to validate the module specifications over the defined operating conditions. The conditions listed in this section are the minimal set of conditions that will be tested. If additional test conditions are needed to validate certain parameters, then they are explicitly mentioned in the test procedure.

Temperature conditions are mentioned in this document with following codes:

- RT = room temperature = 25°C +/- 5°C

- CT = cold temperature = 0°C +/- 5°C

- HT = hot temperature = 55°C +/- 5°C

Supply voltage conditions are defined in the table below:

Table 1: Supply voltage conditions

| Supply level | Minimal | Nominal | Maximal | Comments |
| --- | --- | --- | --- | --- |
| VDD_IN | 6V | 9V | 12V |  |

The different imager modes are defined in the table below. The default imager mode is FHD10 unless otherwise specified.

Table 2: Imager modes

| Imager mode | FPS | Image size height | Image size width |
| --- | --- | --- | --- |
| FHD10 | 10 | 1080 | 1920 |
| FHD30 | 30 | 1080 | 1920 |
| VGA50 | 50 | 480 | 640 |

## Test stages

Table 3 outlines all the test stages required to validate the performance of the test vehicle, while section 2.3 details the sequence in which these test stages fit into the overall test flow.

Table 3: Test stages

| Name | Code | Temperature | Supply | Other conditions | Number of parts | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Production and screening tests |  |  |  |  |  |  |
| Visual inspection | VI | / | / | / | All |  |
| Final test | FT | RT, TBD | Vnom, TBD | / | All |  |
| Device verification tests |  |  |  |  |  |  |
| Functional verification | VER | CT, RT, HT | Vnom, Vmin, Vmax | / | All | Any test conditions (temperature, supply voltage, etc.) deviating from the defaults listed in this table will be explicitly defined within the respective test procedure. |

The following sections provide further details on the different test stages.

### Visual inspection (VI)

The purpose of the external visual inspection test stage is to assess the physical condition of the Device Under Test (DUT) before testing. This stage helps identify any visible defects, such as cracks, issues with pins, delamination, or other mechanical damages that may have occurred during handling, or transportation. By conducting a thorough visual examination, potential issues can be detected early, preventing further testing on compromised components and ensuring the reliability of the test results.

### Functional verification (VER)

The functional verification stage is intended to confirm that the product is in compliance (functional and/or parametric) against its documentation for normal use-case(s).

## Test flow

This section defines the test flow that every part will follow.

Approved devices are defined as PCBs that have undergone and successfully passed all required verifications outlined in their dedicated, product-specific test plan. PCBs achieving this status are formally designated as fit for inclusion within an EVK assembly.

The removal of the imager's glass protection is performed only when the module assembly includes a dedicated housing and mounted lens (see Figure 1, left flow). This step is omitted if the housing is not included (see Figure 1, right flow).

Figure 1: MAG VIS EVK Module production test flow diagram

## Test sequence

Table 4 provides an overview of the various tests conducted at each test stage and outlines the correct test sequence, which should be followed from top to bottom.

Table 4: Test sequence per test stage

| Test | VI | VER | Comments |
| --- | --- | --- | --- |
| T0001: Visual inspection (VI) | X |  | Always the first test. |
| T0002: Power consumption |  | X | Measures current under various conditions. |
| T0003: CXP Core functionality and video data check |  | X | Verifies CoaXPress communication and data integrity. |
| T0004: IMG Core functionality and video data check |  | X | Verifies Imager functionality and internal pattern generator. |
| T0005: IMG Trigger mode |  | X | Verifies the imager's ability to operate under external command. |
| T0006: MAG-DRV Capabilities (only for MAG-VIS1000x modules) |  | X | Conditional: Applicable only to MAG-VIS1000x modules. |
| T0007: MAG-DRV Motor drive capabilities (only for MAG-VIS1002x modules) |  | X | Conditional: Applicable only to MAG-VIS1002x modules. |

## Test equipment

All test equipment is listed in Table 5. This table provide recommendations for the required instruments during the different test, indicate their availability based on the test schedule, and specify whether the equipment remains calibrated throughout the various test phases.

Table 5: Measurement Equipment overview

| Instrument name | Instrument type | Short description | Available | Calibrated |
| --- | --- | --- | --- | --- |
| PC | Rack PC | Test computer | TBC | NA |
| Frame Grabber | Matrox Rapixo CXP Frame Grabber | / | TBC | NA |
| PSU | NGL202 | 2 channel power supply | TBC | Yes |
| DMMV | Keysight 34465A | 6.5-digit basic Truevolt DMM | TBC | Yes |
| THERMO STREAMER | Temptronic ECO-710E-M | / | TBC | Yes |
| SCOPE | Keysight DSOX1204G | 4 Analog Channels Oscilloscope | TBC | Yes |

## Statistical analysis

### Bin groups

Following table gives an overview of the bin groups. Bin 1 is always the PASS bin. Bin groups  between 2 and 99 are reserved for PASS with limited performance. Groups starting from 200 are FAILS and should be scrapped.

Table 6: Bin table

| Bin group | Resolution |
| --- | --- |
| 1 | PASS |
| 200 | FAIL |

# Measurement test setup

This chapter explains how to collect measurement data from individual DUTs while working with a limited amount of lab equipment. It also details the instruments used in the test setup, along with all hardware-related requirements. Additionally, the section covers the software used to control the test setup, including software-specific details.

## Test hardware

### Overview

Figure 2 provides an overview of the complete test setup used for most tests outlined in the test procedures (Chapter 4). As shown in the diagram, the Devices Under Test (DUTs) are placed within a temperature-controlled environment, which may be maintained using a thermostreamer or temperature chamber to ensure stable and controlled test conditions.

To enable automated test execution, all instruments are connected through Ethernet and centrally controlled by a test PC. This setup ensures efficient data acquisition, measurement synchronization, and automated execution of the defined test procedures.

Figure 2: Measurement setup overview

The MAG-VIS100xx modules require an adapted measurement configuration to facilitate the functionality of the MAG-DRV IC. The adapted setup incorporates the following additional equipment:

- Oscilloscope: Required for waveform capture and timing analysis. This verifies the operation of the MAG-DRV IC, including successful generation of ON/OFF output signals and the generation of PWM signals.

- Magics MUX Card: A multiplexer card is integrated to multiplex the outputs of the MAG-DRV IC to the oscilloscope.

- Power resistors: These resistors are added to limit the current flow trough the drivers of MAG-DRV IC.

The measurement setup overview diagram for the MAG-VIS1000x will be constrained to highlight only the new, module-specific test equipment. The core system components (Frame Grabber, PSU, and Control Software) are mandatory elements of the EVK test platform and will be intentionally omitted from the specific diagram to maintain clarity and avoid unnecessary clutter.

Figure 3: MAG-VIS1000X Measurement setup overview

The MAG-VIS1002x modules require an adapted measurement configuration to facilitate the functionality of the MAG-DRV IC to drive motors of a lens. The core system components (Frame Grabber, PSU, and Control Software) are mandatory elements of the EVK test platform and will be intentionally omitted from the specific diagram to maintain clarity and avoid unnecessary clutter.

Figure 4: MAG-VIS1002X Measurement setup overview

### DUT information

To limit the use of equipment channels, the DUTs are measured using a time-multiplexed approach, ensuring that only one device is tested at a time (single-site test setup).

Table 7 provides a detailed overview of all DUT pins and specifies the corresponding test equipment required for each pin to execute the defined test procedures.

For debugging purposes, additional connections to the test equipment may be beneficial. These optional connections are marked with the postfix "∗" in the table. The following codes are used in the matrix:

HI_F: Forced voltage or current at the pin

HI_S, LO_S: Sensed voltage or current at the pin

LO_F: Return path

HI_IN: Sensed voltage or current at the pin connects to HI input of DMM or scope

LO_IN: Sensed voltage or current at the pin connects to LO input of DMM

CHx: Input of oscilloscope with x in range [1 to 4]

Table 7: Pin list vs equipment matrix

| Equip Pin | SCOPE | Frame grabber | PSU (2-wire) |
| --- | --- | --- | --- |
| TRXN |  | HI_F |  |
| TRXP |  | HI_S |  |
| VDD_IN |  |  | HI_F |
| GND | LO_F *(1) |  | LO_F |
| CH0 |  |  |  |
| CH1 *(1) | CH1 |  |  |
| CH2 *(1) | CH1 |  |  |
| CH3 *(1) | CH1 |  |  |
| CH4 *(1) | CH1 |  |  |
| CH5 *(1) | CH1 |  |  |
| CH6 *(1) | CH1 |  |  |
| CH7 *(1) | CH1 |  |  |

*(1) Only applicable for the MAG-VIS1000X modules.

### Pin load requirements

During the hardware development attention needs to be given to following items:

- Low-ohmic connections

  - For ground and power connections.

- Impedance matched signals

  - TRXP/TRXN: The TRXN pin is connected to a Coax transmission line leading to the frame grabber, while the TRXP pin is terminated on the substrate board. Matching must adhere to the 75Ω transmission line standard.

### Power supplies

Power is supplied to the DUT using bench Power Supply Units (PSUs). All DUT supply channels are listed in Table 8, where the "default connected" column indicates whether a channel is connected by default during testing unless otherwise specified in the test procedure. The supply can either be directly connected to the pin, via a relays or via an DMMI for most accurate supply current measurements. The PSU channels can be split over different bench equipment depending on availability and sourcing/measurement requirements.

The power-up and power-down sequence for default-connected supplies is detailed in Section 3.2.3. To prevent voltage overshoot, it is strongly recommended to use power sources with a controllable soft ramp.

Table 8: Power supplies

| DUT pin name | Supply domain (Table 1) | Default connected | PSU channel | Alias | Comments |
| --- | --- | --- | --- | --- | --- |
| VDD_IN | VDD_IN | Yes | 1 |  |  |

## Test software

### Overview

The software test suite is an in-house build environment using Python scripts (i.e. Experimate) to control following:

- settings for MaTeR and measurement equipment,

- connectivity between the DUTs and the equipment,

- supply and temperature control,

- FPGA for e.g. serial interfacing with the DUT,

- logging of measurement data and meta data to a database and

- settings for the load board.

### Park state routine

Before each test procedure starts the DUT and the test setup is brought to a known default state (park state). This includes driving following IO pins to:

Table 9: Park state pin biasing

| Pin name |  | Biased to level / dynamic signal (f, dc, swing) |  | Comment |
| --- | --- | --- | --- | --- |
| VDD_IN |  | Supply 9V |  |  |
| GND |  | Ground |  |  |
| TRXN |  | Frame Grabber |  | The DC state is don’t care since there are DC blocking caps in the pad towards the input of module |

### Power cycle routines

Power-up sequence:

- Disconnect all measurement equipment

- Apply PSU-9V  = VDD_IN  = 0V

- Wait 10ms

- Ramp up VDD_IN  to its defined operating condition 9V in 10ms or slower

Power-down sequence:

- Ramp down VDD_IN  to 0V in 10ms or slower

- Disconnect all measurement equipment

# Test procedures

This chapter contains an overview of all the tests included in the test sequence(s). In the overview items as high-level description, test configuration, test conditions, detailed test procedure and acceptance criteria are discussed.

## T0001: Visual inspection (VI)

Table 10: Visual inspection overview

| Test Description |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| To verify the physical integrity of the DUT prior to electrical testing, ensuring no mechanical damage, solder defects or contamination is present that could affect performance or safety. |  |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |  |
| An ESD-safe work place. No software is required. |  |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |  |
| - Magnification device - High-contrast lighting for performing inspection |  |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |  |
| None. This test is performed at Room Temperature (RT) and Vmin/Vnom/Vmax are not applicable. |  |  |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |  |  |
| Manual inspection performed on an ESD-safe workstation in a clean, ambient environment. |  |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |  |
| Step | Action |  |  | Register parameter |  | Value |  |
|  | Ensure DUT is not connected to any power source. |  |  |  |  |  |  |
|  | Inspect PCB surfaces for scratches, delamination, or burn marks. |  |  |  |  |  |  |
|  | Inspect all components for cracks, correct orientation and clear marking. |  |  |  |  |  |  |
|  | Inspect all solder joints for opens, shorts, bridging, or insufficient wetting (dewetting). |  |  |  |  |  |  |
|  | Inspect all connectors and pins for bending, damage or contamination. |  |  |  |  |  |  |
|  | Inspect imager lens (if installed) and optical window for scratches, dust, or smudges. |  |  |  |  |  |  |
|  | Log PASS if no defects are found. Log the specific defect if FAIL. |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| No electrical measurements or power application is allowed during this test. |  |  |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |  |  |
| ID | Name | Unit | LTL |  | Typ |  | UTL |
| M0001 | VI_Pass |  |  |  |  |  |  |
| M0002 | VI_Defect_Code |  |  |  |  |  |  |

## T0002: Power consumption

Table 11: Power consumption overview

| Test Description |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| This test measures the current consumption of the DUT. |  |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |  |
| Hardware: - PSU for forcing VDD_IN and measuring current consumption: - Current consumption accuracy: 100nA, range 100μA - Voltage source accuracy: 10mV, range 100mV - Frame grabber to initiate the streaming mode - Thermostreamer for CT/RT/HT testing |  |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |  |
| Test is conducted at all power supply levels (see Table 1) and on CT, RT and HT. |  |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |  |
| Step | Action |  |  | Register parameter |  | Value |  |
|  | Start from park state. |  |  |  |  |  |  |
|  | Configure the PSU to the appointed voltage. |  |  |  |  |  |  |
|  | Set up the imager to work in the appointed mode (Table 2). |  |  |  |  |  |  |
|  | Enable continuous acquisition mode |  |  | AcquisitionCommand_Value |  | 8 |  |
|  | Wait 5s. |  |  |  |  |  |  |
|  | Measure the current consumption using the PSU, take an average of 10 measurements. |  |  |  |  |  |  |
|  | Log the current measurement. |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| P_VDD_IN_XXXX is calculated from the applied VDD_IN voltage and the measured current. |  |  |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |  |  |
| ID | Name | Unit | LTL |  | Typ |  | UTL |
| M0003 | I_ VDD_IN_FHD10 | mA |  |  |  |  |  |
| M0004 | P_ VDD_IN_FHD10 | W |  |  |  |  |  |
| M0005 | I_ VDD_IN_FHD30 | mA |  |  |  |  |  |
| M0006 | P_ VDD_IN_FHD30 | W |  |  |  |  |  |
| M0007 | I_ VDD_IN_VGA50 | mA |  |  |  |  |  |
| M0008 | P_ VDD_IN_VGA50 | W |  |  |  |  |  |

## T0003: CXP Core functionality and video data check

Table 12: CXP Core functionality and video data check overview

| Test Description |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| This test verifies the basic functionality of the MAG-CXP00002-NP IC and the CoaXPress video data. It involves checking the lock bits, writing and reading a known register value via CoaXPress to confirm basic communication, enabling internal test pattern mode and verifying the correct pattern is received by the frame grabber. |  |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |  |
| Hardware: - PSU for forcing VDD_IN: - Voltage source accuracy: 10mV, range 100mV - Frame grabber to initiate the streaming mode - Thermostreamer for RT testing |  |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |  |
| Test is conducted only at nominal voltage and room temperature (RT). |  |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |  |
| Step | Action |  | Register parameter |  |  | Value |  |
|  | Start from park state. |  |  |  |  |  |  |
|  | Configure the PSU to the Vnom. voltage. |  |  |  |  |  |  |
|  | Set up the imager to work in FHD10 mode (Table 2). |  |  |  |  |  |  |
|  | Enable continuous acquisition mode |  | AcquisitionCommand_Value |  |  | 8 |  |
|  | Wait 5s. |  |  |  |  |  |  |
|  | Read register |  | (PROX) Analog_constant_test |  |  |  |  |
|  | Check if read value matches and log pass/fail. |  | (PROX) Analog_constant_test |  |  | 0xAA |  |
|  | Write register |  | (PROX) VendorUniqueSPIGPIO_data_mosi |  |  | 0xCAFE123 |  |
|  | Read register |  | (PROX) VendorUniqueSPIGPIO_data_mosi |  |  |  |  |
|  | Check if read value matches and log pass/fail. |  |  |  |  | 0xCAFE123 |  |
|  | Read lock bits register |  | (PROX) Analog_ct_lock_bits |  |  |  |  |
|  | Check if value matches and log value. |  |  |  |  | 2 |  |
|  | Enable color mode of the Test pattern generator (TPG) to be able to capture color pattern. |  | (PROX) Analog_col_mono_tpg |  |  | 1 |  |
|  | Set pixel size |  | (PROX) PixelSize_Value |  |  | 10 |  |
|  | Start of video mode loop. |  |  |  |  |  |  |
|  | Configure the first (next) video mode. |  | See Video mode table |  |  |  |  |
|  | Start of test pattern loop. |  |  |  |  |  |  |
|  | Configure the first (next) test pattern. |  | See Test pattern table |  |  |  |  |
|  | Start of frame rate loop. |  |  |  |  |  |  |
|  | Configure the first (next) frame rate. |  | (PROX) AcquisitionFrameRate_Value |  |  | [10,30] |  |
|  | Enable stream |  |  |  |  |  |  |
|  | Capture image |  |  |  |  |  |  |
|  | Save image |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Video mode table Video mode; Register parameter; Value. VGA; ImageSize_Width; 640. ImageSize_Height; 480. HD; ImageSize_Width; 1280. ImageSize_Height; 720. FHD; ImageSize_Width; 1920. ImageSize_Height; 1080. Test Pattern table Pattern; Register parameter; value. Black; TestPattern_Value; 1. White; TestPattern_Value; 2. Grey55; TestPattern_Value; 3. GreyAA; TestPattern_Value; 4. GrayHorizontalRamp; TestPattern_Value; 16. GrayVerticalRamp; TestPattern_Value; 17. ColorBar; TestPattern_Value; 48. GreyScaleChart; TestPattern_Value; 49. VendorSpecific0; TestPattern_Value; 120. VendorSpecific1; TestPattern_Value; 121. VendorSpecific2; TestPattern_Value; 122. VendorSpecific3; TestPattern_Value; 123. VendorSpecific4; TestPattern_Value; 124. VendorSpecific5; TestPattern_Value; 125. - Test pattern validation: Every captured test pattern image is pixelwise compared to an already saved image of the test pattern. |
| Test parameters and acceptance criteria |  |  |  |  |  |  |  |
| ID | Name | Unit |  | LTL | Typ |  | UTL |
| M0009 | Constant register read transaction | Pass/Fail |  |  |  |  |  |
| M0010 | Register write/read transaction | Pass/Fail |  |  |  |  |  |
| M0011 | Read out lock bits |  |  |  |  |  |  |
| M0012 | Test pattern matching | % |  | 100 | 100 |  | 100 |

## T0004: IMG Core functionality and video data check

Table 13: IMG Core functionality and video data check overview

| Test Description |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| This test verifies the basic functionality of the MAG-IMG00XXX IC. It involves checking the PLL status bits, testing image manipulation logic (y-axis flip), enabling the internal Test Pattern Generator (TPG), and verifying the received image pattern via CoaXPress under various video modes and frame rates. |  |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |  |
| Hardware: - PSU for forcing VDD_IN: - Voltage source accuracy: 10mV, range 100mV - Frame grabber to initiate the streaming mode - Thermostreamer for RT testing |  |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |  |
| Test is conducted only at nominal voltage and room temperature (RT). |  |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |  |
| Step | Action |  | Register parameter |  |  | Value |  |
|  | Start from park state. |  |  |  |  |  |  |
|  | Configure the PSU to the Vnom. voltage. |  |  |  |  |  |  |
|  | Set up the imager to work in FHD10 mode (Table 2). |  |  |  |  |  |  |
|  | Enable continuous acquisition mode |  | AcquisitionCommand_Value |  |  | 8 |  |
|  | Wait 5s. |  |  |  |  |  |  |
|  | Read lock bits register |  | (IMG) AnalogTrimmimg_Pll_lock_detect_locked |  |  | 1 |  |
|  | Check if value matches and log value. |  |  |  |  |  |  |
|  | Start of video mode loop. |  |  |  |  |  |  |
|  | Configure the first (next) video mode. |  | See Table 2 |  |  |  |  |
|  | Start of test pattern loop. |  |  |  |  |  |  |
|  | Configure the first (next) test pattern. |  | See Test pattern table |  |  |  |  |
|  | Enable stream |  |  |  |  |  |  |
|  | Capture image |  |  |  |  |  |  |
|  | Save image |  |  |  |  |  |  |
|  | Set up the imager to work in FHD10 mode (Table 2). |  |  |  |  |  |  |
|  | Enable continuous acquisition mode |  | AcquisitionCommand_Value |  |  | 8 |  |
|  | Enable TPG |  | (IMG) TestPattern_Value |  |  | ?? |  |
|  | Capture and save image (Image_A) |  |  |  |  |  |  |
|  | Enable y-axis mirror |  | (IMG) ReverseY_Value |  |  | 1 |  |
|  | Capture and save image (Image_B) |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Test Pattern table Pattern; Register parameter; value. Black; TestPattern_Value; 1. White; TestPattern_Value; 2. Grey55; TestPattern_Value; 3. GreyAA; TestPattern_Value; 4. GrayHorizontalRamp; TestPattern_Value; 16. GrayVerticalRamp; TestPattern_Value; 17. ColorBar; TestPattern_Value; 48. GreyScaleChart; TestPattern_Value; 49. VendorSpecific0; TestPattern_Value; 120. VendorSpecific1; TestPattern_Value; 121. VendorSpecific2; TestPattern_Value; 122. VendorSpecific3; TestPattern_Value; 123. VendorSpecific4; TestPattern_Value; 124. VendorSpecific5; TestPattern_Value; 125. - Test pattern validation: Every captured test pattern image is pixelwise compared to an already saved image of the test pattern. - Y-Axis Mirror validation: The success of the Y-axis image mirror function must be verified by pixel-wise comparison between a reference image (Image_A) captured before the mirror command and a test image (Image_B) captured after the mirror command is enabled. - Verification method: The validation software must confirm that Image_B is an exact mirror image of Image_A along the Y-axis (vertical center). Any deviation indicates a failure in the image manipulation logic. |
| Test parameters and acceptance criteria |  |  |  |  |  |  |  |
| ID | Name | Unit |  | LTL | Typ |  | UTL |
| M0013 | Read out lock bits |  |  |  |  |  |  |
| M0014 | Test pattern matching | % |  | 100 | 100 |  | 100 |
| M0015 | Mirror y-axis | % |  | 100 | 100 |  | 100 |

## T0005: IMG Trigger mode

Table 14: IMG Trigger mode overview

| Test Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| This test provides full functional verification of all imager (IMG) trigger modes. It consists of three main parts: - CXP-Hardware Trigger Flow: Confirms a trigger command sent via the CXP communication successfully toggles the hardware trigger pin on the MAG-CXP00002-NP IC, initiating a single-frame capture on the MAG-IMG00XXX imager IC. - Imager Free-Run Burst Mode: Verifies the imager's ability to capture a programmed burst sequence of frames (30 frames) in MULTIFRAME mode without an external trigger. - Imager Software Trigger Burst Mode: Confirms the imager's ability to capture a burst sequence (30 frames) when initiated by a direct software trigger command sent via SPI. The Test Pattern Generator (TPG) is used throughout to verify data integrity. |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |
| Hardware: - PSU for forcing VDD_IN: - Voltage source accuracy: 10mV, range 100mV - Frame grabber to initiate the streaming mode - Thermostreamer for RT testing |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |
| Test is conducted only at nominal voltage and room temperature (RT). |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |
| Step | Action |  | Register parameter |  |  | Value |
|  | Start from park state. |  |  |  |  |  |
|  | Configure the PSU to the Vnom. voltage. |  |  |  |  |  |
|  | Set up the imager to work in FHD10 mode (Table 2). |  |  |  |  |  |
|  | Set acquisition command to MULTIFRAME mode. |  | (IMG) AcquisitionCommand_Value |  |  | 9 |
|  | Configure the CXP trigger source to software control. |  | (PROX) TriggerSource_Value |  |  | 64 |
|  | Configure the IMG trigger source to hardware control. |  | (IMG) TriggerSource_Value |  |  | 120 |
|  | Enable TPG |  | (IMG) TestPattern_Value |  |  | ?? |
|  | Set acquisition frame count to single frame capture. |  | (IMG) TriggerAdditionalParameter_Value |  |  | 1 |
|  | Set mode to hold until external (in this case a hardware) trigger is given. |  | (IMG) TriggerMode_Value |  |  | 1 |
|  | Enable a hardware toggle on the CXP IC by writing a register. |  | (CXP) SoftwareTrigger_Value |  |  | 1 |
|  | Capture and save the single frame. |  |  |  |  |  |
|  | Disable the hardware toggle on the CXP IC by writing a register. |  | (CXP) SoftwareTrigger_Value |  |  | 0 |
|  | Set acquisition command to STOP mode. |  | (IMG) AcquisitionCommand_Value |  |  | 1 |
|  | Set acquisition frame count to a burst of 30 frames. |  | (IMG) AcquisitionFrameCount_Value |  |  | 30 |
|  | Set mode to hold until software trigger is given. |  | (IMG) TriggerMode_Value |  |  | 0 |
|  | Set acquisition command to MULTIFRAME mode. To start the multi frame capturing. |  | (IMG) AcquisitionCommand_Value |  |  | 9 |
|  | Capture and save the burst of frames. |  |  |  |  |  |
|  | Read acquisition command, this should be automatically set back to 1. |  | (IMG) AcquisitionCommand_Value |  |  | 1 |
|  | Set acquisition frame count to a burst of 30 frames. |  | (IMG) TriggerAdditionalParameter_Value |  |  | 30 |
|  | Configure the IMG trigger source to software control. |  | (IMG) TriggerSource_Value |  |  | 64 |
|  | Set mode to hold until external (in this case a software) trigger is given. |  | (IMG) TriggerMode_Value |  |  | 1 |
|  | Set acquisition command to MULTIFRAME mode. |  | (IMG) AcquisitionCommand_Value |  |  | 9 |
|  | Give a software trigger directly to the IMG IC. |  | (IMG) TriggerSource_Value |  |  | 1 |
|  | Capture and save the burst of frames. |  |  |  |  |  |
| Notes |  |  |  |  |  |  |
| - Data integrity validation is mandatory for all captured frames. Each captured frame (single and burst) must be compared pixel-wise to a known, pre-saved reference frame corresponding to the configured pattern. This comparison must confirm a 100% match. |  |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |  |
| ID | Name | Unit |  | LTL | Typ | UTL |
| M0016 | HwTrig_SingleFrame_Count | Count |  | 1 | 1 | 1 |
| M0017 | HwTrig_SingleFrame_TPG_Match | % |  | 100 | 100 | 100 |
| M0018 | FreeRun_BurstFrame_Count | Count |  | 30 | 30 | 30 |
| M0019 | FreeRun_BurstFrame_TPG_Match | % |  | 100 | 100 | 100 |
| M0020 | SwTrig_BurstFrame_Count | Count |  | 30 | 30 | 30 |
| M0021 | SwTrig_BurstFrame_TPG_Match | % |  | 100 | 100 | 100 |

## T0006: MAG-DRV Capabilities (only for MAG-VIS1000x modules)

Table 15: MAG-DRV Capabilities overview

| Test Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| This test provides functional verification of the MAG-DRV IC outputs in both steady-state (ON/OFF) and dynamic (PWM) modes. The test confirms that the driver successfully switches the output as commanded, using the oscilloscope and MAGICS MUX Card to quickly check all channels. |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |
| Hardware: - PSU for forcing VDD_IN: - Voltage source accuracy: 10mV, range 100mV - Frame grabber to initiate the streaming mode - Thermostreamer for RT testing The test setup need to be adjusted as depicted in Figure 3. |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |
| Test is conducted only at nominal voltage and room temperature (RT). |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |
| Step | Action |  | Register parameter |  |  | Value |
|  | Start from park state. |  |  |  |  |  |
|  | Configure the PSU to the Vnom. voltage. |  |  |  |  |  |
|  | Set up the imager to work in FHD10 mode (Table 2). |  |  |  |  |  |
|  | Initialize SPI and output clock on the MAG-CXP IC. |  | See initializing code snippet (Appendix A) |  |  |  |
|  | Reset and clear flags MAG-DRV IC . |  | See code snippet (Appendix B) |  |  |  |
|  | Start channel steady-state testing loop. |  |  |  |  |  |
|  | Mux the first (next) channel to the oscilloscope. |  |  |  |  |  |
|  | Verify and log with scope if channel is in OFF state. |  |  |  |  |  |
|  | Drive channel to ON state. |  | (DRV) drv_enable (0x0A) |  |  | Write 1 at Xth bit *(1) |
|  | Verify and log with scope if channel is in ON state. |  |  |  |  |  |
|  | Drive channel to OFF state. |  | (DRV) drv_enable (0x0A) |  |  | Write 0 at Xth bit *(1) |
|  | Verify and log with scope if channel is in OFF state. |  |  |  |  |  |
|  | End channel steady-state testing loop. |  |  |  |  |  |
|  | Reset and clear flags MAG-DRV IC . |  | See code snippet (Appendix B) |  |  |  |
|  | Disable PWM on all channels.. |  | (DRV) PWM_en_X field (0x0C) |  |  | 0 |
|  | Set PWM frequency. |  | (DRV) PWM_freq (0x0D Bits 5:0) |  |  | 0 |
|  | Set PWM low frequency . |  | (DRV) PWM_low_freq (0x0D Bit 6) |  |  | 0 |
|  | Set PWM reduce precision. |  | (DRV) PWM_reduce_precision (0x0D Bit 7) |  |  | 1 |
|  | Start channel PWM testing loop. |  |  |  |  |  |
|  | Set duty cycle to 50% for the corresponding channel. |  | See duty cycle note *(2) |  |  | 512 (decimal) |
|  | Enable PWM for the corresponding channel. |  | (DRV) PWM_en_X (0x0C) |  |  | Write 1 at Xth bit |
|  | Drive channel. |  | (DRV) drv_enable (0x0A) |  |  | Write 1 at Xth bit |
|  | Verify and log with scope if channel is in PWM drive mode. |  |  |  |  |  |
|  | Disable channel. |  | (DRV) drv_enable (0x0A) |  |  | Write 0 at Xth bit |
| Notes |  |  |  |  |  |  |
| - *(1): Each channel in MAG-DRV00002-NP can be independently controlled via the 8 bits in the drv_enable field in register(0x0A). The Xth bit in this register represents the demanded state of the Xth channel of MAG-DRV00002-NP. When this bit is written to 1, the channel will be activated, the switch device will go in a conductive state until the bit is re-written to 0. - *(2): The duty cycle of the channels can be individually programmed from 0% to 100% via the 10-bit field PWM_dcycle_x in registers (0x0F), (0x11), (0x13), (0x15), (0x17), (0x19), (0x1B) and (0x1D) that corresponds to each individual channel. |  |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |  |
| ID | Name | Unit |  | LTL | Typ | UTL |
| M0022 | OFF state verification | Pass/Fail |  |  |  |  |
| M0023 | ON state verification | Pass/Fail |  |  |  |  |
| M0024 | PWM frequency check | Hz |  |  | 40KHz |  |
| M0025 | PWM duty cycle | % |  |  | 50 |  |

## T0007: MAG-DRV Motor drive capabilities (only for MAG-VIS1002x modules)

Table 16: MAG-DRV Motor drive capabilities overview

| Test Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| This test provides functional verification of the MAG-DRV IC ability to drive external loads, specifically to control the directional movement of the Iris, Zoom, and Focus motors. The test iterates through each motor, verifies the correct drv_enable register values for both left and right direction achieve the intended physical motor movement, and ensures the motor can be safely disabled (OFF state). |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |
| Hardware: - PSU for forcing VDD_IN: - Voltage source accuracy: 10mV, range 100mV - Frame grabber to initiate the streaming mode - Thermostreamer for RT testing The test setup need to be adjusted as depicted in Figure 4. |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |
| Test is conducted only at nominal voltage and room temperature (RT). |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |
| Step | Action |  | Register parameter |  |  | Value |
|  | Start from park state. |  |  |  |  |  |
|  | Configure the PSU to the Vnom. voltage. |  |  |  |  |  |
|  | Set up the imager to work in FHD10 mode (Table 2). |  |  |  |  |  |
|  | Initialize SPI and output clock on the MAG-CXP IC. |  | See initializing code snippet (Appendix A) |  |  |  |
|  | Start motor loop: [iris, zoom, focus] |  |  |  |  |  |
|  | Connect the appointed pins from the connector of the EVK to the terminals of the test motor. |  |  |  |  |  |
|  | Drive first (next) motor direction A. |  | (DRV) drv_enable (0x0A) |  |  | See motor drive table |
|  | Verify manually and log if the motor rotate in direction A. |  |  |  |  |  |
|  | Disable motor. |  | (DRV) drv_enable (0x0A) |  |  | 0b00000000 |
|  | Reset and clear flags MAG-DRV IC . |  | See code snippet (Appendix B) |  |  |  |
|  | Change direction of motor. |  | (DRV) drv_enable (0x0A) |  |  | See motor drive table |
|  | Drive iris channel direction B. |  | (DRV) drv_enable (0x0A) |  |  | See motor drive table |
|  | Verify manually and log if the motor rotate in direction B. |  |  |  |  |  |
|  | Disable motor. |  | (DRV) drv_enable (0x0A) |  |  | 0b00000000 |
|  | Reset and clear flags MAG-DRV IC . |  | See code snippet (Appendix B) |  |  |  |
| Notes |  |  |  |  |  |  |
| Motor drive table Motor; Direction; value. Iris; A; 0b00000001. Iris; B; 0b00000011. Zoom; A; 0b00000100. Zoom; B; 0b00001100. Focus; A; 0b00010000. Focus; B; 0b00110000. - The rotation verification is manual, requiring a human operator to confirm movement and direction. |
| Test parameters and acceptance criteria |  |  |  |  |  |  |
| ID | Name | Unit |  | LTL | Typ | UTL |
| M0025 | Iris motor functional test | Pass/Fail |  |  |  |  |
| M0026 | Zoom motor functional test | Pass/Fail |  |  |  |  |
| M0027 | Focus motor functional test | Pass/Fail |  |  |  |  |

List of Acronyms

| Acronym | Definition |
| --- | --- |
| CXP | CoaXPress (The high-speed digital interface standard) |
| DUT | Device Under Test |
| DMMV | Digital Multimeter Voltmeter (Keysight 34465A) |
| DRV | Driver (Referring to the MAG-DRV IC) |
| EVK | Evaluation Kit |
| FHD | Full High Definition (1920×1080) |
| FPS | Frames Per Second |
| FT | Final Test (Test Stage) |
| GND | Ground |
| IMG | Imager (Referring to the MAG-IMG IC) |
| LTL | Lower Test Limit |
| MUX | Multiplexer |
| PSU | Power Supply Unit |
| PWM | Pulse Width Modulation |
| RT | Room Temperature |
| SPI | Serial Peripheral Interface |
| TPG | Test Pattern Generator |
| TRR | Test Readiness Review |
| UTL | Upper Test Limit |
| VDD_IN | Input Supply Voltage |
| VER | Functional Verification (Test Stage) |
| VI | Visual Inspection (Test Stage) |
| VGA | Video Graphics Array (640×480) |
| Vnom | Nominal Voltage |
| Vmin | Minimal Voltage |
| Vmax | Maximal Voltage |

MAG-CXP IC Iniatilze SPI and Clock code snippet

1. # region configure the spi block

2. self.set_digitizer_feature("PROX_VendorUniqueSPIGPIO_spi_enable", 1)  # enable spi block

3. self.set_digitizer_feature("PROX_VendorUniqueSPIGPIO_divider", 62)  # divide 62.5MHz to 1MHz

4. self.set_digitizer_feature("PROX_VendorUniqueSPIGPIO_transaction_size", 15)  # 16bit (15+1)

5. self.set_digitizer_feature("PROX_VendorUniqueSPIGPIO_ssel", 0)  # slave select on gpio 0

6. # endregion

7.

8. # region configure gpio pins

9. # miso gpio 10, mosi gpio 9, sclk gpio 8, cs gpio 0

10. self.set_digitizer_feature("PROX_IOLineModeAll_Value", 0b01111111111)  # 1 - output, 0 - input

11.

12. # selects the I/O line to handle

13. self.set_digitizer_feature("PROX_IOLineSelector_Value", 7)

14. # selects which source signals to connect I/O line: 32 = internal spi

15. self.set_digitizer_feature("PROX_IOLineSource_Value", 32)

16.

17. self.set_digitizer_feature("PROX_IOLineSelector_Value", 8)

18. self.set_digitizer_feature("PROX_IOLineSource_Value", 32)

19.

20. self.set_digitizer_feature("PROX_IOLineSelector_Value", 9)

21. self.set_digitizer_feature("PROX_IOLineSource_Value", 32)

22.

23. self.set_digitizer_feature("PROX_IOLineSelector_Value", 0)

24. self.set_digitizer_feature("PROX_IOLineSource_Value", 32)

25. # endregion

26.

27. self._spi_initialized = True

28.

MAG-DRV IC Reset and clear all

1. self.spi_write_reg_name(name='reset_ocd', data=0b11111111)

2. time.sleep(time_to_wait)

3. self.spi_write_reg_name(name='reset_ocd_start', data=0b1)

4. time.sleep(time_to_wait)

5. # clear ocd flags

6. self.spi_write_reg_name(name='ocd', data=0b11111111)

7. time.sleep(time_to_wait)

8. self.spi_write_reg_name(name='reset_ocd_done', data=0b1)

9. time.sleep(time_to_wait)

10. # clear bist flags

11. self.spi_write_reg_name(name='bist', data=0b11111111)

12. time.sleep(time_to_wait)

13. self.spi_write_reg_name(name='tied_high', data=0b1)

14. time.sleep(time_to_wait)

15. self.spi_write_reg_name(name='tied_low', data=0b1)

16. time.sleep(time_to_wait)

Idm@F4E #