# Vision Series Test Plan Document

## 1. Revision History

| Revision | Date | Description | Author |
| --- | --- | --- | --- |
| 1.0 | 2026-04-10 | Initial draft of the Vision Series Test Plan | Test Architect |

## 2. List of Figures

- Figure 1: Vision Series: three-step verification rail

- Figure 2: MAG-IMG002x1-NC Production Test Flow

- Figure 3: MAG-IMG002x1-NC Trim Test Flow

- Figure 4: MAG-CXP00002-NP Production Test Flow

- Figure 5: MAG-PSU00001-NP Production Test Flow

- Figure 6: MAG-VIS100xx-N Production Test Flow

## 3. List of Tables

- Table 1: MAG-IMG002X1-NC Key Specifications

- Table 2: MAG-CXP00002-NP Key Specifications

- Table 3: MAG-PSU00001-NP Key Specifications

- Table 4: MAG-VIS100xx-N Key Specifications

## 4. Introduction & Scope

This document outlines the electrical testing for the Magics Technologies Vision Series to ensure suitability for nuclear environments. The Vision Series includes standalone radiation-hardened ICs (Image Sensor, Serializer, DC/DC Converter) and fully integrated electronic Vision Modules.

This test plan details the test conditions, test flows, test stages, and the production and trimming tests required to guarantee the reliability and performance of the products.

## 5. Vision products verification process

The verification and testing of the Vision Series products follow a strict three-step horizontal process rail. This ensures that quality is maintained from the IC component level up to the fully integrated module.

Figure 1: Vision Series: three-step verification rail

### 5.1 Stage 1: Component tested in test socket

- Focus: IC products

- Description: Trimming and Final Test of the individual integrated circuits. Tests include continuity, leakage, parametric trimming, and functional verification according to the qualification plan.

### 5.2 Stage 2: Populated PCBs

- Focus: PCB products

- Description: Functional test of post-assembly boards before stack integration. Verifies that the soldering and board-level interconnects are reliable and that the individual sub-systems function correctly.

### 5.3 Stage 3: Integrated module

- Focus: Module product

- Description: Module Final Test. Verifies the CoaXPress downlink/uplink and power delivery at the module scope. Ensures the complete electronic camera module meets the datasheet specifications.

## 6. Test specifications

This chapter outlines the test flows, test stages, and test procedures for each product in the Vision Series.

### 6.1 MAG-IMG002x1-NC (Image Sensor)

#### 6.1.1 Test flow

The production and trimming test flows for the image sensor are depicted below:

Figure 2: MAG-IMG002x1-NC Production Test Flow

Figure 3: MAG-IMG002x1-NC Trim Test Flow

#### 6.1.2 Test stages

- Visual Inspection (VI): Examination of external surfaces, glass lid, and wire bonding.

- Connectivity Test (CONN): Checks for shorts on the DUT interface to prevent damage.

- Trimming (TRIM): Tuning of internal references and blocks.

- Final Testing (FT): Confirms that the product is compliant against test limits.

#### 6.1.3 Test procedures

- Continuity: Verifies that there are no open or short circuits on the pins.

- Power Short: Checks for shorts between power supplies and ground.

- Leakage: Measures the input leakage current on control pins.

- VIL/VIH: Measures the input low and high voltage thresholds.

- VOL/VOH: Measures the output low and high voltage levels.

- IOL/IOH: Measures the output drive strength.

- IDDD: Measures the digital and analog power consumption.

- Trim test: Trims internal references to target values.

### 6.2 MAG-CXP00002-NP (Serializer)

#### 6.2.1 Test flow

Figure 4: MAG-CXP00002-NP Production Test Flow

#### 6.2.2 Test stages

- Visual Inspection (VI): Examination of external surfaces and packaging.

- Connectivity Test (CONN): Checks for shorts on the DUT interface.

- Final Testing (FT): Confirms functional and parametric compliance.

#### 6.2.3 Test procedures

(Similar to MAG-IMG002x1-NC) - Continuity / Power Short: Verifies no open/short circuits on pins and supplies. - Leakage: Measures input leakage current. - Functional Test: Validates video transmission, SPI communication, and CXP link stability.

### 6.3 MAG-PSU00001-NP (DC/DC Converter)

#### 6.3.1 Test flow

Figure 5: MAG-PSU00001-NP Production Test Flow

#### 6.3.2 Test stages

- Visual Inspection (VI): Examination of external surfaces.

- Connectivity Test (CONN): Checks for shorts on the DUT interface.

- Final Testing (FT): Confirms functional and parametric compliance.

#### 6.3.3 Test procedures

- Power Short: Verifies no shorts on input/output power pins.

- Load Short: Verifies the converter’s response to a shorted load.

- Leakage: Measures input leakage current on control pins.

- Threshold Voltage: Measures VIL/VIH and hysteresis.

- UVLO Test: Verifies Under-Voltage Lockout thresholds.

- Efficiency / Load Regulation: Measures converter efficiency under various loads.

### 6.4 MAG-VIS100xx-N (Vision Module)

#### 6.4.1 Test flow

Figure 6: MAG-VIS100xx-N Production Test Flow

#### 6.4.2 Test stages

- Visual Inspection (VI): Examination of the assembled PCB stack.

- Connectivity Test (CONN): Checks for shorts on the module interface.

- Module Final Test (FT): Confirms the integrated module meets specifications.

#### 6.4.3 Test procedures

- CXP Core functionality: Validates the CoaXPress downlink and uplink communication.

- Video data check: Verifies the image sensor readout through the serializer.

- Power delivery check: Ensures the integrated DC/DC converters supply correct voltages.

## 7. Product Specifications & Acceptance Criteria

The key specifications for the Vision Series products are extracted from their respective datasheets. The acceptance criteria and test limits are defined in the production test plans.

### 7.1 MAG-IMG002X1-NC (Full HD CMOS Image Sensor)

Table 1: MAG-IMG002X1-NC Key Specifications

| Parameter | Specification | Test Condition / Acceptance Criteria |
| --- | --- | --- |
| Resolution | 1920 x 1080 (Full HD) | Validated via image capture test |
| Frame Rate | Up to 30 FPS | Validated via continuous mode test |
| Dynamic Range | > 60 dB | Characterized during optical test |
| Supply Voltage | 1.8V (Digital), 3.3V (Analog) | VDD pin voltage test |
| Power Consumption | < 1W (typical) | IDD current measurement |
| Radiation Hardness (TID) | > 1 MGy | Guaranteed by design and lot acceptance |
| Radiation Hardness (SEE) | > 62.5 MeV.cm²/mg | Guaranteed by design |

### 7.2 MAG-CXP00002-NP (CoaXPress Serializer ASIC)

Table 2: MAG-CXP00002-NP Key Specifications

| Parameter | Specification | Test Condition / Acceptance Criteria |
| --- | --- | --- |
| Downlink Speed | 1.25 Gbps | Tested via functional interface test |
| Uplink Speed | 20 Mbps | Tested via functional interface test |
| Supply Voltage | 3.3V | VDD pin voltage test (LSL: 3.0V, USL: 3.6V) |
| Power Consumption | 272 mW (typical) | IDD current measurement |
| Clock Input | 10 - 40 MHz | Tested via PLL lock verification |
| Radiation Hardness (TID) | > 1 MGy (100 Mrad) | Guaranteed by design and lot acceptance |
| Radiation Hardness (SEE) | > 60 MeV.cm²/mg | Guaranteed by design |

### 7.3 MAG-PSU00001-NP (10W Synchronous Step-Down DC/DC Converter)

Table 3: MAG-PSU00001-NP Key Specifications

| Parameter | Specification | Test Condition / Acceptance Criteria |
| --- | --- | --- |
| Input Voltage Range | 5V to 11V | UVLO High (LSL: 4.4V, USL: 4.8V) |
| Load Capability | 4A (active cooling) | IOL/IOH current tests |
| Switching Frequency | 1 - 3 MHz | Functional switching test |
| Input Leakage Current | - | LSL: -10mA, USL: 10mA |
| VIL (Input Low Voltage) | 0.73V (typical) | LSL: 0.67V, USL: 0.78V |
| VIH (Input High Voltage) | 0.815V (typical) | LSL: 0.77V, USL: 0.90V |
| Radiation Hardness (TID) | > 1 MGy | Guaranteed by design and lot acceptance |
| Radiation Hardness (SEE) | 64 MeV.cm²/mg | Tested free of destructive SEEs |

### 7.4 MAG-VIS100xx-N (Electronic Vision Module)

Table 4: MAG-VIS100xx-N Key Specifications

| Parameter | Specification |
| --- | --- |
| Components | MAG-IMG002X1, MAG-CXP0002, 2x MAG-PSU00001 |
| Input Voltage | 9V (generates 3.3V and 1.8V internally) |
| Video Interface | CoaXPress 1.25 Gbps (75-Ohm coax) |
| Radiation Hardness | > 1 MGy TID (Si) |
| Operating Temp | 0 to 50 °C ambient |