# Characterization and product test plan

Subject: MAG-PSU

Author: Jasper Gielis version; Date; Reviewed; Approved by. 1v2; 2026-03-23; Jonathan Vanden Berk.

Revision history

| Version | Date | Revisions |
| --- | --- | --- |
| 1v0 | 2025-04-23 | First version Reviewed by: Jonathan Vanden Berk Approved by: |
| 1v1 | 2025-10-15 | Reviewed to be in line with implementation. This revision is used for the TRR meeting on 15/10/2025 |
| 1v2 | 2026-03-23 | Update sequence of VIS stage |

Table of Contents

Revision history	2

Table of Contents	3

List of Figures	5

List of Tables	6

1	Document purpose and scope	7

2	Test strategy	8

2.1	Test conditions	8

2.2	Test stages	8

2.2.1	Connectivity test (CONN)	9

2.2.2	Characterization (CHAR)	9

2.2.3	Characterization of the OTP block (CHAR_OTP)	9

2.2.4	Final Test (FT)	9

2.2.5	DCDC-Module testing (MODULE)	9

2.2.6	Vision DCDC sub-board testing (VIS)	9

2.2.7	Calibration (CAL)	9

2.3	Test flow	10

2.4	Test sequence	10

2.5	Test time	11

2.6	Test equipment	12

2.7	Statistical analysis	13

2.7.1	Test limits	13

2.7.2	Bin groups	13

2.7.3	Dynamic test limits	13

2.8	QC test stages	13

3	Measurement test setup	14

3.1	Test hardware	14

3.1.1	Overview	14

3.1.2	DUT information	15

3.1.3	Pin load requirements	16

3.1.4	Daughter board and external components	16

3.1.5	Measurement board	16

3.1.6	Power supplies	17

3.2	Test software	17

3.2.1	Overview	17

3.2.2	Park state routine	17

3.2.3	Power-down state routine	18

3.2.4	Power cycle routines	19

3.2.5	Design for test (DfT) features	19

4	Test procedures	21

4.1	T0001: Power Short	21

4.2	T0002: Load Short	21

4.3	T0003: PSU calibration	22

4.4	T0004: SMU calibration	22

4.5	T0005: Leakage test	23

4.6	T0006: Input threshold	24

4.7	T0007: Output drive-strength	25

4.8	T0008: On-chip regulator	25

4.9	T0009: Under Voltage Lock Out	26

4.10	T0010: Output voltage monitor	27

4.11	T0011: Over Temperature Protection	28

4.12	T0012: Startup behaviour	29

4.13	T0013: Inv_Enable functionality	30

4.14	T0014: Line regulation	31

4.15	T0015: Load regulation	31

4.16	T0016: Efficiency	32

4.17	T0017: Output voltage ripple	33

4.18	T0018: VIS module test	34

4.19	T0019: Transient load	35

Bibliography	36

List of Acronyms	37

List of Figures

Figure 1: test flow	10

Figure 3: Measurement setup overview	14

Figure 4: Detailed setup	15

List of Tables

Table 1: Supply voltage conditions	8

Table 2: Test stages	8

Table 3: Test sequence per test stage	11

Table 4: Measurement Equipment overview	12

Table 5: Software bin table	13

Table 6: Pin list vs equipment matrix	15

Table 7: Socket board probe-points	16

Table 8: Power supplies	17

Table 9: Park state pin biasing	17

Table 10: Power-down state pin biasing	18

Table 11: Test Mux chip connections	19

Table 12: Test Mux Load terminal settings	20

Table 13: microcontroller pin settings	20

Table 14: Power Short overview	21

Table 14: Load Short overview	21

Table 15: PSU calibration overview	22

Table 15: SMU calibration overview	22

Table 16: Leakage test overview	23

Table 17: Input threshold overview	24

Table 18: Output drive-strength overview	25

Table 19: On-chip regulator overview	25

Table 20: Under Voltage Lock Out overview	26

Table 21: Output voltage monitor overview	27

Table 22: Over Temperature Protection overview	28

Table 23: Startup behaviour overview	29

Table 24: Inv_Enable functionality overview	30

Table 25: Line regulation overview	31

Table 26: Load regulation overview	31

Table 27: Efficiency overview	32

# Document purpose and scope

This document is part of PSU00001 product and defines the test strategy, test setup and the detailed test procedures to perform electrical characterisation and production test on the product.

All the information in this document is intended to describe the verification tests that will be done on the product mentioned above. Any major deviations from this plan will be discussed prior or at latest during the test readiness review (TRR) meeting. Major changes will be listed in the revision history. Major changes can be (list is not exclusive):

Number of DUT’s

Test conditions

Hardware/equipment changes

Test facility changes

Addition or removal of tests

Any update to a test procedure that changes the test coverage or testing strategy

- Minor changes are kept in the revision history but not explicitly informed to the rest of the team. These changes are limited to:

- Small changes in a test procedure, e.g. additional SPI commands, increasing wait times

- Correction of typo’s

Any changes (major and minor) after the TRR meeting will be communicated via email towards the team and if needed discussed during a meeting.

# Test strategy

This chapter describes the test methods used for the verification and validation of the MAG-PSU00001 product. The test strategy aims at maximal test coverage of the product.

Functional testing and electrical parametric testing of the test structures is used to verify the performance.

## Test conditions

During functional verification and characterization testing the temperature and supply voltage of the DUT’s will be varied to validate the ASIC specifications over the defined operating conditions. The conditions listed in this section are the minimal set of conditions that will be tested. If additional test conditions are needed to validate certain parameters, then they are explicitly mentioned in the test procedure.

Temperature conditions are mentioned in this document with following codes:

- RT = room temperature = 25°C +/- 5°C

- CT = cold temperature = -40°C +/- 5°C

- HT = hot temperature = 85°C +/- 5°C

Supply voltage conditions are defined in the table below:

Table 1: Supply voltage conditions

| Supply level | Minimal | Nominal | Maximal | Comments |
| --- | --- | --- | --- | --- |
| Vin | 5 | 9 | 11 |  |

## Test stages

Table 2 outlines all the test stages required to validate the performance of the test vehicle, while section 2.2.5 details the sequence in which these test stages fit into the overall test flow.

Table 2: Test stages

| Name | Code | Temperature | Supply | Other conditions | Number of parts | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Production and screening tests |  |  |  |  |  |  |
| Final test | FT | RT, TBD other temps | TBD | / | All | MAG-PSU00001-NP |
| DCDC-Module test | MODULE | RT | Vmin, Vnom | / |  | MAG-PSU10xxx-N |
| Vision DCDC sub-board test | VIS | RT | Vmin, Vnom | / |  | MAG-VIS4001x-N |
| Calibration | CAL | RT | Sweep between Vmin and Vmax | / | 1 | Calibration period TBD |
| Validation tests |  |  |  |  |  |  |
| Characterization | CHAR | RT, CT, HT | Vmin, Vnom, Vmax | Different Iload, Vout, half_sw settings | 100 | MAG-PSU00001-NP, These test conditions are minimal, more points can be added. |
| OTP block characterization | OTP_CHAR | Sweep up to 120°C in steps of 5°C | Vnom | / | 30 | MAG-PSU00001-NP |

The following sections provide further details on the different test stages.

### Connectivity test (CONN)

The connectivity test is the initial stage of any other test. It checks for shorts on the DUT interface. DUTs need to successfully pass this test-stage in order to be tested further. The CONN tests are designed to reduce the risk of destructive errors damaging the DUT or setup.

### Characterization (CHAR)

The characterization stage is intended to test the electrical requirements of the DUTs over the operating conditions (temperature, load-current and supply-level) as defined in section 2.1.

During the verification phase of the product development lifecycle the number of parts used in the analysis can be smaller compared to the number used in validation phase. The full characterization as part of the device validation phase should be performed on a number of parts that is statistically meaningful. The test coverage should be similar in both phases. If not, the reason for changing test coverage must be addressed in the characterization report.

### Characterization of the OTP block (CHAR_OTP)

Special test stage where only the behaviour of the OTP block is characterized, since this block reacts on temperature changes, a temperature sweep is needed and this is not compatible with the CHAR test stage since test time will explode.

### Final Test (FT)

The final test stage is intended to verify that the chip is within specifications. This is a reduced test compared to the characterization (CHAR). Where the CHAR test does a high resolution sweep over parameters, the FT only checks if the minima and maxima are within spec.

### DCDC-Module testing (MODULE)

The DCDC-Module test stage is performed on MAG-PSU10XX1 assemblies. Here the efficiency and voltage-ripple are important. Tests like line-regulation and load-regulation are performed as well. When these parameters are within test limits, the module is considered good.

### Vision DCDC sub-board testing (VIS)

The Vision sub-board test stage is performed on the DCDC sub-board of the MAG-VIS400xx PCBs,  in order to verify the functionality of the board. An expected load-profile, mimicking that of the other boards in the stack, can be applied. If the voltage-level and ripple are within test limits, the board is considered functional verified.

### Calibration (CAL)

The calibration step contains a couple test procedures that are performed on an empty socket or empty base-board configuration. They are meant to be used to characterize the setup itself. This includes the measurement equipment but also the fixed pull-up or pull-down resistors in the setup. This will allow for more accurate analysis of the actual Device Under Test (DUT) in the other test stages.

## Test flow

This section defines the test flow that every MAG-PSU00001-NP part will follow. It is either tested as a stand-alone part or it is used as a part in a module or camera module.

Figure 1: test flow

## Test sequence

Table 3 provides an overview of the various tests conducted at each test stage and outlines the correct test sequence, which should be followed from top to bottom.

Table 3: Test sequence per test stage

| Test | CHAR | FT | MODULE | VIS | OTP_CHAR | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| T0001: Power Short | X | X | X | X | X |  |
| T0002: Load Short | X | X | X | X | X |  |
| T0005: Leakage test | X | X |  |  |  |  |
| T0006: Input threshold | X | X |  |  |  |  |
| T0007: Output drive-strength | X | X |  |  |  |  |
| T0008: On-chip regulator | X | X |  |  |  |  |
| T0009: Under Voltage Lock Out | X | X | X |  |  |  |
| T0010: Output voltage monitor | X | X |  |  |  |  |
| T0011: Over Temperature Protection | X | X |  |  | X |  |
| T0012: Startup behaviour | X | X | X |  |  |  |
| T0013: Inv_Enable functionality | X | X |  |  |  |  |
| T0014: Line regulation | X | X | X | X |  |  |
| T0015: Load regulation | X | X | X | X |  |  |
| T0016: Efficiency | X | X | X | X |  |  |
| T0017: Output voltage ripple | X |  | X | X |  | This test is excluded in FT since setup introduces a load on Vi pins that is so large that this parameter is not measured correctly |
| T0018: VIS module test |  |  |  |  |  | This test is added but not supported anymore since the VIS board with the 1V8 DCDC output has become obsolete |
| T0019: Transient load |  |  |  |  |  | This test is not added to any of the test stages since the current load profile for the application is unknown, the test is kept in this documents as placeholder but will not be implemented in code. |

## Test time

| Test ID | Test name | Duration [ MM:SS ] |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | CHAR | FT | MODULE | VIS |
| T0001 | Power Short | 00:01.70 |  |  |  |
| T0002 | Load Short | 00:02.00 |  |  |  |
| T0005 | Leakage test | 00:03.11 |  | - | - |
| T0006 | Input threshold | 00:06.60 |  | - | - |
| T0007 | Output drive-strength | 00:01.25 |  | - | - |
| T0008 | On-chip regulator | 00:55.80 | 00:08.24 | - | - |
| T0009 | Under Voltage Lock Out | 01:44.25 |  | 01:44.25 | - |
| T0010 | Output voltage monitor | 00:15.21 |  | - | - |
| T0011 | Over Temperature Protection | 00:00.80 |  | - | - |
| T0012 | Startup behaviour | 03:30.20 |  |  | - |
| T0013 | Inv_Enable functionality | 00:00.47 | 00:00.46 | - | - |
| T0014 | Line regulation | 00:15.43 | 00:04.97 | 00:02.80 | - |
| T0015 | Load regulation | 00:09.30 |  | 00:01.96 | - |
| T0016 | Efficiency | 03:16.22 | 01:12.47 | 00:04.68 |  |
| T0017 | Output voltage ripple | 00:18.99 | - | 00:03.43 |  |
| T0018 | VIS module test | Not implemented |  |  |  |
| T0019 | Transient load | Not implemented |  |  |  |

| ID | Test stage name | Duration [ MM:SS ] |
| --- | --- | --- |
| CONN | Connectivity test | 00:03.70 |
| CHAR | Characterization | 10:39.93 (at 1 temperature) |
| FT | Final Test | 07:15.01 (at 1 temperature) |
| MODULE | DCDC-Module testing | 05:26.12 (at 1 temperature) |
| VIS | Vision DCDC sub-board testing | 00:10.57 (at 1 temperature) |
| OTP_CHAR | Characterization OTP block | 00:08.60 (at 1 temperature) |

## Test equipment

All test equipment outlined in the hardware overview is listed in Table 5 and Table 6. These tables provide recommendations for the required instruments during the different test, indicate their availability based on the test schedule, and specify whether the equipment remains calibrated throughout the various test phases.

Table 4: Measurement Equipment overview

| Instrument name | Instrument type | Short description | Available | Calibrated |
| --- | --- | --- | --- | --- |
| PC | Rack PC | Test computer | Yes | / |
| PSU-15 (4-wire) | NGL202 | 2 channel power supply | Yes | 10/10/2025 |
| SMU4 | KeysightB2900BL | 1 Ch Precision Source/measure unit | Yes | 13/09/2025 |
| EL1 | Keysight EL34143A | Electronic load capable of sinking 4A | Yes | 01/11/2025 |
| MULTI9 | Keysight 34461A | 6.5-digit basic Truevolt DMM | Yes | 05/05/2026 |
| SCOPE3 | Keysight DSOX1204G | 4 Analog Channels Oscilloscope | Yes | 13/12/2026 |
| THERMO STREAMER | Temptronic ECO-710E-M |  | TBC | 30/12/2025 |

## Statistical analysis

### Test limits

Test limits are defined in the excel file under post-processing and are valid for FT, MODULE and VIS test stages.

### Bin groups

Following table gives an overview of the bin groups. Bin 1 is always the PASS bin. Bin groups between 2 and 99 are reserved for PASS with limited performance, groups between 100 and 199 are FAIL but can be retested. Groups starting from 200 are FAILS and should be scrapped.

Table 5: Software bin table

| Bin group | Resolution |
| --- | --- |
| 1 | PASS |
| 100 | FAIL at short tests, retest can be done |
| 101 | FAIL at Pgood Ron param, retest can be done |
| 200 | FAIL at pin tests |
| 201 | FAIL at block level test |
| 202 | FAIL at application or system level test |
| 300 | FAIL at OTP block |

The hardware bins are simply: one pass and one fail bin.

### Dynamic test limits

This section list all parameters that fall under DPAT control.

Currently, no parameters are under DPAT control.

## QC test stages

No dedicated test programs for running QC version need to be implemented since:

- No NVM on this product

- No trim tests exist

- No stress tests that need to be skipped

During the QC runs the specification limits will be used as acceptance criteria.

# Measurement test setup

This chapter explains how to collect measurement data from individual DUTs while working with a limited amount of lab equipment. It also details the instruments used in the test setup, along with all hardware-related requirements. Additionally, the section covers the software used to control the test setup, including software-specific details.

## Test hardware

### Overview

Figure 3 provides an overview of the complete test setup used for most tests outlined in the test procedures (Chapter 5). As shown in the diagram, the Devices Under Test (DUTs) are placed within a temperature-controlled environment, which may be maintained using a thermostreamer to ensure stable and controlled test conditions. The DUTs are connected to various measurement instruments via a connection matrix, allowing flexible signal routing.

To enable automated test execution, all instruments are connected through Ethernet and centrally controlled by a test PC. This setup ensures efficient data acquisition, measurement synchronization, and automated execution of the defined test procedures.

Figure 3: Measurement setup overview

Figure 4: Detailed setup

### DUT information

The DUT pin configuration can be found in the datasheet MAG-PSU00001-NP. Table 6 provides a detailed overview of all DUT pins and specifies the corresponding test equipment required for each pin to execute the defined test procedures.

For debugging purposes, additional connections to the test equipment may be beneficial. These optional connections are marked with the postfix "∗" in the table. The following codes are used in the matrix:

HI_F: Forced voltage or current at the pin

HI_S, LO_S: Sensed voltage or current at the pin

LO_F: Return path

HI_IN: Sensed voltage or current at the pin connects to HI input of DMM or scope

LO_IN: Sensed voltage or current at the pin connects to LO input of DMM

CHx: Input of oscilloscope with x in range [1 to 4]

Table 6: Pin list vs equipment matrix

| Equip Pin | PSU | SMU | DMMV | SCOPE | EL | uC | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Phase |  |  |  | S, CH1 | F,S |  | Equipment is connected after the LC filter |
| Pvin | F,S |  |  |  |  |  |  |
| Vin | S |  |  |  |  |  | Pin shorted to Pvin on the board |
| Gnd |  |  | LO_S |  |  |  | Pin shorted to PGnd on the board |
| PGnd | LO_F |  |  |  |  |  |  |
| V33Dr |  | F,S | HI_IN |  |  |  |  |
| Rf |  | F,S |  |  |  |  |  |
| Pgood |  | F,S |  | S, CH2 |  | S |  |
| Vi |  | F,S |  |  |  |  |  |
| BootS |  |  |  |  |  |  |  |
| BGP_EA |  |  | HI_IN |  |  |  |  |
| Inv_Enable |  | F,S |  |  |  | F |  |
| HalfSW |  | F,S |  |  |  | F |  |
| Vout |  |  |  |  |  |  |  |
| PTAT |  |  | HI_IN |  |  |  |  |
| En |  | F,S |  |  |  | F |  |

### Pin load requirements

During the hardware development attention needs to be given to following items:

- Load cap specs

  - In order to ensure minimal coupling to sensitive nodes (Rf and Vi), the mux is partially implemented on the socket-board.

- Load res specs

  - The sink-current of Pgood is limited to 50μA

- Low-ohmic connections

  - Main power connection to the DUT needs to be able to drive 4A 10W

  - Secondary power connection to VIS module needs to be able to drive 500mA

- Kelvin connections

  - Muxed Kelvin connection on EXT_S

  - On the Power input and output terminals (after filter)

  - On the Power ground

### Daughter board and external components

A dedicated daughter board will be designed to serve as an interface between the DUT and the measurement board.

Most signals can be probed on test-board terminal EXT_S or V_TEST. These terminals are in this setup connected to an SMU and DMM respectively. Alternatively, the daughter board also has probe-points as defined in Table 7.

Table 7: Socket board probe-points

| Chip terminal | Board Label |
| --- | --- |
| V33Dr | LDO |
| Vin | VIN |
| PTAT | PTT |
| BGP_EA | BGP |
| Vout | VOUT |
| Phase | PHS |
| Gnd | GND |

Each daughter board will have a unique serial number for identification.

### Measurement board

The measurement board will serve as an interface between the daughter board and the power supplies, microcontroller, and other required equipment. These connections will be made using a combination of banana-connectors and SMA connectors. Additionally, the measurement board will function as a load board.

The design will support connections for a single daughter board (single-site testing). To ensure compatibility with different test environments, the measurement board will be designed for use with a thermostreamer.

Each measurement board will be assigned a unique serial number for identification.

All supply domains will be individually accessible on this board.

### Power supplies

Power is supplied to the DUT using bench Power Supply Units (PSUs). The setup contains two power domains. One main power-supply (VDD), that powers the DUT, and the other a 3.3V power-domain for the microcontroller and integrated mux.

The power-up and power-down sequence for default-connected supplies is detailed in Section 3.2.3. To prevent voltage overshoot, it is strongly recommended to use power sources with a controllable soft ramp.

Table 8: Power supplies

| DUT pin name | Supply domain (Table 1) | PSU channel | Alias | Comments |
| --- | --- | --- | --- | --- |
| Pvin | PSU-supply | 1 |  |  |
| Vin | PSU-supply | 1 |  |  |
| / | PSU-3v3 | 2 |  | For external components |

## Test software

### Overview

The software test suite is an in-house build environment using Python scripts to control following:

- settings for MaTeR and measurement equipment,

- connectivity between the DUTs and the equipment,

- supply and temperature control,

- microcontroller for e.g. interfacing with the DUT,

- logging of measurement data and meta data to a database and

- settings for the load board.

### Park state routine

Before each test procedure starts, the DUT and the test setup is brought to a known default state (park state). This includes driving following IO pins to:

Table 9: Park state pin biasing

| Pin name | Biased to level / dynamic signal (f, dc, swing) | Comment |
| --- | --- | --- |
| Phase | High-Z | Via relay (pin after filter) |
| Pvin | Supply 5V |  |
| Vin | Supply 5V |  |
| Gnd | Ground |  |
| PGdn | Ground |  |
| V33Dr | High-Z |  |
| Rf | Pull-down (270kΩ) | Via mux |
| Pgood | Pull-up (250kΩ) | Via relay |
|  | High-Z | Connected to micro-controller |
| Vi | Pull-down (1MΩ) | Via mux |
| BootS | High-Z |  |
| BGP_EA | High-Z | Via mux |
| Inv_Enable | Logic “0” | Driven by micro-controller |
| HalfSw | Logic “0” | Driven by micro-controller |
| Vout | High-Z |  |
| PTAT | High-Z | Via mux |
| En | Logic “0” | Driven by micro-controller |

### Power-down state routine

During CONN testing, the initial state is different from the park state. This Power-down state is a more conservative state. Here the 3v3 power-domain of the mux and micro-controller is powered on, but the main power-supply connected to Vin/Pvin isn’t. All terminals of the DUT are either high-Z or grounded.

Table 10: Power-down state pin biasing

| Pin name | Biased to level / dynamic signal (f, dc, swing) | Comment |
| --- | --- | --- |
| Phase | High-Z | Via relay |
| Pvin | Supply 0V |  |
| Vin | Supply 0V |  |
| Gnd | Ground |  |
| PGdn | Ground |  |
| V33Dr | High-Z |  |
| Rf | Pull-down (270kΩ) | Via mux |
| Pgood | No pull-up | Via relay |
|  | High-Z | Connected to micro-controller |
| Vi | Pull-down | Via mux |
| BootS | High-Z |  |
| BGP_EA | High-Z | Via mux |
| Inv_Enable | Logic “1” (0V) | Driven by micro-controller |
| HalfSw | Logic “1” (0V) | Driven by micro-controller |
| Vout | High-Z |  |
| PTAT | High-Z | Via mux |
| En | Logic “0” | Driven by micro-controller |

### Power cycle routines

Power-up sequence:

- Disconnect all measurement equipment

- Apply PSU-3v3 = PSU-supply = 0V

- Wait 10ms

- Ramp up PSU-3v3 to its defined operating condition 3.3V in 10ms or slower

- Wait 10ms

- Set IO’s as defined in the park state routine (section 3.2.2)

- Ramp up PSU-supply to its defined operating condition 5V-11V in 10ms or slower

Power-down sequence:

- Ramp down PSU-supply to 0V in 10ms or slower

- Ramp down PSU-3v3 to 0V in 10ms or slower

- Disconnect all measurement equipment

### Design for test (DfT) features

The DUT doesn’t have a build-in testmux or other DfT features. The following tables contain the configurations of the mux integrated in the setup. Here the chip-terminals are connected to measurement equipment via specific mux settings

Table 11: Test Mux chip connections

| Chip pin | Connections | Mux setting |  |
| --- | --- | --- | --- |
| PTAT | DMM | Tst_trim | 0x3 |
| BGP_EA | DMM | Tst_trim | 0x2 |
| Pgood | SMU Pull-up resistor | Ext_trim PGood_res | 0x6 0x1 |
| Vi | Pull-down-trim SMU | Vi_trim | Code; [kΩ]; Vout [V]. 0x0; 2200; 1.2. 0x1; 1000; 1.5. 0x2; 680; 1.8. 0x3; 500; 2. 0x4; 390; 2.5. 0x5; 250; 3.3. 0x6; 147; 5. 0x7; Connected to SMU; 0.9. |
| V33Dr | SMU DMM | Ext_trim Tst_trim | 0x1 0x1 |
| Rf | Pull-down-trim SMU | Rf_trim | Code; [kΩ]; [MHz]. 0x0; 270; 1.03. 0x1; 200; 1.35. 0x2; 180; 1.48. 0x3; 150; 1.65. 0x4; 120; 1.99. 0x5; 100; 2.51. 0x6; 82; 2.98. 0x7; Connected to SMU. |

Table 12: Test Mux Load terminal settings

| Load functionality | Comment | Trim setting |  |
| --- | --- | --- | --- |
| Channel-switch | In case of MAG-VIS4001x-N module-test, either one of the PSU chips can be chosen to connect to the measurement equipment. Not used for Socket tests | Channel 1: Load_Sw Channel 2: Load_Sw | 0x0 0x1 |
| Static-load | Some fixed resistive output loads can be provided for functional tests. | Load_res | Code; [Ω]. 0x0; -. 0x1; 270. 0x2; 50. 0x3; 10. |

Table 13: microcontroller pin settings

| Parameter name | Range | Park state | Power-down state |
| --- | --- | --- | --- |
| en | 0 / 1 | 0 | 0 |
| in_en | 0 / 1 | 0 | 0 |
| half_sw | 0 / 1 | 0 | 0 |
| rf_trim | 0x0 – 0x7 | 0 | 0 |
| rf_trim_en | 0 / 1 | 1 | 1 |
| vi_trim | 0x0 – 0x7 | 0 | 0 |
| vi_trim_en | 0 / 1 | 1 | 1 |
| tst_trim | 0x0 – 0x3 | 0 | 0 |
| ext_trim | 0x0 – 0x7 | 4 | 4 |
| ext_trim_en | 0 / 1 | 1 | 1 |
| load_sw | 0 / 1 | 1 | 1 |
| load_res | 0x0 – 0x3 | 0 | 0 |
| pgood_res | 0 / 1 | 1 | 0 |

# Test procedures

This chapter contains an overview of all the tests included in the test sequence(s). In the overview items as high-level description, test configuration, test conditions, detailed test procedure and acceptance criteria are discussed.

## T0001: Power Short

Table 14: Power Short overview

| Test Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Verify that there is no short to ground on the power-terminals of the chip |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |
| - Current measurement: accuracy: 100nA, range 100μA - Voltage source: accuracy: 10mV, range:100mV |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |
| Step | Action |  |  |  |  |
|  | Start from Power-down state |  |  |  |  |
|  | Ramp-up main power-supply on Vin/Pvin to 100mV |  |  |  |  |
|  | Wait 10ms |  |  |  |  |
|  | Measure the current consumption of the chip |  |  |  |  |
|  | Log current |  |  |  |  |
| Notes |  |  |  |  |  |
| This power-on test should always be performed before doing any other tests. Failure of this test will immediately result in a termination of the test procedure. Termination limit is set to 50mA. |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |
| M0001 | I_in_leakage | μA |  |  |  |

## T0002: Load Short

Table 14: Load Short overview

| Test Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Verify that there is no short to ground or power on the load-terminals of the chip |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |
| - Current measurement: accuracy: 100nA, range 100μA - Voltage source: accuracy: 10mV, range:100mV |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |
| Step | Action |  |  |  |  |
|  | Start from Power-down state |  |  |  |  |
|  | Ramp-up the voltage on the load to 100mV |  |  |  |  |
|  | Wait 10ms |  |  |  |  |
|  | Measure the current consumption of the chip |  |  |  |  |
|  | Log current |  |  |  |  |
| Notes |  |  |  |  |  |
| This power-on test should always be performed before doing any other tests. Failure of this test will immediately result in a termination of the test procedure. Termination limit is set to 50mA. |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |
| M0002 | I_out_leakage | μA |  |  |  |

## T0003: PSU calibration

Table 15: PSU calibration overview

| Test Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Measure the static load of the Sense-channel of the PSU. This can then be used for calibration. |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |
| This test is performed without DUT installed. - Calibration of chip-test setup Remove the chip from the socket and perform the calibration-sequence as manual test. - Calibration of module-test setup Remove the socket-module from the test-board and perform the calibration-sequence as manual test. |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |
| - Current measurement: accuracy: 100nA, range: 10mA - Voltage source: accuracy: 10mV, range: 11V, step:100mV |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |
| Step | Action |  |  |  |  |
|  | Start from Power-down state |  |  |  |  |
|  | Sweep main power-supply on Vin/Pvin to 11V |  |  |  |  |
|  | Wait 1s |  |  |  |  |
|  | Measure the current consumption at each of the sweep points |  |  |  |  |
|  | Log current consumption |  |  |  |  |
| Notes |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |
| M0003 | I_sense_leakage | μA |  |  |  |

## T0004: SMU calibration

Table 15: SMU calibration overview

| Test Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Measure the static load of the Sense-channel of the SMU. This can then be used for calibration. |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |
| pin-sweep = [‘en’, ‘inv_en’, ‘vi’, ‘half_sw’, ‘v33dr’ ‘pgood’, ‘rf’] This test is performed without DUT installed. - Calibration of chip-test setup Remove the chip from the socket and perform the calibration-sequence as manual test. - Calibration of module-test setup Remove the socket-module from the test-board and perform the calibration-sequence as manual test. |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |
| - Current measurement: accuracy: 100nA, range: 100μA - Voltage source: accuracy: 1mV, range: 3.3V, step:100mV |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |
| Step | Action |  |  |  |  |
|  | Start from Power-down state |  |  |  |  |
|  | Sweep the pin to be force-sensed by the SMU |  |  |  |  |
|  | Sweep voltage on the pin to 3.3V |  |  |  |  |
|  | Wait 1s |  |  |  |  |
|  | Measure the current consumption at each of the sweep points |  |  |  |  |
|  | Log current consumption |  |  |  |  |
| Notes |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |
| M0004 | I_sense_leakage | μA |  |  |  |

## T0005: Leakage test

Table 16: Leakage test overview

| Test Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Measure the input leakage of all accessible input pins |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |
| The measured digital input needs to have a high-Z connection to the micro-controller. |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |
| - Current measurement: accuracy: 10nA, range: 10μA - Voltage source: accuracy: 10mV, range: 3.3V |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |
| Step | Action |  |  | Ext_trim setting |  |  |
|  | Start from Park state |  |  | Code; Input terminal. 0x0; Rf. 0x2; Inv_En. 0x3; HalfSw. 0x5; Vi. 0x7; En. |
|  | Sweep the input terminal via Ext_trim |  |  |  |  |  |
|  | Sweep the input voltage [0, 3.3] |  |  |  |  |  |
|  | Wait 10ms |  |  |  |  |  |
|  | Measure the current sunk or sourced through the input pin |  |  |  |  |  |
|  | Log current |  |  |  |  |  |
| Notes |  |  |  |  |  |  |
| The leakage currents are logged in the database as positive values when the current direction is from the SMU source into the pin and negative values when the current flows from pin towards source. Note that pin En is loaded in the setup by the input impedance of the oscilloscope (1MOhm towards vss). Pin Vi is loaded by the resistive network (towards vss) to configure the output voltage on the socket board. |  |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |  |
| ID | Name | Unit | LTL |  | Typ | UTL |
| M0005 | IIL | μA |  |  |  |  |
| M0006 | IIH | μA |  |  |  |  |

## T0006: Input threshold

Table 17: Input threshold overview

| Test Description |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Find the both high and low threshold voltages VIL/VIH for the En pin |  |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |  |
| - Sense Pgood state - Voltage source: accuracy: 10mV, range: 3.3V, step:10mV |  |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |  |
| Step | Action |  |  | Mux parameter |  | code |  |
|  | Start from Park state |  |  |  |  |  |  |
|  | Select pin En to be forced by the SMU |  |  | Ext_trim |  | 0x7 |  |
|  | Sweep input voltage 0V → 3.3V |  |  |  |  |  |  |
|  | Wait 10ms |  |  |  |  |  |  |
|  | sense the Pgood state, log the voltage at the PUT when the Pgood pin is asserted as VIH |  |  |  |  |  |  |
|  | Sweep input voltage 3.3V → 0V |  |  |  |  |  |  |
|  | Wait 10ms |  |  |  |  |  |  |
|  | sense the Pgood state, log the voltage at the PUT when the Pgood pin is de-asserted as VIL |  |  |  |  |  |  |
|  | Log the thresholds and hysteresis of each pin |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| To save test time, during production testing the input sweep can be replaced by go/no-go testing |  |  |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |  |  |
| ID | Name | Unit | LTL |  | Typ |  | UTL |
| M0007 | VIH | V |  |  |  |  |  |
| M0008 | VIL | V |  |  |  |  |  |
| M0035 | Hyst | V |  |  |  |  |  |

## T0007: Output drive-strength

Table 18: Output drive-strength overview

| Test Description |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Measure the current that can be sunk by the Pgood. |  |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |  |
| - Current Measurement: accuracy: 500nA, range 100uA - Voltage source: accuracy: 10mV, range: 3.3V, points:[50mV, 500mV, 3.3V], CC:50μA |  |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |  |
| Step | Action |  |  | Mux parameter |  | Code |  |
|  | Start from Park state |  |  |  |  |  |  |
|  | Disconnect pull-up on Pgood |  |  | Pgood_res |  | 0x0 |  |
|  | Select pin Pgood to be force-sensed by the SMU |  |  | Ext_trim |  | 0x6 |  |
|  | Sweep SMU voltage [50mV, 500mV, 3.3V] limiting the current to 50μA |  |  |  |  |  |  |
|  | Wait 10ms |  |  |  |  |  |  |
|  | Log Pgood current |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |  |  |
| ID | Name | Unit | LTL |  | Typ |  | UTL |
| M0009 | Ron | Ω |  |  |  |  |  |
| M0010 | IOL | μA |  |  |  |  |  |
| M0011 | IOH | μA |  |  |  |  |  |

## T0008: On-chip regulator

Table 19: On-chip regulator overview

| Test Description |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Measure voltage of regulator and load regulation test |  |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |  |
| - Current sink: accuracy: 100μA, range:50mA - Voltage measurement: accuracy: 10mV, range: 3.6V |  |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |  |
| Measure the load regulation at both minimal and maximal level for pVin, also during production testing. |  |  |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |  |
| Step | Action |  |  | Mux parameter |  | code |  |
|  | Start from Park state |  |  |  |  |  |  |
|  | Select pin V33Dr to be forced by the SMU |  |  | Ext_trim |  | 0x1 |  |
|  | Select pin V33Dr to be sensed by DMM |  |  | Tst_trim |  | 0x1 |  |
|  | Set the supply voltage |  |  |  |  |  |  |
|  | Sweep load current on V33Dr 0mA → 50mA, 1mA step |  |  |  |  |  |  |
|  | Wait 10ms |  |  |  |  |  |  |
|  | Measure the voltage on V33Dr |  |  |  |  |  |  |
|  | Calculate load regulation parameter (k_50ma=deltaV/deltaI) |  |  |  |  |  |  |
|  | Log V33Dr voltage |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| The pull-down resistor on the V33Dr is placed by default. This resistor is needed for stability. |  |  |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |  |  |
| ID | Name | Unit | LTL |  | Typ |  | UTL |
| M0012 | K_50mA | mV/mA |  |  |  |  |  |
| M0036 | V33dr | V |  |  |  |  |  |

## T0009: Under Voltage Lock Out

Table 20: Under Voltage Lock Out overview

| Test Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Find the threshold-voltages where the chip goes into and out of UVLO |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |
| - Voltage source: accuracy: 10mV, range: 4V - 6V step: 10mV - Voltage measurement: accuracy: 10mV |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |
| Step | Action |  |  |  |  |
|  | Start from Park state |  |  |  |  |
|  | Enable EL |  |  |  |  |
|  | Write a logic 1 to En |  |  |  |  |
|  | Sweep the voltage of the main power-supply 4V → 6V |  |  |  |  |
|  | Wait 10ms |  |  |  |  |
|  | Measure the output-voltage of the DUT using the EL |  |  |  |  |
|  | Log the threshold voltage where the chip goes out of reset |  |  |  |  |
|  | Sweep the voltage of the main power-supply 6V → 4V |  |  |  |  |
|  | Wait 10ms |  |  |  |  |
|  | Measure the output-voltage of the DUT using the EL |  |  |  |  |
|  | Log the threshold voltage where the chip goes into reset |  |  |  |  |
|  | Log the output voltage as a function of the input-voltage sweeps |  |  |  |  |
| Notes |  |  |  |  |  |
| To save test time, during production testing the input sweep can be replaced by go/no-go testing |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |
| M0013 | UVLO_H | V |  |  |  |
| M0014 | UVLO_L | V |  |  |  |
| M0037 | UVLO_hyst | V |  |  |  |

## T0010: Output voltage monitor

Table 21: Output voltage monitor overview

| Test Description |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Check the ±6.5% limits of the control-loop by forcing Vi. |  |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |  |
| During this test the control loop of the DUT is intentional broken, the output voltage will not be regulated, even more it will be beneficial for the accuracy of this test that the PWM controller is forced to a fixed position, this will keep the power stage in one state, reducing self-heating and cleaner supply, ground and substrate potentials. |  |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |  |
| - Voltage source: accuracy: 2mV, range: 500mV – 700mV step: 5mV - Voltage measurement: accuracy: 10mV, range: 3.6V, high-Z |  |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |  |
| Step | Action |  |  | Mux parameter |  | code |  |
|  | Start from Park state |  |  |  |  |  |  |
|  | Connect Vi to the SMU |  |  | Ext_trim Vi_trim |  | 0x5 0x7 |  |
|  | Connect BGP_EA to DMM |  |  | Tst_trim |  | 0x2 |  |
|  | Write a logic 1 to En |  |  |  |  |  |  |
|  | Measure and log the BGP_EA voltage |  |  |  |  |  |  |
|  | Sweep the voltage on Vi 500mV → 700mV |  |  |  |  |  |  |
|  | Wait 10ms |  |  |  |  |  |  |
|  | Monitor the Pgood state |  |  |  |  |  |  |
|  | Log the threshold voltage where Pgood becomes logic 1 in absolute value |  |  |  |  |  |  |
|  | Log the threshold voltage where Pgood becomes logic 0 in absolute value |  |  |  |  |  |  |
|  | Sweep the voltage on Vi 700mV → 500mV |  |  |  |  |  |  |
|  | Monitor the Pgood state |  |  |  |  |  |  |
|  | Log the threshold voltage where Pgood becomes logic 1 in absolute value |  |  |  |  |  |  |
|  | Log the threshold voltage where Pgood becomes logic 0 in absolute value |  |  |  |  |  |  |
|  | Log the Pgood state as function of control-voltage sweeps. Log the lowest threshold where Pgood goes from low to high state relative to the V_BGR in units of % and the highest threshold where Pgood goes from high to low state relative to the V_BGR in units of %. |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |  |  |
| ID | Name | Unit | LTL |  | Typ |  | UTL |
| M0015 | V_BGR | mV |  |  | 600 |  |  |
| M0016 | Vth_Vi_rise_L | mV |  |  | V_BGP_typ×93.5% |  |  |
| M0017 | Vth_Vi_fall_L | mV |  |  | V_BGP_typ×93.5% |  |  |
| M0018 | Vth_Vi_rise_H | mV |  |  | V_BGP_typ×106.5% |  |  |
| M0019 | Vth_Vi_fall_H | mV |  |  | V_BGP_typ×106.5% |  |  |
| M0038 | UV_Pgood_thres | % |  |  | -6.5 |  |  |
| M0039 | OV_Pgood_thres | % |  |  | 6.5 |  |  |

## T0011: Over Temperature Protection

Table 22: Over Temperature Protection overview

| Test Description |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Check the OTP via Pgood and PTAT voltage. |  |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |  |
| During this test the control loop of the DUT is intentional broken, the output voltage will not be regulated, even more it will be beneficial for the accuracy of this test that the PWM controller is forced to a fixed position, this will keep the power stage in one state, reducing self-heating and cleaner supply, ground and substrate potentials. A dedicated test stage OPT_CHAR was created since this test requires multiple temperature points to calculate the OTP thresholds. Also this test falls out of the normal operation condition HT. However the test can also be executed during CHAR or FT test stage. For these cases the thresholds will not be found but the PTAT level can be measured. |  |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |  |
| - Voltage source: accuracy: 10mV, range: 3.3 - Voltage measurement: accuracy: 10mV, range: 3.6V, high-Z - Temperature: accuracy: 2°C, range:-40°C-120°C, step: 5°C |  |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |  |
| From 60 degrees on use 5 degrees temperature steps |  |  |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |  |  |
| Full temperature range will not be needed to characterize OTP behaviour but will be determined by experiment during test development. |  |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |  |
| Step | Action |  |  | Mux parameter |  | code |  |
|  | Start from Park state |  |  |  |  |  |  |
|  | Connect PTAT to DMM |  |  | Tst_trim |  | 0x3 |  |
|  | Write a logic 1 to En |  |  |  |  |  |  |
|  | Sweep the temperature 70°C → 125°C step 5°C |  |  |  |  |  |  |
|  | Wait for the temperature to settle |  |  |  |  |  |  |
|  | Check Pgood |  |  |  |  |  |  |
|  | Log the threshold temperature where the chip goes into OTP |  |  |  |  |  |  |
|  | Measure the PTAT voltage |  |  |  |  |  |  |
|  | Sweep the temperature 125°C → 60°C step 5°C |  |  |  |  |  |  |
|  | Wait for the temperature to settle |  |  |  |  |  |  |
|  | Check Pgood |  |  |  |  |  |  |
|  | Log the threshold temperature where the chip goes out of OTP |  |  |  |  |  |  |
|  | Measure the PTAT voltage |  |  |  |  |  |  |
|  | Log the Pgood state versus temperature-sweeps |  |  |  |  |  |  |
|  | Log the PTAT voltage versus temperature-sweeps |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Calculate PTAT slope in postprocessing |  |  |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |  |  |
| ID | Name | Unit | LTL |  | Typ |  | UTL |
| M0020 | Tth_OTP_H | °C |  |  | 103 |  |  |
| M0021 | Tth_OTP_L | °C |  |  | 70 |  |  |
| M0022 | V_PTAT | V |  |  |  |  |  |

## T0012: Startup behaviour

Table 23: Startup behaviour overview

| Test Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Measure the start-up behaviour of the output voltage and check the level/time where Pgood flag goes high |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |
| - Scope voltage: Load and Pgood ¦ accuracy: 10mV, range: 6V |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |
| Perform this test for the 3 different load resistors |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |
| Step | Action |  |  |  |  |
|  | Start from Park state |  |  |  |  |
|  | Enable scope-channels |  |  |  |  |
|  | Start measuring the voltage-transient of Pgood, En and Load |  |  |  |  |
|  | Write a logic 1 to En |  |  |  |  |
|  | Wait 2ms |  |  |  |  |
|  | Stop measuring the voltage-transient of Pgood, En and Load |  |  |  |  |
|  | Log the voltage at node Load when Pgood is asserted and the startup time |  |  |  |  |
| Notes |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |
| M0023 | V_Start | V |  |  |  |
| M0024 | t_Start | s |  |  |  |

## T0013: Inv_Enable functionality

Table 24: Inv_Enable functionality overview

| Test Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Check that the functionality of En can be inverted using Inv_Enable |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |
| Step | Action |  | En state | Inv_En state |  |
|  | Start from Park state |  |  |  |  |
|  | Wait 10 ms |  |  |  |  |
|  | Read Pgood state |  | 0 | 0 |  |
|  | Write 1 to En |  |  |  |  |
|  | Wait 10ms |  |  |  |  |
|  | Read Pgood state |  | 1 | 0 |  |
|  | Write 1 to Inv_Enable |  |  |  |  |
|  | Wait 10ms |  |  |  |  |
|  | Read state of Pgood |  | 1 | 1 |  |
|  | Write 0 to En |  |  |  |  |
|  | Wait 10 ms |  |  |  |  |
|  | Read state of Pgood |  | 0 | 1 |  |
|  | Log the states of Pgood |  |  |  |  |
| Notes |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |
| M0025 | Inv_en_ok |  | Pass / Fail |  |  |

## T0014: Line regulation

Table 25: Line regulation overview

| Test Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Measure the input voltage-dependency of the output voltage |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |
| System test |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |
| - source voltage: accuracy: 10mV, range: 5V - 11V - measure voltage: accuracy: 10mV, range: 6V |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |
| Fixed load condition of 100mA |  |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |
| Step | Action |  |  | Vi_trim settings |  |  |
|  | Start from Park state |  |  | Code; Output voltage. 0x0; 1.2. 0x1; 1.5. 0x2; 1.8. 0x3; 2. 0x4; 2.5. 0x5; 3.3. 0x6; 5. 0x7; 0.9. |
|  | Enable the electronic load for voltage measurement and current sinking |  |  |  |  |  |
|  | Sweep target output-voltage via Vi_trim |  |  |  |  |  |
|  | Sweep input voltage 5V → 11V , 1V step |  |  |  |  |  |
|  | Wait 10ms |  |  |  |  |  |
|  | Measure the load voltage |  |  |  |  |  |
|  | Log the load voltage versus input voltage |  |  |  |  |  |
| Notes |  |  |  |  |  |  |
| Depending on the test time, only one output voltage could be sufficient during production testing |  |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |  |
| ID | Name | Unit | LTL |  | Typ | UTL |
| M0026 | V_line_delta | mV |  |  |  |  |

## T0015: Load regulation

Table 26: Load regulation overview

| Test Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Measure the load-current dependency of the output voltage |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |
| System test |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |
| - sink current: accuracy: 1mA, range: 4A - measure voltage: accuracy: 10mV, range: 6V |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |
| Measure the load regulation at both minimal and maximal level for pVin, also during production testing. |  |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |
| Step | Action |  |  | Vi_trim settings |  |  |
|  | Start from Park state |  |  | Code; Output voltage. 0x0; 1.2. 0x1; 1.5. 0x2; 1.8. 0x3; 2. 0x4; 2.5. 0x5; 3.3. 0x6; 5. 0x7; 0.9. |
|  | Enable the electronic load for voltage measurement and current sinking |  |  |  |  |  |
|  | Sweep target output-voltage via Vi_trim |  |  |  |  |  |
|  | Sweep load current 0A → 1.2A, 200mA step |  |  |  |  |  |
|  | Wait 10ms |  |  |  |  |  |
|  | Measure the load voltage |  |  |  |  |  |
|  | Log the load voltage versus input voltage, calculate the output voltage drop between min and max current load and log this parameter as V_load_delta |  |  |  |  |  |
| Notes |  |  |  |  |  |  |
| A load-current of more than 1A can only be achieved when the chip is sufficiently cooled. During production the number of loads steps can be reduced and the output voltage can be set to 1 single setting. |  |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |  |
| ID | Name | Unit | LTL |  | Typ | UTL |
| M0027 | V_load_delta | mV |  |  |  |  |

## T0016: Efficiency

Table 27: Efficiency overview

| Test Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Efficiency over different loads and Vin/Vout conditions |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |
| System test |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |
| - source voltage: accuracy: 10mV, range: 5V - 12V - source current: accuracy: 1mA, range: 2.4A - sink current: accuracy: 1mA, range: 4A - measure voltage: accuracy: 10mV, range: 6V |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |
| Beside the test conditions defined in chapter 2 following test conditions are applied during this test: - Halfsw functionality: enabled and disabled - Rf = 150kΩ - All Vout settings available on the test hardware board - input voltages: [5V, 7V, 9V, 11V] - load currents: [0.02A, 0.2A, 0.5A, 1A, 2A, 3A, 4A] |  |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |
| Step | Action |  |  | Vi_trim settings |  |  |
|  | Start from Park state |  |  | Code; Output voltage. 0x0; 1.2. 0x1; 1.5. 0x2; 1.8. 0x3; 2. 0x4; 2.5. 0x5; 3.3. 0x6; 5. 0x7; 0.9. |
|  | Enable Electronic load for voltage monitoring and current forcing |  |  |  |  |  |
|  | Optionally set HalfSw |  |  |  |  |  |
|  | Sweep target output-voltage via Vi_trim |  |  |  |  |  |
|  | Sweep the input voltage 5V → 11V |  |  |  |  |  |
|  | Sweep load current 0A → 1A |  |  |  |  |  |
|  | Wait 10ms |  |  |  |  |  |
|  | Measure the load voltage and current |  |  |  |  |  |
|  | Measure the input voltage and current |  |  |  |  |  |
|  | Remove load current and wait to cool down |  |  |  |  |  |
|  | Log the efficiency of the system |  |  |  |  |  |
| Notes |  |  |  |  |  |  |
| A DC load-current of more than 1A can only be achieved when the chip is sufficiently cooled. When HalfSw is enabled, the maximum load-current is further reduced to 800mA During production testing the number of test conditions can be reduced to the worst case combinations or application conditions. |  |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |  |
| ID | Name | Unit | LTL |  | Typ | UTL |
| M0028 | Efficiency | % |  |  |  |  |

## T0017: Output voltage ripple

| Test Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Measure ripple amplitude and frequency |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |
| System test |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |
| - scope voltage: accuracy: 10mV, range: 6V, high-Z - sink current: accuracy: 1mA, range: 4A - measure voltage: accuracy: 10mV, range: 6V |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |
| Beside the test conditions defined in chapter 2 following test conditions are applied during this test: - Halfsw functionality: disabled - Rf = 150kΩ - All Vout settings available on the test hardware board - 4 different input voltages: [5V, 7V, 9V, 11V] - 1 load currents: 0.2A TBC |  |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |
| Step | Action |  |  | Vi_trim settings |  |  |
|  | Start from Park state |  |  | Code; Output voltage. 0x0; 1.2. 0x1; 1.5. 0x2; 1.8. 0x3; 2. 0x4; 2.5. 0x5; 3.3. 0x6; 5. 0x7; 0.9. |
|  | Enable EL and Scope |  |  |  |  |  |
|  | Sweep target output voltage via Vi_trim |  |  |  |  |  |
|  | Sweep the supply-voltage |  |  |  |  |  |
|  | Write a logic 1 to En |  |  |  |  |  |
|  | Sweep the load current |  |  |  |  |  |
|  | Wait 5ms |  |  |  |  |  |
|  | Measure amplitude and frequency of the ripple |  |  |  |  |  |
|  | Log amplitudes and frequencies |  |  |  |  |  |
| Notes |  |  |  |  |  |  |
| During production testing the number of test conditions can be reduced to the worst case combinations or application conditions. |  |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |  |
| ID | Name | Unit | LTL |  | Typ | UTL |
| M0029 | Frequency | kHz |  |  |  |  |
| M0030 | Pk_pk_ripple | mV |  |  |  |  |

## T0018: VIS module test

| Test Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Measure the ripple and average output voltage for the two outputs. |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |
| The socket-board is swapped with the Vision-stack DCDC-board. |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |
| - scope voltage: accuracy: 10mV, range: 6V, high-Z - source voltage: accuracy: 10mV, range 5V – 12V, step 1V - measure voltage: accuracy: 10mV, range: 6V |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |
| Beside the test conditions defined in chapter 2 following test conditions are applied during this test: - 4 different input voltages: [5V, 7V, 9V, 11V] - 1 load current: 0.5A |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |
| Step | Action |  |  |  |  |
|  | Start from Park state |  |  |  |  |
|  | Enable Scope |  |  |  |  |
|  | Sweep supply voltage 5V → 12V |  |  |  |  |
|  | Write a logic 1 to En |  |  |  |  |
|  | Sweep output channel via Load_Sw: 0x0, 0x1 |  |  |  |  |
|  | Wait 5ms |  |  |  |  |
|  | Measure the voltage-transient |  |  |  |  |
|  | Log voltage transient |  |  |  |  |
| Notes |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |
| M0031 | V_out | V |  |  |  |
| M0032 | Pk_Pk_ripple | mV |  |  |  |

## T0019: Transient load

| Test Description |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| An expected load profile is applied to the chip. This load-profile mimics the imager and serializer chips that will be powered by the DUT when integrated into a camera unit |  |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |  |
| Application test |  |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |  |
| - scope voltage: accuracy: 10mV, range: 6V, high-Z - source voltage: accuracy: 10mV, range: 5V - 12V, step 1V - sink current: accuracy: 1mA, range: 4A - measure voltage: accuracy: 10mV, range: 6V |  |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |  |
| A realistic application load current profile should be added to the EL memory to make this test meaningful, as long as this profile is not clear this test should not be implemented |  |  |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |  |
| Step | Action |  |  | Rf_trim settings |  | Vi_trim settings |  |
|  | Start from Park state |  |  | Code; Rf [kΩ]. 0x0; 270. 0x1; 200. 0x2; 180. 0x3; 150. 0x4; 120. 0x5; 100. 0x6; 82. | Code; Output voltage. 0x0; 1.2. 0x1; 1.5. 0x2; 1.8. 0x3; 2. 0x4; 2.5. 0x5; 3.3. 0x6; 5. 0x7; 0.9. |
|  | Enable EL and Scope |  |  |  |  |  |  |
|  | Set target output voltage via Vi_trim |  |  |  |  |  |  |
|  | Sweep input supply voltage Vin |  |  |  |  |  |  |
|  | Sweep the load current |  |  |  |  |  |  |
|  | Write a logic 1 to En |  |  |  |  |  |  |
|  | Wait 5ms |  |  |  |  |  |  |
|  | Apply transient load-profile |  |  |  |  |  |  |
|  | Measure the voltage-transient |  |  |  |  |  |  |
|  | Log voltage transient |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |  |  |
| ID | Name | Unit | LTL |  | Typ |  | UTL |
| M0033 | V_out | V |  |  |  |  |  |
| M0034 | Pk_Pk_ripple | mV |  |  |  |  |  |

Bibliography

List of Acronyms

| Acronym | Definition |
| --- | --- |

Idm@F4E #