# Characterization and product test plan

Subject: MAG-CXP00002

Author: Bert Boons version; Date; Reviewed; Approved by. 1v1; 2025-11-18; Jonathan Vanden Berk; Jonathan Vanden Berk.

Revision history

| Version | Date | Revisions |
| --- | --- | --- |
| 1v0 | 2025-11-17 | First version Reviewed by: Jonathan Vanden Berk Approved by: Jonathan Vanden Berk |
| 1v1 | 2025-11-18 | Reviewed to be in line with implementation. This revision is used for the TRR meeting on 19/11/2025 |
| 1v2 | 2025-12-03 | Video data pinnen geupdate in test plan voor de pin leakage test |

Table of Contents

Revision history	2

Table of Contents	3

List of Figures	5

List of Tables	6

1	Document purpose and scope	8

2	Test strategy	9

2.1	Test conditions	9

2.2	Test stages	9

2.2.1	Characterization (CHAR)	10

2.2.2	Trimming test (TRIM)	10

2.2.3	Final Test (FT)	10

2.2.4	Vision CXP sub-board testing (VIS)	10

2.3	Test flow	10

2.4	Test sequence	14

2.5	Trimming sequence	16

2.6	Test time	16

2.7	Test equipment	17

2.8	Statistical analysis	18

2.8.1	Test limits	18

2.8.2	Bin groups	18

2.8.3	Dynamic test limits	18

2.9	QC test stages	18

3	Measurement test setup	20

3.1	Test hardware	20

3.1.1	Overview	20

3.1.2	DUT information	21

3.1.3	Pin load requirements	23

3.1.4	Daughter board and external components	24

3.1.5	Measurement board	25

3.1.6	Power supplies	26

3.2	Test software	27

3.2.1	Overview	27

3.2.2	Park state routine	27

3.2.3	Power cycle routines	29

3.2.4	Design for test (DfT) features	30

4	Test procedures	32

4.1	T0001: Continuity Test	33

4.2	T0002: Power Short Test	34

4.3	T0003: Pin leakage Test	35

4.4	T0004: VIL VIH Test	38

4.5	T0005: VOH VOL Test	41

4.6	T0006: IOL IOH Test	44

4.7	T0007: IDDD Test	45

4.8	T0008: Test Buffer Offset Test	47

4.9	T0009: Block Trimming Test	49

4.10	T0010: Block Sweep Test	51

4.11	T0011: Static Signal Test	53

4.12	T0012: Regulators Test	56

4.13	T0013: Oscillator Test	58

4.14	T0014: PLL Open Loop Test	59

4.15	T0015: PLL Lock Test	61

4.16	T0016: IO SPI Slave Test	62

4.17	T0017: IO SPI Master CAM Test	63

4.18	T0018: IO SPI Master GPIO Test	63

4.19	T0019: Eye Diagram Test	65

4.20	T0020: CXP Uplink Receiver Sensitivity Test	65

4.21	T0021: CXP Uplink Receiver Sensitivity Sweep Test	66

4.22	T0022: Internal Test Patterns Test	67

4.23	T0023: Camera Test	69

4.24	T0024: OTP Write Test	70

4.25	T0025: OTP Data Retention Test	72

4.26	T0026: VIS VDD3V3_DCDC test	72

5	Bibliografie	74

List of Acronyms	75

List of Figures

Figure 1: MAG-CXP00002-NP production test flow	11

Figure 2: MAG-VIS4003x-N production test flow	12

Figure 3: MAG-CXP00002-NP characterization test flow	13

Figure : MAG-CXP00002-NP qualification test flow	14

Figure : Measurement setup overview	21

Figure : RF network	25

List of Tables

Table 1: Supply voltage conditions	9

Table 2: Test stages	9

Table 3: Test sequence per test stage	14

Table 4: Trimming sequence	16

Table : Estimated Test Time (TT) per test stage	16

Table : Measurement Equipment overview	17

Table : Software bin table	18

Table : Pin list vs equipment matrix	22

Table : External components	24

Table : RF network component summary	25

Table : Power supplies	26

Table 12: FPGA IO supply state	26

Table : Park state pin biasing	27

Table : Additional register settings after reset of DUT.	29

Table : Trim registers	29

Table : test mux summary	30

Table 17: Subsets of pins	32

Table : T0001: Continuity Test Overview	33

Table : T0002: Power Short Test Overview	34

Table : T0003: Pin Leakage Test Overview	35

Table : T0004: VIL VIH Test Overview	38

Table : T0005: VOH VOL Test Overview	41

Table : T0006: IOL IOH Test Overview	44

Table : T0007: IDDD Test Overview	45

Table : T0008: Test Buffer Offset Test Overview	47

Table : T0009: Block Trimming Test Overview	49

Table : T0010: Block Sweep Test Overview	51

Table : T0011: Static Signal Test Overview	53

Table : T0012 Regulators Test Overview	56

Table :  T0013: Oscillator Test Overview	58

Table : T0014: PLL Open Loop Test Overview	59

Table : T0015: PLL Lock Test Overview	61

Table : T0016: IO SPI Slave Test Overview	62

Table : T0017: IO SPI Master CAM Test Overview	63

Table : T0018: IO SPI Master GPIO Test Overview	63

Table : T0020: CXP Uplink Receiver Sensitivity Test Overview	65

Table : T0021: CXP Uplink Receiver Sensitivity Sweep Test Overview	66

Table : T0022: Internal Test Patterns Test Overview	67

Table : T0023: Camera Test Overview	69

Table : T0024: OTP Write Test Overview	70

Table : T0025: OTP Data Retention Test Overview	72

Table : T0026: VIS VDD3V3_DCDC Test Overview	72

# Document purpose and scope

This document is part of the MAG-CXP Product Testing project and defines the test strategy, test setup and the detailed test procedures to perform electrical verification, functional verification and radiation testing on the product.

This document also provides a detailed test schedule and lists test facilities.

All the information in this document is intended to describe the verification tests that will be done on the product mentioned above. Any major deviations from this plan will be discussed prior or at latest during the test readiness review (TRR) meeting with the customer. Major changes will be listed in the revision history. Major changes can be (list is not exclusive):

Number of DUT’s

Test conditions

Hardware/equipment changes

Test facility changes

Addition or removal of tests

Any update to a test procedure that changes the test coverage or testing strategy

- Minor changes are kept in the revision history but not explicitly informed to the customer. These changes are limited to:

- Small changes in a test procedure, e.g. additional SPI commands, increasing wait times

- Correction of typo’s

Any changes (major and minor) after the TRR meeting will be discussed with the customer.

# Test strategy

This chapter describes the test methods used for the verification and validation of the MAG_CXP00002_NP product. The test strategy aims at maximal test coverage of the product.

## Test conditions

During functional verification and characterization testing the temperature and supply voltage of the DUT’s will be varied to validate the ASIC specifications over the defined operating conditions. The conditions listed in this section are the minimal set of conditions that will be tested. If additional test conditions are needed to validate certain parameters, then they are explicitly mentioned in the test procedure.

Temperature conditions are mentioned in this document with following codes:

- RT = room temperature = 25°C +/- 5°C

- CT = cold temperature = -40°C +/- 5°C

- HT = hot temperature = 85°C +/- 5°C

Supply voltage conditions are defined in the table below:

Table 1: Supply voltage conditions

| Supply level | Minimal | Nominal | Maximal | Comments |
| --- | --- | --- | --- | --- |
| vdd3v3 | 2.97 | 3.3V | 3.63V |  |

## Test stages

Table 2 outlines all the test stages required to validate the performance of the test vehicle, while section 2.2.2 details the sequence in which these test stages fit into the overall test flow.

Table 2: Test stages

| Name | Code | Temperature | Supply | Other conditions | Number of parts | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Production and screening tests |  |  |  |  |  |  |
| Trimming | TRIM | RT | Vnom | / | All |  |
| Final test | FT | RT, TBD | TBD | / | All | TBD will be updated after analysing of CHAR results for at least 100 parts |
| Vision CPX sub-board test | VIS | RT | Vnom | / |  | MAG-VIS4003x-N |
| Device validation tests |  |  |  |  |  |  |
| Characterization | CHAR | RT, CT, HT | Vnom, Vmin, Vmax | / | 100 | These test conditions are minimal, more points can be added. |

The following sections provide further details on the different test stages.

### Characterization (CHAR)

The characterization stage is intended to test the electrical requirements of the DUTs over the operating conditions (temperature and supply) as defined in section 2.1.

During the verification phase of the product development lifecycle the number of parts used in the analysis can be smaller compared to the number used in validation phase. The full characterization as part of the device validation phase should be performed on a number of parts that is statistically meaningful. The test coverage should be similar in both phases. If not, the rationally for changing test coverage must be addressed in the characterization report.

### Trimming test (TRIM)

The trimming stage is intended obtain the best-fit trim codes for the building blocks. These code are programmed in the NVM memory at the end of this test stage.

### Final Test (FT)

The final test stage is intended to verify that the chip is within specifications. This is a reduced test compared to the characterization (CHAR). Where the CHAR test does a high resolution sweep over parameters to confirm the performance of the IC, the FT stage is more focussed on detecting bad parts.

### Vision CXP sub-board testing (VIS)

The Vision sub-board test stage is performed on the CXP sub-board of the MAG-VIS400xx PCBs,  in order to verify the functionality of the board. An external pattern is applied to the video input pins, mimicking the output of a imager. If the captured image from the CXP interface has no pixel errors, the test functionality of the board is considered to be proven. One additional test in the test stage is to measure the output voltage of the DCDC that is located on the same PCB. Note: this needs a manual change to the test setup.

## Test flow

This section defines the test flow that every part will follow. For the production testing of the serializer chip (MAG-CXP00002-NP) itself the test flow is given in Figure 1. The test flow for the VIS PCB is given in Figure 2.

Figure 3 shows the test flow of the chip (MAG-CXP00002-NP) that needs to be followed during characterization of the product.  Figure 4 shows the test flow for the qualification testing of the chip (MAG-CXP00002-NP).

Figure 1: MAG-CXP00002-NP production test flow

Figure 2: MAG-VIS4003x-N production test flow

Figure 3: MAG-CXP00002-NP characterization test flow

Figure 4: MAG-CXP00002-NP qualification test flow

## Test sequence

Table 3 provides an overview of the various tests conducted at each test stage and outlines the correct test sequence, which should be followed from top to bottom.

Table 3: Test sequence per test stage

| Test | TRIM | FT | CHAR | VIS | Comments |
| --- | --- | --- | --- | --- | --- |
| T0001: Continuity Test | X | X | X |  |  |
| T0002: Power Short | X | X | X |  |  |
| T0003: Pin leakage | X | X | X |  |  |
| T0004: VIL VIH |  | X | X |  |  |
| T0005: VOH VOL |  | X | X |  |  |
| T0006: IOL IOH |  | X | X |  |  |
| T0007: IDDD | X | X | X |  |  |
| T0008: Test Buffer Offset | X | X | X |  |  |
| T0009: Block Trimming | X |  |  |  |  |
| T0010: Block Sweep |  |  | X |  |  |
| T0011: Static Signal Test |  | X | X |  |  |
| T0012: Regulators |  | X | X |  |  |
| T0013: Oscillator |  | X | X |  |  |
| T0014: PLL Open Loop |  | X | X |  |  |
| T0015: PLL Lock |  | X | X |  |  |
| T0016: IO SPI Slave | X | X | X |  |  |
| T0017: IO SPI Master CAM |  | X | X | X |  |
| T0018: IO SPI Master GPIO |  | X | X | X |  |
| T0019: Eye Diagram |  |  | X |  | 20251118: currently this test is not yet implemented |
| T0020: CXP Uplink Receiver Sensitivity |  | X | X |  |  |
| T0021: CXP Uplink Receiver Sensitivity Sweep Test |  |  | X |  |  |
| T0022: Internal Test Patterns |  | X | X | X |  |
| T0023: Camera Test |  | X | X | X |  |
| T0024: OTP Write | X |  |  |  |  |
| T0025: OTP Data Retention Test | X | X | X | X |  |
| T0026: VDD3V3 DCDC test |  |  |  | X |  |

## Trimming sequence

In order to start tuning all trim settings it is necessary to respect the block level hierarchy of the DUT. To first trim top level blocks and after that to trim blocks that depend on other blocks. Therefore a specific trimming sequence is written down in Table 4.

Table 4: Trimming sequence

| Step | Test number | Description | Trim target |
| --- | --- | --- | --- |
|  | T0009 | BIAS LDO trim | 1.2V |
|  | T0009 | Bandgap reference trim | 650mV |
|  | T0009 | Bandgap current trim | 10µA |
|  | T0009 | Analog LDO trim | 1.25V |
|  | T0009 | Digital LDO trim | 1.2V |
|  | T0009 | XO LDO trim | 1V |
|  | T0009 | PLL LDO trim | 1.25V |
|  | T0009 | CP LDO trim | 1V |
|  | T0009 | LDIV LDO trim | 1V |
|  | T0009 | VCO LDO trim | 1.25V |
|  | T0009 | ODIV LDO trim | 1V |
|  | T0009 | PFD LDO trim | 1V |
|  | T0009 | CAM LDO trim | 1.8V |
|  | T0009 | CAM ref trim | 1.2V |
|  | T0024 | OTP write | / |

## Test time

Test times in the table below are valid for one temperature and 1 supply level.

Table 5: Estimated Test Time (TT) per test stage

| Test | TRIM [s] | FT_FAST [s] | CHAR [s] | VIS [s] | Comments |
| --- | --- | --- | --- | --- | --- |
| T0001 Continuity | 4.6 | 4.6 | 4.6 |  |  |
| T0002 Power Short | 2 | 2 | 2 |  |  |
| T0003 Pin leakage | 11.9 | 11.9 | 11.9 |  |  |
| T0004 Digital input hysteresis |  | 102.8 | 805 |  |  |
| T0005 Digital output voltage |  | 20.1 | 20.1 |  |  |
| T0006 Digital output current |  | 20.1 | 20.1 |  |  |
| T0007 Current Consumption | 19.2 | 19.2 | 19.2 |  |  |
| T0008 Test buffer offset | 12.2 | 12.2 | 12.2 |  |  |
| T0009 Block trim | 103 |  |  |  |  |
| T0010 Block trim sweep |  |  | 61.9 |  |  |
| T0011 Static Signals |  | 18.6 | 18.6 |  |  |
| T0012 LDO Load |  | 143.6 | 143.6 |  |  |
| T0013 Oscillator |  | 3.6 | 3.6 |  |  |
| T0014 PLL Open Loop |  | 7.1 | 7.1 |  |  |
| T0015 PLL lock |  | 4.5 | 4.5 |  |  |
| T0016 Debug SPI Slave | 2.8 | 2.8 | 2.8 |  |  |
| T0017 Camera SPI Master |  | 6.9 | 6.9 |  |  |
| T0018 GPIO SPI Master |  | 5.9 | 5.9 |  |  |
| T0019 Eye diagram |  |  | ? |  |  |
| T0020 Receiver sensitivity |  | 28.9 | 28.9 |  |  |
| T0021 Receiver sensitivity sweep |  |  | 162.6 |  |  |
| T0022 Test patterns |  | 8.6 | 285.6 | 8.6 |  |
| T0023 External video |  | 37.2 | 37.2 | 37.2 |  |
| T0024 OTP write | 12.8 |  |  |  |  |
| T0025 OTP data retention | 6 | 6 | 6 |  |  |
| T0026 VIS VDD3V3_DCDC |  |  |  | 5.8 |  |
| Total test time: | 178.5 (~3min) | 466.7 (~8min) | 1670.3 (~28min) | 51.6 (~1min) | Excluding transition times thermostreamer |
| Target test time: NA |  |  |  |  |  |

## Test equipment

All test equipment outlined in the hardware overview is listed in Table 6. These tables provide recommendations for the required instruments during the different test, indicate their availability based on the test schedule, and specify whether the equipment remains calibrated throughout the various test phases.

Table 6: Measurement Equipment overview

| Instrument name | Instrument type | Short description | Available | Calibrated |
| --- | --- | --- | --- | --- |
| PC | Rack PC | Test computer | Yes | NA |
| Measurement board | MAG CXP Measurement board | Dedicated measurement board for MAG CXP | Yes | NA |
| Daughter board | MAG CXP Socket board. | Dedicated socketboard for MAG CXP | Yes | NA |
| Module | MAG CXP Vision Module | Module holding the MAG CXP chip | Yes | NA |
| PSU (2-wire), Channels 1, 2, 3 and 4 in section 3.1.6 | NGL202 (2x) | 2 channel power supply | Yes | Yes |
| SMU | KeysightB2901BL | 1 Ch Precision Source/measure unit | Yes | Yes |
| DMMV | Keysight 34465A | 6.5-digit basic Truevolt DMM | Yes | No (until 05/06/25) |
| DMMI | Keysight 34465A | 6.5-digit basic Truevolt DMM | Yes | No (until 05/06/25) |
| SCOPE | Keysight DSOX1204G | 4 Analog Channels Oscilloscope | Yes | Yes |
| HS SCOPE | Keysight DSOX3102G | 2 Analog Channels Oscilloscope | Yes | ? |
| THERMO STREAMER | Temptronic ECO-710E-M |  | TBC | Yes |

As listed in Table 6,custom equipment and boards are required to test the DUT. The measurement board is described in section 3.1.5. The daughter board is described in section 3.1.4. The module is part of the larger vision stack, which utilizes the daughter board and is pin compatible. This makes it possible to also utilize the setup to test assembled the MAG CXP on assembled MAG-VIS40031-N modules. For more information about the modules refer to [1]

## Statistical analysis

### Test limits

Test limits are defined in the excel file under post-processing and are valid for FT, TRIM and VIS test stages

### Bin groups

Following table gives an overview of the bin groups. Bin 1 is always the PASS bin. Bin groups  between 2 and 99 are reserved for PASS with limited performance, groups between 100 and 199 are FAIL but can be retested. Groups starting from 200 are FAILS and should be scrapped.

Table 7: Software bin table

| Bin group | Resolution |
| --- | --- |
| 1 | PASS |
| 100 | FAIL at continuity test |
| 200 | FAIL at pin test |
| 201 | FAIL at IDD |
| 202 | FAIL at block level test |
| 203 | FAIL in TRIM sweep |
| 204 | FAIL in TRIMMED parameter |
| 205 | FAIL at interface |
| 206 | FAIL at application or system test |
| 207 | FAIL in OTP |
| 208 | FAIL of DCDC component (only used for VIS test stage) |

The hardware bins are simply: one pass and one fail bin.

### Dynamic test limits

This section list all parameters that fall under DPAT control.

Currently, no parameters are under DPAT control.

## QC test stages

The TRIM test stage needs a QC version that:

- Prevents the OTP to be written during the QC stage

- Trim tests are executed but the best fit code will not be used in the rest of the test program

The other test stages don’t need a QC version.

During the QC runs the specification limits will be used as acceptance criteria.

# Measurement test setup

This chapter explains how to collect measurement data from individual DUTs while working with a limited amount of lab equipment. It also details the instruments used in the test setup, along with all hardware-related requirements. Additionally, the section covers the software used to control the test setup, including software-specific details.

## Test hardware

### Overview

Figure 5 provides an overview of the complete test setup used for most tests outlined in the test procedures (Chapter 4). As shown in the diagram, the Devices Under Test (DUTs) are placed within a temperature-controlled environment, which may be maintained using a thermostreamer or temperature chamber to ensure stable and controlled test conditions. The DUTs are connected to various measurement instruments via a connection matrix, allowing flexible signal routing.

To enable automated test execution, all instruments are connected through Ethernet and centrally controlled by a test PC. This setup ensures efficient data acquisition, measurement synchronization, and automated execution of the defined test procedures.

Figure 5: Measurement setup overview

### DUT information

To limit the use of equipment channels, the DUTs are measured using a time-multiplexed approach, ensuring that only one device is tested at a time (single-site test setup).

The DUT pin configuration can be found in the datasheet [2]. Table 8 provides a detailed overview of all DUT pins and specifies the corresponding test equipment required for each pin to execute the defined test procedures.

For debugging purposes, additional connections to the test equipment may be beneficial. These optional connections are marked with the postfix "∗" in the table. The following codes are used in the matrix:

HI_F: Forced voltage or current at the pin

HI_S, LO_S: Sensed voltage or current at the pin

LO_F: Return path

HI_IN: Sensed voltage or current at the pin connects to HI input of DMM or scope

LO_IN: Sensed voltage or current at the pin connects to LO input of DMM

HI_FS, LO_FS: Forced and sensed voltage at pin

CHx: Input of oscilloscope with x in range [1 to 4]

- The SMU matrix output pin can also be connected to SCOPE channel 2. This scope channel has no separate switch matrix structure, the connection goes via the SMU switch matrix.

Table 8: Pin list vs equipment matrix

| Equip Pin | PSU | SMU | FGEN | SCOPE | HS SCOPE | Frame Grabber | FPGA | DMM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PIXEL_CLK_CAM |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| VIDEO_DATA_11 |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| VIDEO_DATA_10 |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| VIDEO_DATA_9 |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| VIDEO_DATA_8 |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| VIDEO_DATA_7 |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| VIDEO_DATA_6 |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| VSS | LO_F | LO_FS |  |  |  |  | LO_F | LO_S |
| VSS_DIG | LO_F | LO_FS |  |  |  |  | LO_F | LO_S |
| VIDEO_DATA_5 |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| VIDEO_DATA_4 |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| VIDEO_DATA_3 |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| VIDEO_DATA_2 |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| VIDEO_DATA_1 |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| VIDEO_DATA_0 |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| MASTER_SPI_MISO_CAM |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| MASTER_SPI_MOSI_CAM |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| MASTER_SPI_CLK_CAM |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| MASTER_SPI_CSB_CAM |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| TRIGGER_CAM |  | HI_FS |  | CH1 |  |  | HI_S |  |
| RESET_CAM |  | HI_FS |  | CH1 |  |  | HI_S |  |
| LDO_DIG_OUT |  | HI_FS |  | CH1 |  |  |  |  |
| SLAVE_SPI_MOSI |  | HI_FS |  | CH1 |  |  | HI_F |  |
| SLAVE_SPI_MISO |  | HI_FS |  | CH1 |  |  | HI_S |  |
| SLAVE_SPI_CLK |  | HI_FS |  | CH1 |  |  | HI_F |  |
| SLAVE_SPI_CSB |  | HI_FS |  | CH1 |  |  | HI_F |  |
| RESETN |  | HI_FS |  | CH1 |  |  | HI_F |  |
| VDD2V5_OTP |  | HI_F |  |  |  |  |  |  |
| VDD3V3_DIG | HI_F |  |  |  |  |  |  |  |
| VDD3V3_ANA | HI_F |  |  |  |  |  |  |  |
| LDO_ANA_OUT |  | HI_FS |  | CH1 |  |  |  |  |
| VSS_ANA | LO_F | LO_FS |  |  |  |  | LO_F | LO_S |
| TRXN |  |  | HI_F |  | CH1 | HI_F |  |  |
| TRXP |  |  |  |  |  |  |  |  |
| VDD3V3_CAM | HI_F |  |  |  |  |  |  |  |
| VREF_CAM |  | HI_S |  | CH1 |  |  |  |  |
| LDO_CAM_OUT |  | HI_FS |  | CH1 |  |  |  |  |
| LDO_VCO_OUT |  | HI_FS |  | CH1 |  |  |  |  |
| REF_CLK |  | HI_F |  | CH1 |  |  | HI_F |  |
| XTALN |  |  |  |  |  |  |  |  |
| XTALP |  |  |  |  |  |  |  |  |
| LDO_PLL_OUT |  | HI_FS |  | CH1HI_S |  |  |  |  |
| LDO_BIAS_OUT |  | HI_FS |  | CH1HI_S |  |  |  |  |
| VDD3V3_PLL | HI_F |  |  |  |  |  |  |  |
| OSC |  | HI_FS |  | CH1 |  |  | HI_S |  |
| GPIO_0 (testbus) |  | HI_FS |  | CH1 |  |  | HI_FS | HI_S |
| GPIO_1 (testbus) |  | HI_FS |  | CH1 |  |  | HI_FS | HI_S |
| GPIO_2 (testbus) |  | HI_FS |  | CH1 |  |  | HI_FS | HI_S |
| GPIO_3 (testbus) |  | HI_FS |  | CH1 |  |  | HI_FS | HI_S |
| GPIO_4 |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| GPIO_5 |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| GPIO_6 |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| GPIO_7 |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| GPIO_8 |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| GPIO_9 |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| GPIO_10 |  | HI_FS |  | CH1 |  |  | HI_FS |  |
| FRAME_VALID_CAM |  | HI_FS |  | CH1 |  |  | HI_F |  |
| LINE_VALID_CAM |  | HI_FS |  | CH1 |  |  | HI_F |  |
| DATA_VALID_CAM |  | HI_FS |  | CH1 |  |  | HI_F |  |
| THERMAL PAD | LO_F | LO_F |  |  |  |  | LO_F | LO_S |

### Pin load requirements

During the hardware development attention needs to be given to following items:

- Load cap specs

  - Should be limited to 100pF for digital IO’s.

- Low-ohmic connections

  - For ground and power connections.

- Shielded/guarded signals

  - All connections between the function generator, scope, SMU and the pins of the ASIC should be done with coax or triax cables. Pay attention to the shielding of these signals.

- Impedance matched signals

  - TRXP/TRXN: The TRXN pin is connected to a Coax transmission line leading to the frame grabber, while the TRXP pin is terminated on the substrate board. Matching must adhere to the 75Ω transmission line standard.

### Daughter board and external components

A dedicated daughter board will be designed to serve as an interface between the DUT and the measurement board.

The socket should have sufficient performance for high frequency signals, mainly for the TRXP,TRXN output signals. This signal should also be impedance matched on the daughter board to 75 Ohms.

Due to simplicity of the daughter board which holds the socket, no additional special test points will be added to the board.

Each daughter board will have a unique serial number for identification.

#### External components

- In Table 9 the external components for each daughter board are listed. Decoupling/stabilization capacitors should be placed as close to the pin as possible to reduce ESR.

Table 9: External components

| Terminal A | Terminal B | Type | Value | Comments |
| --- | --- | --- | --- | --- |
| VDD3V3_DIG | GND | Decoupling Capacitor | 100 nF | C13 |
| VDD3V3_DIG | GND | Decoupling Capacitor | 100 nF | C14 |
| VDD3V3_DIG | GND | Decoupling Capacitor | 1 uF | C15 |
| VDD3V3_PLL | GND | Decoupling Capacitor | 100 nF | C16 |
| VDD3V3_CAM | GND | Decoupling Capacitor | 100 nF | C17 |
| LDO_DIG_OUT | GND | Decoupling Capacitor | 100 nF | C10 |
| LDO_ANA_OUT | GND | Decoupling Capacitor | 1 uF | C9 |
| LDO_CAM_OUT | GND | Decoupling Capacitor | 1 uF | C8 |
| LDO_PLL_OUT | GND | Decoupling Capacitor | 100 nF | C7 |
| LDO_BIAS_OUT | GND | Decoupling Capacitor | 100 nF | C6 |
| VDD2V5_OTP | GND | Decoupling Capacitor | 100 nF | C12 |
| VDD2V5_OTP | GND | Pull down resistor | 10 kOhm | R5 |
| XTAL_P | XTAL_N | Crystal | 20 MHz | Y1 |
| XTAL_P | GND | Coupling Capacitor | 20 pF | C19 |
| XTAL_N | GND | Coupling Capacitor | 20 pF | C20 |

Figure 6 depicts the RF network in use, this network is also placed on the daughter board. Positioned on the left side is the Serializer ASIC, featuring down-link driver pins TRXN and TRXP. As CoaXPress operates in a single-ended fashion, the TRXP pin is connected to ac-ground, presenting the same impedance (R3) as the TRXN pin experiences through the transmission line. Given the absence of on-chip resistors in the Serializer ASIC to convert the current from the LVDS driver to voltage, external resistors (R1 and R2) are employed. These external resistors (R1 and R2) serve the dual purpose of converting current to voltage in parallel to the transmission line impedance and acting as termination resistance for the uplink signal. In Table 10 all components are summarized. [2]

Figure 6: RF network

Table 10: RF network component summary

| Designator | Type | Value | Comment |
| --- | --- | --- | --- |
| R1 | Resistor | 75Ω | Current conversion / uplink termination |
| R2 | Resistor | 75Ω | Current conversion / uplink termination |
| R3 | Resistor | 75Ω | Impedance matching resistor |
| C1 | Capacitor | 1nF | Current conversion / uplink termination |
| C2 | Capacitor | 1nF | Current conversion / uplink termination |
| C3 | Capacitor | 56nF | Coupling capacitor RF signal |
| C4 | Capacitor | 56nF | Coupling capacitor RF signal |

### Measurement board

The measurement board will serve as an interface between the daughter board and the connection matrix, power supplies, FPGA, and other required equipment. These connections will be made using a combination of flat cables, SMA connectors, and power connectors. Additionally, the measurement board will function as a load board.

The design will support connections for a single daughter board (single-site testing). To ensure compatibility with different test environments, the measurement board will be designed for use with a thermostreamer.

Each measurement board will be assigned a unique serial number for identification.

All supply domains will be individually accessible on this board.

The measurement has the following requirements:

- Respect the pin requirements which are already described in section 3.1.3.

- Implement the sufficient muxing capabilities on the measurement board itself.

  - Simplifies wiring of the setup compared to using MaTeR MUX AC/DC cards.

  - Use solid state switches as much as possible to reduce area required for the board.

  - The required muxing capabilities are described in section 3.1.2.

- Test points should be foreseen for each signal pin of the DUT, to ease up debugging.

- An FPGA should be placed on the measurement board itself, to ensure signal integrity of the high speed video interface. The FPGA can also be used to control the muxes on the measurement board.

- A dedicated mux for the TRX signal:

  - Matched to 75 Ohms.

  - Use RF relays for minimal insertion loss.

- Add LEDs to all switch and relay controls to aid in debugging.

### Power supplies

Power is supplied to the DUT using bench Power Supply Units (PSUs). All DUT supply channels are listed in Table 11, where the "default connected" column indicates whether a channel is connected by default during testing unless otherwise specified in the test procedure. The supply can either be directly connected to the pin, via a relay or via an DMMI for most accurate supply current measurements. The PSU channels can be split over different bench equipment depending on availability and sourcing/measurement requirements.

The power-up and power-down sequence for default-connected supplies is detailed in Section 3.2.3. To prevent voltage overshoot, it is strongly recommended to use power sources with a controllable soft ramp.

Table 11: Power supplies

| DUT pin name | Supply domain (Table 1) | Default connected | PSU channel | Alias | Comments |
| --- | --- | --- | --- | --- | --- |
| VDD3V3_ANA | vdd3v3 | Yes | 2 |  |  |
| VDD3V3_PLL | vdd3v3 | Yes | 3 |  | Connected to VDD3V3_DIG |
| VDD3V3_DIG | vdd3v3 | Yes |  |  | Connected to VDD3V3_PLL |
| VDD3V3_CAM | vdd3v3 | Yes | 4 |  |  |
| VDD2V5_OTP | vdd2v5 | No | SMU |  |  |

The supply for the FPGA’s IO bank which interfaces with the DUT can be selected from the options given in Table 12. This ensures that maximum flexibility for different kinds of tests, to minimize the influence of the FPGA for the different types of tests. Note that only the supply connection to the FPGA’s IO bank can be altered, the ground connection of the FPGA’s IO supply is always connected.

Table 12: FPGA IO supply state

| Mode | Source | Comments |
| --- | --- | --- |
| Float | Not connected to any supply | To be used during the measurement of the IIL parameter in the leakage test (not IIH!) |
| Board | Main 3.3 V supply of the measurement board | To be used during continuity test, power short test and IDD test |
| DUT | Digital IO voltage from DUT, VDD3V3_DIG. | To be used during all other tests, also when measuring the IIH parameter in the leakage test. |

## Test software

### Overview

The software test suite is an in-house build environment using Python scripts to control following:

- settings for MaTeR and measurement equipment,

- connectivity between the DUTs and the equipment,

- supply and temperature control,

- FPGA for e.g. serial interfacing with the DUT,

- logging of measurement data and meta data to a database and

- settings for the load board.

### Park state routine

Before each test procedure starts the DUT and the test setup is brought to a known default state (park state). This includes driving following IO pins to:

Table 13: Park state pin biasing

| Pin name | Biased to level dynamic signal (f, dc, swing) | Comment |
| --- | --- | --- |
| PIXEL_CLK_CAM | HI-Z |  |
| VIDEO_DATA_11 | HI-Z |  |
| VIDEO_DATA_10 | HI-Z |  |
| VIDEO_DATA_9 | HI-Z |  |
| VIDEO_DATA_8 | HI-Z |  |
| VIDEO_DATA_7 | HI-Z |  |
| VIDEO_DATA_6 | HI-Z |  |
| VSS | GND |  |
| VSS_DIG | GND |  |
| VIDEO_DATA_5 | HI-Z |  |
| VIDEO_DATA_4 | HI-Z |  |
| VIDEO_DATA_3 | HI-Z |  |
| VIDEO_DATA_2 | HI-Z |  |
| VIDEO_DATA_1 | HI-Z |  |
| VIDEO_DATA_0 | HI-Z |  |
| MASTER_SPI_MISO_CAM | HI-Z |  |
| MASTER_SPI_MOSI_CAM | HI-Z |  |
| MASTER_SPI_CLK_CAM | HI-Z |  |
| MASTER_SPI_CSB_CAM | HI-Z |  |
| TRIGGER_CAM | HI-Z |  |
| RESET_CAM | HI-Z |  |
| LDO_DIG_OUT | HI-Z | LDO Output |
| SLAVE_SPI_MOSI | 0 V |  |
| SLAVE_SPI_MISO | HI-Z | Digital output pin |
| SLAVE_SPI_CLK | 0 V | Driven by FPGA. |
| SLAVE_SPI_CSB | 3.3 V | Debug SPI Slave not selected. Driven by FPGA. |
| RESETN | 3.3 V | Reset released, driven by FPGA. |
| VDD2V5_OTP | 0 V |  |
| VDD3V3_DIG | 3.3 V | Connected to PSU |
| VDD3V3_ANA | 3.3 V | Connected to PSU |
| LDO_ANA_OUT | HI-Z | LDO Output |
| VSS_ANA | GND |  |
| TRXN | Frame Grabber | DC level of this pin cannot be controlled due to blocking cap. |
| TRXP | Frame Grabber | AC Coupled to VSS, shield of coaxial cable. |
| VDD3V3_CAM | 3.3 V | Connected to PSU |
| VREF_CAM | HI-Z | LDO Output |
| LDO_CAM_OUT | HI-Z | LDO Output |
| LDO_VCO_OUT | HI-Z | LDO Output |
| REF_CLK | HI-Z |  |
| XTALN | Connected to XTAL |  |
| XTALP | Connected to XTAL |  |
| LDO_PLL_OUT | HI-Z | LDO Output |
| LDO_BIAS_OUT | HI-Z | LDO Output |
| VDD3V3_PLL | 3.3 V | Connected to PSU |
| OSC | HI-Z |  |
| GPIO_0 | HI-Z |  |
| GPIO_1 | HI-Z |  |
| GPIO_2 | HI-Z |  |
| GPIO_3 | HI-Z |  |
| GPIO_4 | HI-Z |  |
| GPIO_5 | HI-Z |  |
| GPIO_6 | HI-Z |  |
| GPIO_7 | HI-Z |  |
| GPIO_8 | HI-Z |  |
| GPIO_9 | HI-Z |  |
| GPIO_10 | HI-Z |  |
| FRAME_VALID_CAM | HI-Z |  |
| LINE_VALID_CAM | HI-Z |  |
| DATA_VALID_CAM | HI-Z |  |
| THERMAL PAD | GND |  |

- First all relays towards the test equipment are set to non-conducting state.

- Secondly, pin RESETN is connected to the FPGA and asserted, after this the reset is released. This reset will clear all register settings to default.

- Then, the debug SPI slave interface is connected to the FPGA and the frame grabber is connected to the CXP interface.

- Followed by the additional configuration listed in Table 14.

Table 14: Additional register settings after reset of DUT.

| Register | Value | Description |
| --- | --- | --- |
| Analog_en_sys_out | 0 | Disable refence clock output on OSC pin. This to avoid crosstalk through the muxes to other sensitive analog signals. |

During the trim flow, when the OTP is not yet written, each execution of the park state also reloads the trim settings which were measured so far. All trim related settings which can be configured during park state are summarized in Table 15. When a trim setting is not yet measured, the default setting will not be altered in the DUT’s register map.

Table 15: Trim registers

Register

Analog_trim_ldo_bias

Analog_trim_vireftempco

Analog_trim_vref

Analog_trim_iref10u

Analog_trim_ldo_ana

Analog_trim_ldo_dig

Analog_trim_ref_ldo

Analog_trim_ldo_pll

Analog_trim_cpldo

Analog_trim_ldivldo

Analog_trim_vcoldo

Analog_trim_odivldo

Analog_trim_pfdldo

Analog_trim_ldo_cam

Analog_trim_cam_1v2_buf

### Power cycle routines

Power-up sequence:

- Disconnect all measurement equipment

- Disconnect all digital signals coming from the FPGA

- Connect RESETN pin to FPGA and drive to 0V

- Switch FPGA IO Voltage to VDD3V3_DIG.

- Turn on supply VDD3V3_DIG to 3.3V current limit: 100mA

- Turn on supply VDD3V3_ANA to 3.3V current limit: 100mA

- Turn on supply VDD3V3_CAM to 3.3V current limit: 100mA

- Set IO’s as defined in the park state routine (section 3.2.2)

Power-down sequence:

- Disconnect all measurement equipment

- Disconnect all digital signals coming from the FPGA

- Turn off supply VDD3V3_CAM

- Turn off supply VDD3V3_ANA

- Turn off supply VDD3V3_DIG

### Design for test (DfT) features

Table 16 Lists all internal test signals that are available via one of the test mux pins. The test mux is controlled via the register map of the ASIC. A lot of these signals will be accessed during the tests described in this test procedure. Although some signals could be accessed via external pads directly it is better to use the TMUX during automated testing because attaching cables to all pads and routing it to multiplexers can introduce wrong loading of LDOs and noise injection through these cables.

Table 16: test mux summary

| Signal | Block | Mux register | Mux setting | Analog Buffer | Access Pin |
| --- | --- | --- | --- | --- | --- |
| hyb_out | hybrid | tstmux_a1sel | 1 |  | GPIO0 |
| ldo_dig | ldo_dig |  | 2 | N |  |
| trx_n | lvds |  | 3 |  |  |
| ldo_pll | ldo_pll |  | 4 | N |  |
| ldo_ana | ldo_ana |  | 5 | N |  |
| vtoi_crt | vtoi |  | 6 | N |  |
| ptat_crt | vref |  | 7 | N |  |
| cpout | cp |  | 9 | Y |  |
| vco_crt | vco |  | 10 | N |  |
| cam_buf | cam_buf |  | 11 | Y |  |
| ldo_bias | ldo_bias |  | 12 | N |  |
| vref | ref |  | 13 | Y |  |
| lvds_cm | lvds |  | 14 |  |  |
| vdd_xo | ldo_xo | tstmux_a2sel | 1 | N | GPIO1 |
| vddvco | ldo_vco |  | 2 | N |  |
| trx_p | lvds |  | 3 |  |  |
| vddldiv | ldo_ldiv |  | 4 | N |  |
| vddcp | ldo_cp |  | 5 | N |  |
| vref_ptat | vref |  | 6 | Y |  |
| vddpfd | ldo_pfd |  | 7 | N |  |
| vss_pll | pll |  | 8 | N |  |
| vddodiv | ldo_odiv |  | 9 | N |  |
| vtune | pll |  | 10 | Y |  |
| hyb_cm | hybrid |  | 11 |  |  |
| vss_dig | digital |  | 13 | N |  |
| vtoi_ota_out | vtoi |  | 14 | Y |  |
| rx_out | rx | tstmux_d1sel | 1 | NA | GPIO2 |
| ldivout | ldo_ldiv |  | 2 | NA |  |
| d_ct<0> | pll |  | 3 | NA |  |
| cp_up | cp |  | 4 | NA |  |
| ref_out | ref |  | 5 | NA |  |
| lvds_predrv_p | lvds |  | 6 | NA |  |
| ser_out | ser | tstmux_d2sel | 1 | NA | GPIO3 |
| clkref | pll |  | 2 | NA |  |
| d_ct<1> | pll |  | 3 | NA |  |
| cp_dn | cp |  | 4 | NA |  |
| lvds_predrv_n | lvds |  | 5 | NA |  |
| baseline_out | hybrid |  | 6 | NA |  |

On chip test buffers are used when measuring internal voltages without sufficient drive strength. The test buffer is automatically enabled when selecting these voltages via the test mux. In the tests the offset of the buffers is compensated before the value of the internal voltages is reported.

# Test procedures

This chapter contains an overview of all the tests included in the test sequence(s). In the overview items as high-level description, test configuration, test conditions, detailed test procedure and acceptance criteria are discussed.

Some tests are repeated on multiple pins. To avoid listing all pins every time such test is described, sometimes a subset of pins described in Table 8 is referenced. These subsets are summarized in Table 17.

Table 17: Subsets of pins

| Subset | Pins |
| --- | --- |
| All pins | See Table 8 |
| Digital Inputs | data_valid_cam frame_valid_cam gpio_0 gpio_1 gpio_2 gpio_3 gpio_4 gpio_5 gpio_6 gpio_7 gpio_8 gpio_9 gpio_10 line_valid_cam master_miso_cam pixel_clk_cam ref_clk resetn slave_csb slave_mosi slave_spi_clk video_data_0 video_data_1 video_data_2 video_data_3 video_data_4 video_data_5 video_data_6 video_data_7 video_data_8 video_data_9 video_data_10 video_data_11 |
| Digital Outputs | gpio_0 gpio_1 gpio_2 gpio_3 gpio_4 gpio_5 gpio_6 gpio_7 gpio_8 gpio_9 gpio_10 master_csb_cam master_mosi_cam master_spi_clk_cam osc reset_cam slave_miso trigger_cam |
| Digital pins | Digital Inputs + Digital Outputs |

## T0001: Continuity Test

Table 18: T0001: Continuity Test Overview

| Test Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Contact test on both digital and analog pins. During this test the DUT’s power supplies are turned off. Serially, each pin is connected to the SMU and a small current is pushed into the pin. The voltage that’s being measured, should be equivalent to the voltage drop over the ESD diode connected to the measured pin. When no such drop is measured, the pin is not connected properly in the package or the DUT is not connected to the socket pin. |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |
| During this test the FPGA IO supply is connected to the FPGA supply source. All IO pins of the FPGA that connect to the DUT are set High-Z |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |
| SMU with: - Source current of 1mA, with 1uA accuracy - Measure voltage accuracy with 10mV |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |
| During this test the DUT is supplied with 0V on all power domains, instead of the usual 3.3 V. |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |
| This is an automated tests that can be used in thermal controlled environment. |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |
| Step | Action |  |  | Equipment setting | Value |
|  | Disconnect FPGA from DUT, set all output buffers tri-state. |  |  |  |  |
|  | Configure SMU. to source current of 1000 uA with a voltage limit of 1.2 V. |  |  | Mode | Current |
|  |  |  |  | Output Current | 1mA |
|  |  |  |  | Voltage limit | 1.2 V |
|  | Connect FPGA IO bank supply to the board’s 3.3V supply. |  |  |  |  |
|  | Configure the DUT power supplies to 0V and turn on all DUT power supplies. |  |  |  |  |
|  | Start of pin loop |  |  |  |  |
|  | Connect the (next) IO pin to the SMU. |  |  |  |  |
|  | Turn on SMU. |  |  |  |  |
|  | Wait 50 ms. |  |  |  |  |
|  | Measure the voltage and save to database. |  |  |  |  |
|  | Turn off SMU. |  |  |  |  |
|  | Disconnect the pin from the SMU. |  |  |  |  |
|  | End of pin loop, go to step 5 if more pins are remaining. |  |  |  |  |
|  | Restore FPGA connections to DUT. |  |  |  |  |
| Notes |  |  |  |  |  |
| This test the continuity of all pins, except: - trxp, trxn: High speed CXP interface. To reduce parasitic load on this interface, these pins were omitted. They are tested implicitly during tests which make use of this CXP interface. - xtalp, xtaln: These are connected directly to the crystal, and are omitted for this test to reduce the parasitic load on these pins. They are tested implicitly during the oscillator test. Pin name will be added as test condition in the measurement log 100uA current was not sufficient at HT to deliver the leakage current of the on-chip decoupling capacitors on the digital domain, hence the current was increased to 1mA for all pins Sinking current into the SMU was not done since the paths towards the FPGA cannot be disconnected, Hence it will not be possible to detect defects on the DUT with this test |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |
| M0001 | T0001_M0001_pin_voltage | mV |  |  |  |

## T0002: Power Short Test

Table 19: T0002: Power Short Test Overview

| Test Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| This test checks for shorts on the DUT's power rails by supplying a very low voltage. In case there is a short, the resulting current will be small and will prevent further damage to DUT, socket and test equipment. The DUT is powered on with a very low supply voltage. Then the current of each power domain is measured from the power supply. These should be close to zero, as the supplied voltage is too low to actually turn on the DUT. |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |
| During this test the FPGA IO supply is connected to the FPGA supply source. All IO pins of the FPGA that connect to the DUT are set High-Z |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |
| A power supply with: - 100 mV output voltage with 10 mV accuracy - 100 uA current measurement with 10 nA accuracy |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |
| During this test the DUT is supplied with only 100 mV on all power domains, instead of the usual 3.3 V. |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |
| This test will be executed in an automated way and in a temperature controlled environment |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |
| Step | Action |  | Equipment setting |  | Value |
|  | Connect FPGA IO bank supply to the board’s 3.3V supply. |  |  |  |  |
|  | Configure power supply for all power domains. |  | Output Voltage |  | 100 mV |
|  |  |  | Current Limit |  | 100 mA |
|  | Disconnect FPGA from DUT, set all output buffers tri-state. |  |  |  |  |
|  | Turn on the power supplies, the order doesn’t matter. |  |  |  |  |
|  | Measure the current of each power domain and save to database. |  |  |  |  |
|  | Turn off all power supplies |  |  |  |  |
|  | Restore FPGA connections to DUT. |  |  |  |  |
| Notes |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |
| M0002 | T0002_M0002_vdd3v3_ana_current | uA |  |  |  |
| M0004 | T0002_M0004_vdd3v3_dig_current | uA |  |  |  |
| M0005 | T0002_M0005_vdd3v3_cam_current | uA |  |  |  |

## T0003: Pin leakage Test

Table 20: T0003: Pin Leakage Test Overview

| Test Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| This test checks the leakage current of each input pin or output pin which can be put into tri-state. The SMU will apply a low and high voltage on each pin and measure the leakage current into each pin. |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |
| During this test the FPGA IO supply is floating. All IO pins of the FPGA that connect to the DUT are set High-Z |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |
| An SMU with: - Source voltage range: 3.3V - Source voltage accuracy: 10 mV - Measure current range: 10 uA - Measure current accuracy: 10 nA |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |
| No deviations from chapter 2 applies to this test. |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |
| This test will be executed in an automated way in a temperature controlled environment |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |
| Step | Action |  |  | Equipment setting | Value |
|  | Start from park state |  |  |  |  |
|  | If applicable, configure the pin as input or tri-state. |  |  |  | Input configuration table |
|  | Configure the SMU as a voltage source |  |  | Current Limit | 100 uA |
|  | Disconnect FPGA from DUT, set all output buffers tri-state. |  |  |  |  |
|  | Connect SMU to pin and wait until muxes are settled. |  |  |  |  |
|  | Set the SMU output voltage |  |  | Voltage | 0.0 V |
|  | Enable the SMU |  |  |  |  |
|  | Measure the low state leakage current of the pin and save to database. |  |  |  |  |
|  | Set the SMU output voltage |  |  | Voltage | vdd3v3 supply domain voltage, Table 1 |
|  | Measure the high state leakage current of the pin and save to database. |  |  |  |  |
|  | Disable the SMU |  |  |  |  |
|  | Disconnect the SMU from the pin |  |  |  |  |
|  | Restore FPGA connections to DUT. |  |  |  |  |
|  | If applicable, cleanup the pin configuration |  |  |  | Pin cleanup table |
|  | Repeat the test for each pin and group the results per pin type |  |  |  | Pin groups table |
| Notes |  |  |  |  |  |
| This test will check the leakage current of all high impedance inputs and outputs which can be put into tri-state. Starting from all pins, the following pins are excluded: - master_mosi_cam, master_spi_clk_cam, master_csb_cam, trigger_cam, reset_cam, osc, slave_miso: Digital outputs without tri-state. - ldo_dig_out, ldo_out_analog, vref_cam, ldo_cam_out, ldo_vco_out, ldo_out_pll, ldo_out_bias: LDO outputs - trxp, trxn: High speed CXP interface. To reduce parasitic load on this interface, these pins were omitted. They are tested implicitly during tests which make use of this CXP interface. - xtalp, xtaln: These are connected directly to the crystal, and are omitted for this test to reduce the parasitic load on these pins. They are tested implicitly during the test T0013: Oscillator. - vdd2v5_otp, vdd3v3_dig, vdd3v3_ana, vdd3v3_cam, vdd3v3_pll: Power supplies. - vss, vss_dig, vss_ana: Ground connections. Input configuration table If the pin is not mentioned, the pin doesn't require additional configuration to use it as in input. Pin; Register parameter; Value; Note. gpio[10:0]; IOLineModeAll_Value; 0; Write 0 at bit position of the current GPIO channel.. Pin cleanup table If the pin is not mentioned the pin doesn’t require additional cleanup Pin; Register parameter; Value; Note. All pins; Pins are configured as input which is safe, no cleanup is required.. Pin groups table To simplify reporting, pins which share the same IO cell are grouped into a single measurement ID. To optimize the test time, pins which belong to the same group can share initialization to speed up the measurements. For example: - Park state needs only be run once. - The SMU needs to be configured only once - If a group of pins share the same register in the input configuration table, the register can be configured for all pins in the group in a single transaction. The same applies for the pin cleanup table. Group; Pins; Note. video; video_data[11:0] frame_valid_cam line_valid_cam data_valid_cam pixel_clk_cam; Video pins have by default a pull down enabled. gpio_pull_none; gpio[10:0]; GPIO IO cells which have by default no pull resistor enabled. gpio_pull_up; slave_csb; GPIO IO cells which have by default a pull up resistor enabled. gpio_pull_down; master_miso_cam slave_spi_clk slave_mosi; GPIO IO cells which have by default a pull down resistor enabled. ref_clk_3v3; ref_clk; Reference clock input running in 3.3V mode.. ref_clk_2v5; ref_clk; Reference clock input running in 2.5V mode.. ref_clk_1v8; ref_clk; Reference clock input running in 1.8V mode.. ref_clk_1v2; ref_clk; Reference clock input running in 1.2V mode.. |
| Test parameters and acceptance criteria |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |
| M0006 | T0003_M0006_i_ih_video | uA |  |  |  |
| M0007 | T0003_M0007_i_il_video | uA |  |  |  |
| M0008 | T0003_M0008_i_ih_gpio_pull_none | uA |  |  |  |
| M0009 | T0003_M0009_i_ih_gpio_pull_none | uA |  |  |  |
| M0010 | T0003_M0010_i_il_gpio_pull_up | uA |  |  |  |
| M0011 | T0003_M0011_i_ih_gpio_pull_up | uA |  |  |  |
| M0012 | T0003_M0012_i_ih_gpio_pull_down | uA |  |  |  |
| M0013 | T0003_M0013_i_il_gpio_pull_down | uA |  |  |  |
| M0014 | T0003_M0014_i_ih_ref_clk_3v3 | mA |  |  |  |
| M0015 | T0003_M0015_i_ih_ref_clk_3v3 | mA |  |  |  |
| M0165 | T0003_M0165_i_il_ref_clk_2v5 | mA |  |  |  |
| M0166 | T0003_M0166_i_ih_ref_clk_2v5 | mA |  |  |  |
| M0167 | T0003_M0167_i_ih_ref_clk_1v8 | mA |  |  |  |
| M0168 | T0003_M0168_i_il_ref_clk_1v8 | mA |  |  |  |
| M0169 | T0003_M0169_i_ih_ref_clk_1v2 | mA |  |  |  |
| M0170 | T0003_M0170_i_il_ref_clk_1v2 | mA |  |  |  |

## T0004: VIL VIH Test

Table 21: T0004: VIL VIH Test Overview

| Test Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| This test checks the input hysteresis of all digital input pins. During this test, a high to low and low to high voltage sweep are applied to each digital input pin. At each sweep point the digital state is observed. The combination of the input voltage, sweep direction and observed state can be used to extract the hysteresis parameters of each digital input. |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |
| Default configuration (functional) |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |
| For this test an SMU is required: - Voltage range: 0 .. 3.3V - Accuracy: 10 mV - Step size: 10 mV |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |
| No deviations from chapter 2 applies to this test. |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |
| This test will be executed in an automated way, in a temperature controlled environment. |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |
| Step | Action |  |  | Register parameter | Value |
|  | Start from park state |  |  |  |  |
|  | Configure the SMU as a voltage source |  |  | Current Limit | 1 mA |
|  | If applicable, configure the pin as input. |  |  | Input configuration table |  |
|  | If applicable, configure the observation method for this pin. |  |  | Observation configuration table |  |
|  | Connect the SMU to the pin |  |  |  |  |
|  | Connect the scope to the pin |  |  |  |  |
|  | Sweep the SMU output voltage from low to high, and observe the pin's state at each sweep point |  |  | Voltage | 0.0V..vdd3v3 supply domain voltage, Table 1 |
|  |  |  |  | Step Size | 10 mV |
|  | Observe the pin state at each sweep point from step 7 |  |  |  | Observation table |
|  | Save the voltage sweep and observed state to the database. |  |  | Observation table | pin state |
|  | Sweep the SMU output voltage from high to low, and observe the pin's state at each sweep point |  |  | Voltage | vdd3v3 supply domain voltage, Table 1..0 |
|  |  |  |  | Step Size | 10 mV |
|  | Observe the pin state at each sweep point from step 7 |  |  |  | Observation table |
|  | Disconnect SMU from DUT |  |  |  |  |
|  | Disconnect scope from DUT |  |  |  |  |
|  | If applicable, cleanup the observation configuration for the pin. |  |  |  | Observation cleanup table |
| Notes |  |  |  |  |  |
| This test tests all digital inputs except: - video_data[11:6] : State needs to be observed through functional test - master_miso_cam : State needs to be observed through functional test - slave_spi_clk : State needs to be observed through functional test - slave_csb : State needs to be observed through functional test - slave_mosi : State needs to be observed through functional test slave_miso: State needs tob observed through functional test Input configuration table If the pin is not mentioned, the pin doesn't require additional configuration to use it as in input. Pin; Register parameter; Value; Note. gpio[10:0]; IOLineModeAll_Value; 0; Write 0 at bit position of the current GPIO channel.. Observation configuration table If the pin is not mentioned, the pin doesn't require additional configuration to be able to read it's digital state. Pin; Action; Register Parameter; Value. video_data[5:0]; Forward video bus to test bus on GPIOs; Analog_test_mux_vect_sel(2); 2. Enable test bus output on GPIO pins; Analog_test_mux_sel(1); 1. Configure all GPIOs as output; IOLineModeAll_Value; 0x7FF. Connect clock to pixel_clk_cam input; Frequency; 1 MHz. Enable all video_data bits; PixelSize_Value; 12. Select external video input; TestPattern_Value; 0. pixel_clk_cam; Forward video bus to test bus on GPIOs; Analog_test_mux_vect_sel(2); 2. Enable test bus output on GPIO pins; Analog_test_mux_sel(1); 1. Configure all GPIOs as output; IOLineModeAll_Value; 0x7FF. frame_valid_cam; Forward video bus to test bus on GPIOs; Analog_test_mux_vect_sel(2); 2. line_valid_cam; Enable test bus output on GPIO pins; Analog_test_mux_sel(1); 1. data_valid_cam; Configure all GPIOs as output; IOLineModeAll_Value; 0x7FF. Connect clock to pixel_clk_cam input; Frequency; 1 MHz. Enable all video_data bits; PixelSize_Value; 12. Select external video input; TestPattern_Value; 0. Enable continues acquisition mode; A6000_AcquisitionMode; 2. Start acquisition; A6000_AcquisitionStart; 1. ref_clk; Disable crystal oscillator; Analog_en_xo; 0. Enable external reference clock input; Analog_en_ref_buf; 1. Configure clk_ref_tst in test mux; IOLineModeAll_Value; 0b1100. Analog_tstmux_d1sel; 5. Analog_spare2_register; 0. Analog_en_tref_out; 1. Observation table Pin; Register Parameter; Note. gpio[10:0]; IOLineStatusAll_Value; Read the bit at the bit position of the current GPIO channel.. video_data[5:0]; Output pins gpio[10:5]; Read with FPGA.. pixel_clk_cam; Output pin gpio[0]; Read with FPGA.. frame_valid_cam; Output pin gpio[4]; Read with FPGA.. line_valid_cam; Output pin gpio[3]; Read with FPGA.. data_valid_cam; Output pin gpio[2]; Read with FPGA.. resetn; Analog_constant_test; Read through spi slave interface.. 0xAA: resetn = 1. 0x00: resetn = 0. ref_clk; Output pin gpio[2]; Read with FPGA, inverted input state.. Observation cleanup table Pin; Action; Register Parameter; Value. video_data[5:0]; Disable test bus output on GPIO pins; Analog_test_mux_sel; 0. Configure all GPIOs as input; IOLineModeAll_Value; 0. Disconnect clock from pixel_clk_cam input. pixel_clk_cam; Disable test bus output on GPIO pins; Analog_test_mux_sel; 0. Configure all GPIOs as input; IOLineModeAll_Value; 0. frame_valid_cam; Disable test bus output on GPIO pins; Analog_test_mux_sel; 0. line_valid_cam; Configure all GPIOs as input; IOLineModeAll_Value; 0. data_valid_cam; Disconnect clock from pixel_clk_cam input. ref_clk; Enable crystal oscillator; Analog_en_xo; 0. Disable external reference clock input; Analog_en_ref_buf; 1. Configure all GPIOs as input; IOLineModeAll_Value; 0. Disconnect clk_ref_tst in test mux; Analog_tstmux_d1sel; 0. Analog_spare2_register; 0. Analog_en_tref_out; 0. Pin groups table To simplify reporting, pins which share the same IO cell are grouped into a single measurement ID. To optimize the test time, pins which belong to the same group can share initialization to speed up the measurements. For example: - Park state needs only be run once. - The SMU needs to be configured only once - If a group of pins share the same register in the input configuration table, the register can be configured for all pins in the group in a single transaction. The same applies for the pin cleanup table. Group; Pins; Note. resetn; resetn. video; video_data[5:0] frame_valid_cam line_valid_cam data_valid_cam pixel_clk_cam. gpio; gpio[10:0]. ref_clk_3v3; ref_clk; Reference clock input running in 3.3V mode.. ref_clk_2v5; ref_clk; Reference clock input running in 2.5V mode.. ref_clk_1v8; ref_clk; Reference clock input running in 1.8V mode.. ref_clk_1v2; ref_clk; Reference clock input running in 1.2V mode.. |
| Test parameters and acceptance criteria |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |
| M0022 | T0004_M0022_v_il_resetn | V |  |  |  |
| M0023 | T0004_M0023_v_ih_resetn | V |  |  |  |
| M0024 | T0004_M0024_v_hyst_resetn | V |  |  |  |
| M0017 | T0004_gpio_M0017_v_ih_gpio | V |  |  |  |
| M0016 | T0004_gpio_M0016_v_il_gpio | V |  |  |  |
| M0018 | T0004_gpio_M0018_v_hyst_gpio | V |  |  |  |
| M0020 | T0004_video_M0020_v_ih_video | V |  |  |  |
| M0019 | T0004_video_M0019_v_il_video | V |  |  |  |
| M0021 | T0004_video_M0021_v_hyst_video | V |  |  |  |
| M0026 | T0004_M0026_v_il_ref_clk_3v3 | V |  |  |  |
| M0027 | T0004_M0027_v_ih_ref_clk_3v3 | V |  |  |  |
| M0028 | T0004_M0028_v_hyst_ref_clk_3v3 | V |  |  |  |
| M0171 | T0004_M0171_v_il_ref_clk_2v5 | V |  |  |  |
| M0172 | T0004_M0172_v_ih_ref_clk_2v5 | V |  |  |  |
| M0173 | T0004_M0173_v_hyst_ref_clk_2v5 | V |  |  |  |
| M0174 | T0004_M0174_v_il_ref_clk_1v8 | V |  |  |  |
| M0175 | T0004_M0175_v_ih_ref_clk_1v8 | V |  |  |  |
| M0176 | T0004_M0176_v_hyst_ref_clk_1v8 | V |  |  |  |
| M0177 | T0004_M0177_v_il_ref_clk_1v2 | V |  |  |  |
| M0178 | T0004_M0178_v_ih_ref_clk_1v2 | V |  |  |  |

## T0005: VOH VOL Test

Table 22: T0005: VOH VOL Test Overview

| Test Description |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| This test measures the low and high voltage of digital outputs. During this test each digital output will be put in both the low and high state. In each state, the output voltage will be measured under a load current of 1 mA. |  |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |  |
| Default configuration (functional) |  |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |  |
| For this test an SMU is required: - Current source/sink: 1mA - Voltage measurement accuracy: < 10mV, - Voltage measurement range: 3.6 V |  |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |  |
| No deviations from chapter 2 applies to this test. |  |  |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |  |  |
| This test will be executed in an automated way and in a temperature controlled environment |  |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |  |
| Step | Action |  |  | Register parameter |  | Value |  |
|  | Start from park state |  |  |  |  |  |  |
|  | Start of pin loop |  |  |  |  |  |  |
|  | Configure the SMU which belongs to the pin as a current source. |  |  | Mode |  | Current |  |
|  |  |  |  | Voltage limit |  | 3.63V |  |
|  | Connect SMU to the pin |  |  |  |  |  |  |
|  | If applicable configure the pin as output. |  |  | Output configuration table |  |  |  |
|  | Set the pin state to a logic low |  |  | Output state table |  |  |  |
|  | Configure SMU to source current into the pin |  |  | Current |  | 1 mA |  |
|  | Enable the SMU |  |  |  |  |  |  |
|  | Measure the pin's output voltage |  |  | Voltage |  | V_OL |  |
|  | Disable the SMU |  |  |  |  |  |  |
|  | Set the pin state to a logic high |  |  | Output state control table |  |  |  |
|  | Configure the SMU to sink current from the pin |  |  | Current |  | -1 mA |  |
|  | Enable the SMU |  |  |  |  |  |  |
|  | Measure the pin's output voltage |  |  | Voltage |  | V_OH |  |
|  | Disable the SMU |  |  |  |  |  |  |
|  | Disconnect the SMU from the pin |  |  |  |  |  |  |
|  | If applicable, cleanup the pin configuration |  |  | Output configuration cleanup table |  |  |  |
|  | End of pin loop, go to start of pin loop if more pins remain |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| This test tests all digital outputs except: - master_spi_clk_cam: Static state can only be low. - master_csb_cam: Static state can only be high. - slave_miso: State needs to be set through a functional test. Output configuration table Pin; Action; Register parameter; Value. gpio[10:0]; Configure GPIO pin as output; IOLineModeAll_Value; Write a 1 to the bit which matches the GPIO channel.. trigger_cam; Configure trigger source to software control; TriggerSource_Value; 64. osc; Disconnect frame grabber from DUT. Disable crystal oscillator; Analog_en_xo; 0. Enable external reference clock input; Analog_en_ref_buf; 1. Enable the osc output; Analog_en_sys_out; 1. Connect FPGA to ref_clk input. Output state control table Pin; Action; Register parameter; Value. gpio[10:0]; Write the output state to the register; IOUserOutputValueAll_Value; Write the state to the bit which matches the GPIO channel.. reset_cam; Write the output state to the register; Analog_rst_cam_do; Inverted output state. trigger_cam; Actual state of trigger cam can't be read. Therefore it's required to keep track of the state manually. After reset the state of the trigger pin is 0.. Retrieve the current state of the pin.; Variable in test; After reset the pin state is logic low.. Set the state of the trigger pin; SoftwareTrigger_Value; Write 1 in case required state doesn't match current state. To toggle state.. Save the current state for future state changes; Variable in test. osc; Control osc using ref_clk input through FPGA; Inverted output state. master_mosi_cam; Connect master_miso_cam to the FPGA. Configure the desired state for master_miso_cam on master_mosi_cam using the FPGA.. Trigger any SPI transaction on the camera SPI master interface over the CXP interface.; Any register above 0x1000_0000 (E.g. ReverseXReg). Disconnect master_miso_cam from the FPGA. Output configuration cleanup table Pin; Action; Register parameter; Value. osc; Disconnect FPGA from ref_clk_input. Disable the osc output; Analog_en_sys_out; 0. Disable external reference clock input; Analog_en_ref_buf; 0. Enable crystal oscillator; Analog_en_xo; 1. Connect frame grabber from DUT. Pin groups table To simplify reporting, pins which share the same IO cell are grouped into a single measurement ID. To optimize the test time, pins which belong to the same group can share initialization to speed up the measurements. For example: - Park state needs only be run once. - The SMU needs to be configured only once - If a group of pins share the same register in the input configuration table, the register can be configured for all pins in the group in a single transaction. The same applies for the pin cleanup table. Group; Pins; Note. gpio; gpio[10:0] reset_cam trigger_cam osc master-mosi_cam. |
| Test parameters and acceptance criteria |  |  |  |  |  |  |  |
| ID | Name | Unit | LTL |  | Typ |  | UTL |
| M0029 | T0005_M0029_v_ol_gpio | V |  |  |  |  |  |
| M0030 | T0005_M0030_v_oh_gpio | V |  |  |  |  |  |

## T0006: IOL IOH Test

Table 23: T0006: IOL IOH Test Overview

| Test Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| This test measures the low and high level output current of digital outputs. During this test each digital output will be put in both the low and high state. In each state, the output current will be measured. |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |
| Default configuration (functional) |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |
| For this test an SMU is required: - Voltage source range: 3.3V - Voltage source accuracy: 10 mV - Current measurement range: 50mA - Current measurement accuracy: 10 uA |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |
| No deviations from chapter 2 applies to this test. |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |
| This test will be executed in an automated way and in a temperature controlled environment |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |
| Step | Action |  | Register parameter |  | Value |
|  | Start from park state |  |  |  |  |
|  | Start of pin loop |  |  |  |  |
|  | Configure the SMU which belongs to the pin as a voltage source. |  | Mode |  | Voltage |
|  |  |  | Current limit |  | 100 mA |
|  | Connect SMU to the pin |  |  |  |  |
|  | If applicable configure the pin as output. |  | Output configuration table |  |  |
|  | Set the pin state to a logic low |  | Output state table |  |  |
|  | Configure SMU to apply voltage to pin |  | Voltage |  | vdd3v3 supply domain voltage, Table 1 |
|  | Enable the SMU |  |  |  |  |
|  | Measure the pin's output current |  | Current |  | I_OL |
|  | Disable the SMU |  |  |  |  |
|  | Set the pin state to a logic high |  | Output state control table |  |  |
|  | Configure the SMU to sink current from the pin |  | Voltage |  | 0 V |
|  | Enable the SMU |  |  |  |  |
|  | Measure the pin's output current |  | Current |  | I_OH |
|  | Disable the SMU |  |  |  |  |
|  | Disconnect the SMU from the pin |  |  |  |  |
|  | If applicable, cleanup the pin configuration |  | Output configuration cleanup table |  |  |
|  | End of pin loop, go to start of pin loop if more pins remain |  |  |  |  |
| This test tests all digital outputs except: - master_spi_clk_cam: Static state can only be low. - master_csb_cam: Static state can only be high. - slave_miso: State needs to be set through a functional test. Output configuration table Refer to the output configuration table of T0005. Output state control table Refer to the output state control table of T0005. Output configuration cleanup table Refer to the output configuration cleanup table of T0005. |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |
| M0031 | T0006_M0031_i_ol_gpio | mA |  |  |  |
| M0032 | T0006_M0032_i_oh_gpio | mA |  |  |  |

## T0007: IDDD Test

Table 24: T0007: IDDD Test Overview

| Test Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| This test will measure the current consumption of the DUT in multiple states, video modes, frame rates and test patterns. |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |
| A power supply with: - 4 Channels - Output voltage: 3.3 V - Output voltage accuracy: 10 mV - Current limit: 100 mA - Current accuracy: 100 uA - Current resolution: 10 uA |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |
| No deviations from chapter 2 applies to this test. |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |
| This test will be executed in an automated way, at room temperature and in a temperature controlled environment |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |
| Step | Action |  |  | Register parameter | Value |
|  | Start from park state |  |  |  |  |
|  | Assert resetn |  |  | resetn | 0 |
|  | Measure reset current levels for the 3 supplies |  |  |  |  |
|  | Re-enter park state |  |  |  |  |
|  | Measure reset current levels for the 3 supplies |  |  |  |  |
|  | Start of video mode loop |  |  |  |  |
| 6.1. | Configure the first (next) video mode |  |  | ImageSize_Height | Video mode table |
|  |  |  |  | ImageSize_Width | Video mode table |
| 6.2 | Start of frame rate loop |  |  |  |  |
| 6.2.1 | Configure first (next) frame rate |  |  | AcquisitionFrameRate_Value | Frame rate table |
| 6.2.2 | Start of test pattern loop |  |  |  |  |
| 6.2.2.1 | Configure first (next) test pattern |  |  | TestPattern_Value | Test pattern table |
| 6.2.2.2 | Start data acquisition |  |  | AcquisitionStart | True |
| 6.2.2.3 | Measure currents for the 3 supplies |  |  |  |  |
| 6.2.2.4 | Stop data acquisition |  |  | AcquisitionStop | True |
| 6.2.2.5 | End of test pattern loop, go to step 6.2.2 if more test patterns remain. |  |  |  |  |
| 6.2.3 | End of video mode loop, go to step 6.2 if more video modes remain. |  |  |  |  |
| 6.3 | End of frame rate loop, go to step 6 if more frame rates remain. |  |  |  |  |
| Notes |  |  |  |  |  |
| The test will loop over the following combinations: Parameter; Values. video format; VGA,HD,FHD. frame rate; 10,25. test pattern; GreyHorizontalRamp. state; reset, park, active. Test Pattern table Pattern; Register parameter; value. GreyHorizontalRamp; TestPattern_Value; 16. Frame rate table Frame rate; Register parameter; Value. 10; AcquisitionFrameRate_Value; 10. 25; AcquisitionFrameRate_Value; 25. Video mode table Video mode; Register parameter; Value. VGA; ImageSize_Width; 640. ImageSize_Height; 480. HD; ImageSize_Width; 1280. ImageSize_Height; 720. VGA; ImageSize_Width; 1920. ImageSize_Height; 1080. DUT state table State; register parameter; Value. Reset; Park state. resetn pin; 0. Park; Park state. Active; Park state. TestPattern_Value; Test pattern table. A6000_AcquisitionMode; 2. A6000_AcquisitionStart; 1. |
| Test parameters and acceptance criteria |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |
| M0033 | T0007_M0033_idd_ana_reset | mA |  |  |  |
| M0034 | T0007_M0033_idd_ana_park | mA |  |  |  |
| M0035 | T0007_M0033_idd_ana_active | mA |  |  |  |
| M0039 | T0007_M0033_idd_dig_reset | mA |  |  |  |
| M0040 | T0007_M0033_idd_dig_park | mA |  |  |  |
| M0041 | T0007_M0033_idd_dig_active | mA |  |  |  |
| M0042 | T0007_M0033_idd_cam_reset | mA |  |  |  |
| M0043 | T0007_M0033_idd_cam_park | mA |  |  |  |
| M0044 | T0007_M0033_idd_cam_active | mA |  |  |  |

## T0008: Test Buffer Offset Test

Table 25: T0008: Test Buffer Offset Test Overview

| Test Description |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| This test will measure the offset of both test buffers on the test mux. During this test a loop is created inside the test mux from GPIO 1 to GPIO 0 for the test buffer on, and the other way around for the test buffer on GPIO 1. A sweep is applied on GPIO 1 and the test buffer's output voltage is measured on GPIO 0. This result can be used in other measurements which measure buffered voltages through the test mux. |  |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |  |
| An SMU with: - Source voltage range: 1.2 V - Source voltage accuracy: 10 mV - Source voltage sweep step size: 10mV A DMM with: - Measurement voltage range: 1.2 V - Measurement accuracy: 10 mV |  |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |  |
| No deviations from chapter 2 applies to this test. |  |  |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |  |  |
| This test will be executed in an automated way and in a temperature controlled environment |  |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |  |
| Step | Action |  |  | Register parameter |  | Value |  |
|  | Start from park state |  |  |  |  |  |  |
|  | Create a loop between GPIO0 and GPIO1 through the test mux |  |  | IOLineModeAll_Value |  | 0 |  |
|  |  |  |  | Analog_spare2_register[4] |  | 1 |  |
|  |  |  |  | Analog_spare2_register[5] |  | 1 |  |
|  |  |  |  | Analog_en_tmux_switch_1 |  | 1 |  |
|  |  |  |  | Analog_en_tmux_switch_2 |  | 1 |  |
|  |  |  |  | Analog_tstmux_a1sel |  | 15 |  |
|  |  |  |  | Analog_tstmux_a2sel |  | 15 |  |
|  | Measure test buffer on GPIO0 |  |  |  |  |  |  |
|  | Initialize the SMU as a voltage source |  |  | Mode |  | Voltage |  |
|  |  |  |  | Output voltage |  | 0.0 V |  |
|  |  |  |  | Current protection |  | 1 mA |  |
|  | Connect SMU to GPIO1 |  |  |  |  |  |  |
|  | Connect DMM to GPIO0 |  |  |  |  |  |  |
|  | Enable test buffer on GPIO0 |  |  | Analog_tstmux_a1bufen |  | 1 |  |
|  |  |  |  | Analog_tstmux_a2bufen |  | 0 |  |
|  | Configure the SMU |  |  | Voltage sweep range |  | 0 .. 1.2 V |  |
|  |  |  |  | Voltage step size |  | 10 mV |  |
|  | Enable the SMU |  |  |  |  |  |  |
|  | Perform the voltage sweep on the SMU |  |  |  |  |  |  |
|  | Measure buffer output voltage for each sweep point on the DMM |  |  |  |  |  |  |
|  | Disable the SMU |  |  |  |  |  |  |
|  | Disconnect DMM from GPIO0 |  |  |  |  |  |  |
|  | Disconnect SMU from GPIO1 |  |  |  |  |  |  |
|  | Measure test buffer on GPIO1 |  |  |  |  |  |  |
|  | Initialize the SMU as a voltage source |  |  | Mode |  | Voltage |  |
|  |  |  |  | Output voltage |  | 0.0 V |  |
|  |  |  |  | Current protection |  | 1 mA |  |
|  | Connect SMU to GPIO0 |  |  |  |  |  |  |
|  | Connect DMM to GPIO1 |  |  |  |  |  |  |
|  | Enable test buffer on GPIO1 |  |  | Analog_tstmux_a1bufen |  | 0 |  |
|  |  |  |  | Analog_tstmux_a2bufen |  | 1 |  |
|  | Configure the SMU |  |  | Voltage sweep range |  | 0 .. 1.2 V |  |
|  |  |  |  | Voltage step size |  | 10 mV |  |
|  | Enable the SMU |  |  |  |  |  |  |
|  | Perform the voltage sweep on the SMU |  |  |  |  |  |  |
|  | Measure buffer output voltage for each sweep point on the DMM |  |  |  |  |  |  |
|  | Disable the SMU |  |  |  |  |  |  |
|  | Disconnect SMU from GPIO0 |  |  |  |  |  |  |
|  | Disconnect DMM from GPIO1 |  |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |  |  |
| ID | Name | Unit | LTL |  | Typ |  | UTL |
| M0045 | T0008_M0045_a1_offset | mV |  |  |  |  |  |
| M0046 | T0008_M0046_a2_offset | mV |  |  |  |  |  |

## T0009: Block Trimming Test

Table 26: T0009: Block Trimming Test Overview

| Test Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| This test will trim each block (LDO, voltage reference, current reference) inside the DUT. During this test the sweep from T0010 is performed first on the block in the DUT. From this sweep result the trim setting that results in the closest target value is selected. |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |
| An SMU with: - Measure voltage range: 1.2 V - Measure voltage accuracy: 10 mV |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |
| No deviations from chapter 2 applies to this test. This test is only used in the TRIM test stage, here the Vnom and RT conditions are used. |  |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |  |
| This test will be executed in an automated way in a temperature controlled environment |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |
| Step | Action |  | Register parameter |  | Value |  |
|  | Start of block loop. |  |  |  |  |  |
| 1.1 | Select the first (next) block that needs to be trimmed. |  | Block specification table |  |  |  |
| 1.2 | Perform the sweep from T0010 on the block. |  |  |  |  |  |
| 1.3 | Find the trim setting(s) which result in an output voltage closest to the target value. |  | Block specification table |  |  |  |
| 1.4 | Apply the trim value to the block. |  | Block specification table |  |  |  |
| 1.5 | Save the trim value for later use in the park state. |  |  |  |  |  |
| 1.6 | End of block loop, go to 1. if more blocks remain. |  |  |  |  |  |
| Notes |  |  |  |  |  |  |
| Block specification table See T0010 |  |  |  |  |  |  |
| Measurements – Acceptance criteria |  |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ |  | UTL |
| M0047 | T0009_ldo_bias_M0047_ldo_bias_voltage_sweep | V |  |  |  |  |
| M0048 | T0009_ldo_bias_M0048_trim_ldo_bias_trimmed | # |  |  |  |  |
| M0049 | T0009_ldo_bias_M0049_ldo_bias_voltage | V |  |  |  |  |
| M0181 | T0009_ldo_bias_M0181_trim_ldo_bias_default | # |  |  |  |  |
| M0182 | T0009_ldo_bias_M0182_ldo_bias_default_voltage | V |  |  |  |  |
| M0050 | T0009_bandgap_vref_M0050_bandgap_vref_voltage_sweep | V |  |  |  |  |
| M0051 | T0009_bandgap_vref_M0051_trim_vireftempco_trimmed | # |  |  |  |  |
|  | T0009_bandgap_vref_M0051_trim_vref_trimmed | # |  |  |  |  |
| M0052 | T0009_bandgap_vref_M0052_bandgap_vref_voltage | V |  |  |  |  |
| M0183 | T0009_bandgap_vref_M0183_trim_vireftempco_default | # |  |  |  |  |
|  | T0009_bandgap_vref_M0183_trim_vref_default | # |  |  |  |  |
| M0184 | T0009_bandgap_vref_M0184_bandgap_vref_default_voltage | V |  |  |  |  |
| M0053 | T0009_bandgap_iref_M0053_bandgap_iref_current_sweep | uA |  |  |  |  |
| M0054 | T0009_bandgap_iref_M0054_trim_iref_trimmed | # |  |  |  |  |
| M0055 | T0009_bandgap_iref_M0055_bandgap_iref_current | uA |  |  |  |  |
| M0185 | T0009_bandgap_iref_M0185_trim_iref_trimmed | # |  |  |  |  |
| M0186 | T0009_bandgap_iref_M0186_bandgap_iref_default_current | uA |  |  |  |  |
| M0056 | T0009_ldo_analog_M0056_ldo_analog_voltage_sweep | V |  |  |  |  |
| M0057 | T0009_ldo_analog_M0057_trim_ldo_ana_trimmed | # |  |  |  |  |
| M0058 | T0009_ldo_analog_M0058_ldo_analog_voltage | V |  |  |  |  |
| M0187 | T0009_ldo_analog_M0187_trim_ldo_ana_default | # |  |  |  |  |
| M0188 | T0009_ldo_analog_M0188_ldo_analog_default_voltage | V |  |  |  |  |
| M0059 | T0009_ldo_digital_M0059_ldo_digital_voltage_sweep | V |  |  |  |  |
| M0060 | T0009_ldo_digital_M0060_trim_ldo_dig_trimmed | # |  |  |  |  |
| M0061 | T0009_ldo_digital_M0061_ldo_digital_voltage | V |  |  |  |  |
| M0189 | T0009_ldo_digital_M0189_trim_ldo_dig_default | # |  |  |  |  |
| M0190 | T0009_ldo_digital_M0190_ldo_digital_default_voltage | V |  |  |  |  |
| M0062 | T0009_ldo_xo_M0062_ldo_xo_voltage_sweep | V |  |  |  |  |
| M0063 | T0009_ldo_xo_M0063_trim_ldo_xo_trimmed | # |  |  |  |  |
| M0064 | T0009_ldo_xo_M0064_ldo_xo_voltage | V |  |  |  |  |
| M0191 | T0009_ldo_xo_M0191_trim_ldo_xo_default | # |  |  |  |  |
| M0192 | T0009_ldo_xo_M0192_ldo_xo_default_voltage | V |  |  |  |  |
| M0065 | T0009_ldo_pll_M0065_ldo_pll_voltage_sweep | V |  |  |  |  |
| M0066 | T0009_ldo_pll_M0066_trim_ldo_pll_trimmed | # |  |  |  |  |
| M0067 | T0009_ldo_pll_M0067_ldo_pll_voltage | V |  |  |  |  |
| M0193 | T0009_ldo_pll_M0193_trim_ldo_pll_default | # |  |  |  |  |
| M0194 | T0009_ldo_pll_M0194_ldo_pll_default_voltage | V |  |  |  |  |
| M0068 | T0009_ldo_cp_M0068_ldo_cp_voltage_sweep | V |  |  |  |  |
| M0069 | T0009_ldo_cp_M0069_trim_ldo_cp_trimmed | # |  |  |  |  |
| M0070 | T0009_ldo_cp_M0070_ldo_cp_voltage | V |  |  |  |  |
| M0195 | T0009_ldo_cp_M0195_trim_ldo_cp_default | # |  |  |  |  |
| M0196 | T0009_ldo_cp_M0196_ldo_cp_default_voltage | V |  |  |  |  |
| M0071 | T0009_ldo_ldiv_M0071_ldo_ldiv_voltage_sweep | V |  |  |  |  |
| M0072 | T0009_ldo_ldiv_M0072_trim_ldo_ldiv_trimmed | # |  |  |  |  |
| M0073 | T0009_ldo_ldiv_M0073_ldo_ldiv_voltage | V |  |  |  |  |
| M0197 | T0009_ldo_ldiv_M0197_trim_ldo_ldiv_default | # |  |  |  |  |
| M0198 | T0009_ldo_ldiv_M0198_ldo_ldiv_default_voltage | V |  |  |  |  |
| M0074 | T0009_ldo_vco_M0074_ldo_vco_voltage_sweep | V |  |  |  |  |
| M0075 | T0009_ldo_vco_M0075_trim_ldo_vco_trimmed | # |  |  |  |  |
| M0076 | T0009_ldo_vco_M0076_ldo_vco_voltage | V |  |  |  |  |
| M0199 | T0009_ldo_vco_M0199_trim_ldo_vco_default | # |  |  |  |  |
| M0200 | T0009_ldo_vco_M0200_ldo_vco_default_voltage | V |  |  |  |  |
| M0077 | T0009_ldo_odiv_M0077_ldo_odiv_voltage_sweep | V |  |  |  |  |
| M0078 | T0009_ldo_odiv_M0078_trim_ldo_odiv_trimmed | # |  |  |  |  |
| M0079 | T0009_ldo_odiv_M0079_ldo_odiv_voltage | V |  |  |  |  |
| M0201 | T0009_ldo_odiv_M0201_trim_ldo_odiv_default | # |  |  |  |  |
| M0202 | T0009_ldo_odiv_M0202_ldo_odiv_default_voltage | V |  |  |  |  |
| M0080 | T0009_ldo_pfd_M0080_ldo_pfd_voltage_sweep | V |  |  |  |  |
| M0081 | T0009_ldo_pfd_M0081_trim_ldo_pfd_trimmed | # |  |  |  |  |
| M0082 | T0009_ldo_pfd_M0082_ldo_pfd_voltage | V |  |  |  |  |
| M0203 | T0009_ldo_pfd_M0203_trim_ldo_pfd_default | # |  |  |  |  |
| M0204 | T0009_ldo_pfd_M0204_ldo_pfd_default_voltage | V |  |  |  |  |
| M0083 | T0009_ldo_camera_M0083_ldo_camera_voltage_sweep | V |  |  |  |  |
| M0084 | T0009_ldo_camera_M0084_trim_ldo_camera_trimmed | # |  |  |  |  |
| M0085 | T0009_ldo_camera_M0085_ldo_camera_voltage | V |  |  |  |  |
| M0205 | T0009_ldo_camera_M0205_trim_ldo_camera_default | # |  |  |  |  |
| M0206 | T0009_ldo_camera_M0206_ldo_camera_default_voltage | V |  |  |  |  |
| M0086 | T0009_vref_M0086_vref_voltage_sweep | V |  |  |  |  |
| M0087 | T0009_vref_M0087_trim_vref_camera_trimmed | # |  |  |  |  |
| M0088 | T0009_vref_M0088_vref_voltage | V |  |  |  |  |
| M0207 | T0009_vref_M0207_trim_vref_camera_default | # |  |  |  |  |
| M0208 | T0009_vref_M0208_vref_default_voltage | V |  |  |  |  |

## T0010: Block Sweep Test

Table 27: T0010: Block Sweep Test Overview

| Test Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| This test will sweep over the trim setting of each block in the DUT. During this test a sweep is performed over all trim settings of each block. For each trim setting the resulting output value (voltage, current) is recorded. |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |
| An SMU with: - Measure voltage range: 1.2 V - Measure voltage accuracy: 10 mV - Measure current range: 10 uA - Measure current accuracy: 1 uA |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |
| No deviations from chapter 2 applies to this test. |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |
| This test will be executed in an automated way and in a temperature controlled environment |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |
| Step | Action |  | Register parameter | Value |  |
|  | Start from park state |  |  |  |  |
|  | Beginning of the block loop |  |  |  |  |
| 2.1 | Select the first (next) block |  | Block specification table |  |  |
| 2.2 | Connect the block’s output to an external pin. (Priority is given to direct accessible output pins over test mux pins) |  | Block specification table |  |  |
| 2.3 | Connect the SMU to the external pin |  | Block specification table |  |  |
| 2.4 | Configure the SMU in voltage or current mode. |  | SMU Mode | Block Specification table |  |
|  |  |  | Current Mode |  |  |
|  |  |  | Current | 1 pA |  |
|  |  |  | Voltage limit | 3.5 V |  |
|  |  |  | Voltage Mode |  |  |
|  |  |  | Voltage | 0 V |  |
|  |  |  | Current limit | 200 uA |  |
| 2.5 | Measure the block’s output for each trim setting |  | Block specification table |  |  |
| 2.6 | Disconnect the SMU from the external pin |  |  |  |  |
| 2.7 | End of the block loop, go to 2 if more blocks remain. |  |  |  |  |
| Notes |  |  |  |  |  |
| Block specification table Block Name; Test mux setting; Test Mux Pin; Output Pin; Trim registers; SMU Mode. bandgap voltage; vref_tst; gpio_0; /; trim_vireftempco trim_vref; Current. ptat; vref_ptat; gpio_1; /; trim_ptat_vref; Current. bandgap current; ptat_crt_10u; gpio_0; /; trim_iref10u; Voltage. ldo_cp; vtst_vddcp; gpio_1; /; trim_cpldo; Current. ldo_ldiv; vtst_vddldiv; gpio_1; /; trim_ldivldo; Current. ldo_vco; vtst_vddvco; gpio_1; ldo_vco_out; trim_vcoldo; Current. ldo_odiv; vtst_vddodiv; gpio_1; /; trim_odivldo; Current. ldo_pfd; vtst_ldo_pfd; gpio_1; /; trim_pfdldo; Current. ldo_bias; ldo_bias_tst; gpio_0; ldo_out_bias; trim_ldo_bias; Current. ldo_xo; vdd_xo; gpio_1; /; trim_ref_ldo; Current. ldo_digital; ldo_dig_tst; gpio_0; ldo_dig_out; trim_ldo_dig; Current. ldo_pll; ldo_pll_tst; gpio_0; ldo_out_pll; trim_ldo_pll; Current. ldo_analog; ldo_ana_tst; gpio_0; ldo_out_analog; trim_ldo_ana; Current. ldo_camera; /; /; ldo_cam_out; trim_ldo_cam; Current. vref; /; /; vref_cam; trim_cam_1v2_buf; Current. |
| Measurements – Acceptance criteria |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |
| M0089 | LDO bias sweep | V |  |  |  |
| M0090 | Bandgap voltage sweep | mV |  |  |  |
| M0091 | Bandgap current sweep | uA |  |  |  |
| M0092 | LDO analog sweep | V |  |  |  |
| M0093 | LDO digital sweep | V |  |  |  |
| M0094 | LDO XO sweep | V |  |  |  |
| M0095 | LDO PLL sweep | V |  |  |  |
| M0096 | LDO CP sweep | V |  |  |  |
| M0097 | LDO LDIV sweep | V |  |  |  |
| M0098 | LDO VCO sweep | V |  |  |  |
| M0099 | LDO ODIV sweep | V |  |  |  |
| M0100 | LDO PFD sweep | V |  |  |  |
| M0101 | LDO camera sweep | V |  |  |  |
| M0102 | VREF camera sweep | V |  |  |  |

## T0011: Static Signal Test

Table 28: T0011: Static Signal Test Overview

| Test Description |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| This test will measure static signals in the DUT, and store them in the database. During this test, all test signals available on the test mux related to internal static signals are measured and stored in the database. |  |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |  |
| A multimeter with: - Voltage measurement range: 2 V; 3.6V - Voltage measurement accuracy: 2 mV; 10mV - Current measurement range: 100 uA - Current measurement accuracy: 100 nA |  |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |  |
| No deviations from chapter 2 applies to this test. |  |  |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |  |  |
| This test will be executed in an automated way and in a temperature controlled environment |  |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |  |
| Step | Action |  |  | Register parameter |  | Value |  |
|  | Start from park state |  |  |  |  |  |  |
|  | Static signals accessible on gpio_0 |  |  | Table 16: test mux summary |  |  |  |
|  | Connect SMU to gpio_0 |  |  |  |  |  |  |
|  | Configure SMU as voltage meter |  |  |  |  |  |  |
|  | Start of gpio_0 voltage loop |  |  |  |  |  |  |
| 5.1 | Connect first (next) voltage signal that needs to be measured |  |  | Test signal table |  |  |  |
| 5.2 | Measure voltage with SMU |  |  |  |  |  |  |
| 5.3 | End of gpio_0 voltage loop, go to 5 if more signals remain. |  |  |  |  |  |  |
|  | Configure SMU as current meter |  |  |  |  |  |  |
| 7. | Start of gpio_0 current loop |  |  |  |  |  |  |
| 7.1 | Connect first (next) current signal that needs to be measured |  |  | Test signal table |  |  |  |
| 7.2 | Measure voltage with SMU |  |  |  |  |  |  |
| 7.3 | End of gpio_0 current loop, go to 7 if more signals remain. |  |  |  |  |  |  |
| 8. | Disconnect SMU from gpio_0 |  |  |  |  |  |  |
| 9. | Static signals accessible on gpio_1 |  |  | Table 16: test mux summary |  |  |  |
| 10. | Connect SMU to gpio_1 |  |  |  |  |  |  |
| 11. | Configure SMU as voltage meter |  |  |  |  |  |  |
| 12. | Start of gpio_1 voltage loop |  |  |  |  |  |  |
| 12.1 | Connect first (next) voltage signal that needs to be measured |  |  | Test signal table |  |  |  |
| 12.2 | Measure voltage with SMU |  |  |  |  |  |  |
| 12.3 | End of gpio_1 voltage loop, go to 12 if more signals remain. |  |  |  |  |  |  |
| 13. | Configure SMU as current meter |  |  |  |  |  |  |
| 14. | Start of gpio_1 current loop |  |  |  |  |  |  |
| 14.1 | Connect first (next) current signal that needs to be measured |  |  | Test signal table |  |  |  |
| 14.2 | Measure voltage with SMU |  |  |  |  |  |  |
| 14.3 | End of gpio_1 current loop, go to 14 if more signals remain. |  |  |  |  |  |  |
| 15 | Disconnect SMU from gpio_1 |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| #### Test signals table Test Signal Name; Type. gpio_0. hyb_out_tst; Voltage. ldo_dig_tst; Voltage. trx_n_tst; Voltage. ldo_pll_tst; Voltage. ldo_ana_tst; Voltage. vtoi_crt_10u; Voltage. ptat_crt_10u; Voltage. vco_crt_tst; Voltage. ldo_bias_tst; Voltage. ldo_cam_tst; Voltage. vref_tst; Voltage. lvds_cm; Voltage. vtoi_crt_10u; Current. ptat_crt_10u; Current. vco_crt_tst; Current. gpio_1. vdd_xo; Voltage. vtst_vddvco; Voltage. vtst_vddldiv; Voltage. vtst_vddcp; Voltage. vref_ptat; Voltage. vtst_vddpfd; Voltage. vtst_vddodiv; Voltage. vtst_vtune; Voltage. hyb_vcm; Voltage. vtst_v2i_ota_out; Current. |
| Test parameters and acceptance criteria |  |  |  |  |  |  |  |
| ID | Name | Unit | LTL |  | Typ |  | UTL |
| M0103 | T0011_M0103_hyb_out_tst | mV |  |  |  |  |  |
| M0104 | T0011_M0104_ldo_dig_tst | V |  |  |  |  |  |
| M0105 | T0011_M0105_trx_n_tst | mV |  |  |  |  |  |
| M0106 | T0011_M0106_ldo_pll_tst | V |  |  |  |  |  |
| M0107 | T0011_M0107_ldo_ana_tst | V |  |  |  |  |  |
| M0108 | T0011_M0108_ldo_bias_tst | V |  |  |  |  |  |
| M0109 | T0011_M0109_ldo_cam_tst | V |  |  |  |  |  |
| M0110 | T0011_M0110_vref_tst | mV |  |  |  |  |  |
| M0111 | T0011_M0111_lvds_cm | mV |  |  |  |  |  |
| M0112 | T0011_M0112_vtoi_crt_10u | uA |  |  |  |  |  |
| M0113 | T0011_M0113_ptat_crt_10u | uA |  |  |  |  |  |
| M0114 | T0011_M0114_vco_crt_tst | uA |  |  |  |  |  |
| M0115 | T0011_M0115_vdd_xo | V |  |  |  |  |  |
| M0116 | T0011_M0116_vtst_vddvco | V |  |  |  |  |  |
| M0117 | T0011_M0117_vtst_vddldiv | V |  |  |  |  |  |
| M0118 | T0011_M0118_vtst_vddcp | V |  |  |  |  |  |
| M0119 | T0011_M0119_vref_ptat | mV |  |  |  |  |  |
| M0120 | T0011_M0120_vtst_vddpfd | V |  |  |  |  |  |
| M0121 | T0011_M0121_vtst_vddodiv | V |  |  |  |  |  |
| M0122 | T0011_M0122_vtst_vtune | mV |  |  |  |  |  |
| M0123 | T0011_M0123_hyb_vcm | mV |  |  |  |  |  |
| M0124 | T0011_M0124_vtst_v2i_ota_out | uA |  |  |  |  |  |

## T0012: Regulators Test

Table 29: T0012 Regulators Test Overview

| Test Description |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| This test performs a load test on the LDO's which have external decoupling. During this test a sweep for different load currents is performed on the output of the LDO. For each load current the internal output voltage is measured through the test mux. |  |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |  |
| An SMU with: - Output current range: -20 mA - Output current step size: 1 mA - Output current accuracy: 50uA A DMM with: - Voltage measure range: 3.6 V - Voltage measure accuracy: 10 mV |  |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |  |
| During this test the LDOs will be always be tested at Vmin, Vnom and Vmax supply levels. The park state is altered during this test. The switch matrix is explicitly reset before the park state. The connection of DMM and SMU are also done before the park state. Because of this, the switch matrix will not be reset during the park state routine itself. This is required because the SMU can cause a glitch on the supply rail when connecting to the LDO output. Such glitch causes a chip reset, which also resets the test mux settings, which finally causes the tests to fail. |  |  |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |  |  |
| This test will be executed in an automated way and in a temperature controlled environment |  |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |  |
| Step | Action |  |  | Register parameter |  | Value |  |
| 1. | Beginning of LDO loop |  |  |  |  |  |  |
| 1.1 | Select the first (next) LDO |  |  |  |  |  |  |
| 1.2 | Reset switch matrix |  |  |  |  |  |  |
| 1.3 | Connect SMU to LDO pin on DUT |  |  | T0010: Block Sweep: Block specification |  |  |  |
| 1.4 | Configure SMU |  |  | Mode |  | Current |  |
|  |  |  |  | Current level |  | 0 mA |  |
|  |  |  |  | Current range |  | 20 mA |  |
|  |  |  |  | Voltage protection |  | 3.5 V |  |
| 1.5 | Enable the SMU |  |  |  |  |  |  |
| 1.6 | Connect the DMM to GPIO 0 or 1, depending on which pin the test mux can output the current LDO’s output. if the LDO is not accessible via the test mux, do not use the DMM and ignore this action. |  |  | T0010: Block Sweep: Block specification table |  |  |  |
| 1.7 | Park state, without reset of the switch matrix |  |  |  |  |  |  |
| 1.8 | Connect LDO’s output to GPIO 0 or 1 pin through test mux, if the LDO is accessible via the test mux. |  |  | T0010: Block Sweep: Block specification table |  |  |  |
| 1.9 | Sweep a load current with the SMU. Also sweep over default, min and max trim settings and 3 supply level corners. |  |  |  |  | Current table |  |
| 1.10 | Measure output voltage at each load current of the LDO with: - DMM through test bus, if the LDO is accessible via the test mux - SMU directly on the pin, if the LDO is not accessible via the test mux. |  |  |  |  |  |  |
| 1.11 | Load regulation parameter is calculated as the voltage difference between the condition at max current load vs no current load. |  |  |  |  |  |  |
| 1.12 | Turn off SMU. |  |  |  |  |  |  |
| 1.13 | Disconnect DMM from test mux, if applicable. |  |  |  |  |  |  |
| 1.14 | Disconnect SMU from LDO. |  |  |  |  |  |  |
| 1.15 | End of LDO loop, go 1 in case more LDOs remain. |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Current Table LDO; Max Current [mA]. LDO bias; 5. LDO analog; 20. LDO digital; 20. LDO PLL; 10. LDO VCO; 10. LDO camera; 20. Sweeping load currents is from high to low, this helps unloaded LDOs to settle to the correct output value. |
| Test parameters and acceptance criteria |  |  |  |  |  |  |  |
| ID | Name | Unit | LTL |  | Typ |  | UTL |
| M0209 | T0012_ldo_bias_M0209_output_voltage_ldo_bias | V |  |  |  |  |  |
| M0125 | T0012_ldo_bias_M0125_v_delta_ldo_bias | mV |  |  |  |  |  |
| M0210 | T0012_ldo_analog_M0210_output_voltage_ldo_analog | V |  |  |  |  |  |
| M0126 | T0012_ldo_analog_M0126_v_delta_ldo_analog | mV |  |  |  |  |  |
| M0211 | T0012_ldo_digital_M0211_output_voltage_ldo_digital | V |  |  |  |  |  |
| M0127 | T0012_ldo_digital_M0127_v_delta_ldo_digital | mV |  |  |  |  |  |
| M0212 | T0012_ldo_pll_M0212_output_voltage_ldo_pll | V |  |  |  |  |  |
| M0128 | T0012_ldo_pll_M0128_v_delta_ldo_pll | mV |  |  |  |  |  |
| M0213 | T0012_ldo_vco_M0213_output_voltage_ldo_vco | V |  |  |  |  |  |
| M0129 | T0012_ldo_vco_M0129_v_delta_ldo_vco | mV |  |  |  |  |  |
| M0214 | T0012_ldo_camera_M0214_output_voltage_ldo_camera | V |  |  |  |  |  |
| M0130 | T0012_ldo_camera_M0130_v_delta_ldo_camera | mV |  |  |  |  |  |

## T0013: Oscillator Test

Table 30:  T0013: Oscillator Test Overview

| Test Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| This test checks the reference clock circuits of the DUT. |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |
| An oscilloscope with: - Frequency measurement range: 20 MHz - Frequency measurement accuracy: 10 kHz |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |
| No deviations from chapter 2 applies to this test. |  |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |  |
| This test will be executed in an automated way and in a temperature controlled environment |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |
| Step | Action |  | Register parameter | Value |  |
|  | Start from park state |  |  |  |  |
|  | Enable reference clock ouput |  | Analog_en_sys_out | 1 |  |
|  | Connect scope to osc pin |  |  |  |  |
|  | Configure scope for frequency measurement |  | trigger.channel | 1 |  |
|  |  |  | channel.measure.clear | 1 |  |
|  |  |  | channel.offset | 0 |  |
|  |  |  | channel.probe.attenuation | 1 |  |
|  |  |  | channel.scale | 1 |  |
|  |  |  | timebase.scale | 50ns |  |
|  |  |  | trigger.level | 1.65 |  |
|  | Measurement crystal oscillator frequency on osc pin with scope |  | channel.measure.counter |  |  |
|  | Connect ref_clk input to FPGA |  |  |  |  |
|  | Switch to reference clock input |  | Analog_en_xo | 0 |  |
|  |  |  | Analog_en_ref_buf | 1 |  |
|  | Apply 20 MHz clock to ref_clk with FPGA |  |  |  |  |
|  | Measure ref_clk input frequency on osc pin with scope |  | channel.measure.counter |  |  |
|  | Disconnect ref_clk input from FPGA |  |  |  |  |
|  | Disconnect scope from osc pin |  |  |  |  |
| Notes |  |  |  |  |  |  |
| Crystal of 20MHz is used. |  |  |  |  |  |  |
| Measurements – Acceptance criteria |  |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |
| M0131 | T0013_M0131_xtal_frequency | MHz |  | 20 |  |
| M0132 | T0013_M0132_ref_clk_frequency | MHz |  | 20 |  |

## T0014: PLL Open Loop Test

Table 31: T0014: PLL Open Loop Test Overview

| Test Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| This test will check the PLL open loop behavior. In this test the PLL loop will be opened to check the range of the v_tune voltage. This will be done by changing the input divider to force the v_tune signal into low, mid and high state. |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |
| An oscilloscope with: - Frequency measurement range: 20 MHz - Frequency measurement accuracy: 10 kHz An SMU with: - Voltage measurement range: 2 V - Voltage measurement accuracy: 2 mV |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |
| No deviations from chapter 2 applies to this test. |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |
| This test will be executed in an automated way, at room temperature and in a temperature controlled environment |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |
| Step | Action | Register parameter |  |  | Value |
|  | Park state |  |  |  |  |
|  | Configure SMU as voltage meter |  |  |  |  |
|  | Configure scope for frequency measurement | timebase.scale |  |  | 50 ns |
|  |  | trigger.channel |  |  | 1 |
|  |  | trigger.level |  |  | 1.65 |
|  |  | channel.measure.clear |  |  | 1 |
|  |  | channel.offset |  |  | 0 |
|  |  | channel.probe.attenuation |  |  | 1 |
|  |  | channel.scale |  |  | 1 |
|  | Connect vtst_vtune on test mux on gpio_1 |  |  |  |  |
|  | Connect ldiv_out_tst on test mux on gpio_2 |  |  |  |  |
|  | Connect SMU to gpio_1 (vtune) |  |  |  |  |
|  | Connect scope to gpio_2 (ldiv) |  |  |  |  |
|  | Enable manual control of coarse bank control word. | Analog_ct_test_en |  |  | 1 |
|  | Start of PLL coarse cap bank control word sweep loop |  |  |  |  |
| 9.1 | Configure the first (next) PLL coarse cap bank control word | Analog_vco_cap_ctrl |  |  | 0, 31, 63 |
| 9.2 | Start of v_tune sweep loop |  |  |  |  |
| 9.2.1 | Configure the first (next) input divider word | Analog_idiv_div_wrd |  |  | 0, 1, 2 |
| 9.2.2 | Measure the v_tune voltage with the SMU |  |  |  |  |
| 9.2.3 | Measure the ldiv output frequency with the scope | channel.measure.counter |  |  |  |
| 9.2.4 | End of v_tune sweep loop, go to start of loop if more settings remain. |  |  |  |  |
| 9.3 | End of PLL coarse cap bank control word, go 8 if more settings remain. |  |  |  |  |
| 10. | Disconnect scope from DUT |  |  |  |  |
| 11. | Disconnect SMU from DUT |  |  |  |  |
| Notes |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |
| M0133 | T0014_M0133_cap_ctrl_0_ldiv_frequency_range | kHz |  |  |  |
| M0134 | T0014_M0134_cap_ctrl_0_ldiv_frequency_mean | MHz |  |  |  |
| M0135 | T0014_M0135_cap_ctrl_31_ldiv_frequency_range | kHz |  |  |  |
| M0136 | T0014_M0136_cap_ctrl_31_ldiv_frequency_mean | MHz |  |  |  |
| M0137 | T0014_M0137_cap_ctrl_63_ldiv_frequency_range | kHz |  |  |  |
| M0138 | T0014_M0138_cap_ctrl_63_ldiv_frequency_mean | MHz |  |  |  |

## T0015: PLL Lock Test

Table 32: T0015: PLL Lock Test Overview

| Test Description |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| This test will check the PLL closed loop behaviour. During this test the key parameters of the locked PLL are observed. |  |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |  |
| An oscilloscope with: - Frequency measurement range: 20 MHz - Frequency measurement accuracy: 10 kHz An SMU with: - Voltage measurement range: 2 V - Voltage measurement accuracy: 2 mV |  |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |  |
| No deviations from chapter 2 applies to this test. |  |  |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |  |  |
| This test will be executed in an automated way and in a temperature controlled environment |  |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |  |
| Step | Action |  |  | Register parameter |  | Value |  |
|  | Park state |  |  |  |  |  |  |
|  | Configure scope for frequency measurement |  |  | timebase.scale |  | 50 ns |  |
|  |  |  |  | trigger.channel |  | 1 |  |
|  |  |  |  | trigger.level |  | 1.65 |  |
|  |  |  |  | channel.measure.clear |  | 1 |  |
|  |  |  |  | channel.offset |  | 0 |  |
|  |  |  |  | channel.probe.attenuation |  | 1 |  |
|  |  |  |  | channel.scale |  | 1 |  |
|  | Connect vtst_vtune on test mux on gpio_1 |  |  |  |  |  |  |
|  | Connect DMM to gpio_1 |  |  |  |  |  |  |
|  | Measure the v_tune voltage with the DMM |  |  |  |  |  |  |
|  | Connect ldiv_out_tst on test mux on gpio_2 |  |  |  |  |  |  |
|  | Connect scope to gpio_2 |  |  |  |  |  |  |
|  | Measure the ldiv output frequency with the scope |  |  | channel.measure.counter |  |  |  |
|  | Read PLL Lock bits |  |  | Analog_ct_lock_bits |  |  |  |
|  | Read coarse tune cap bank setting |  |  | Analog_ct_test_val |  |  |  |
|  | Disconnect scope from DUT |  |  |  |  |  |  |
|  | Disconnect SMU from DUT |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |  |  |
| ID | Name | Unit | LTL |  | Typ |  | UTL |
| M0139 | T0015_M0139_ldiv_frequency | MHz |  |  | 10 |  |  |
| M0140 | T0015_M0140_v_tune | mV |  |  |  |  |  |
| M0141 | T0015_M0141_coarse_control | # |  |  |  |  |  |
| M0142 | T0015_M0142_lock_bits | # |  |  |  |  |  |

## T0016: IO SPI Slave Test

Table 33: T0016: IO SPI Slave Test Overview

| Test Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| This test will check the debug spi slave interface. This test will check the debug spi slave interface by trying to read and write a register. |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |
| Controllable Master SPI peripheral compatible for this ASIC’s 96bit transactions. |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |
| No deviations from chapter 2 applies to this test. |  |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |  |
| This test will be executed in an automated way and in a temperature controlled environment |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |
| Step | Action |  |  | Register parameter |  | Value |
|  | Park state |  |  |  |  |  |
|  | Read register |  |  | Analog_constant_test |  |  |
|  | Check if read value matches |  |  | Analog_constant_test |  | 0xAA |
|  | Write register |  |  | VendorUniqueSPIGPIO_data_mosi |  | 0xCAFE1234 |
|  | Read register |  |  | VendorUniqueSPIGPIO_data_mosi |  |  |
|  | Check if read value matches |  |  |  |  | 0xCAFE1234 |
| Notes |  |  |  |  |  |  |
| The register VendorUniqueSPIGPIO_data_mosi is chosen because when running this test, the gpio spi master will not be affected as long as the trigger register is not written. Therefore it’s a harmless setting. |  |  |  |  |  |  |
| Measurements – Acceptance criteria |  |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |
| M0143 | T0016_M0143_debug_spi_slave_write_success | # | 1 | 1 | 1 |
| M0144 | T0016_M0144_debug_spi_slave_read_success | # | 1 | 1 | 1 |

## T0017: IO SPI Master CAM Test

Table 34: T0017: IO SPI Master CAM Test Overview

| Test Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| This test will check the camera SPI master interface. This test will check the camera SPI master interface by trying to read and write a register in FPGA which acts as the SPI slave of a camera chip. |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |
| FPGA with emulated Camera SPI Slave |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |  |
| This test will be executed in an automated way and in a temperature controlled environment |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |
| Step | Action |  | Register parameter |  | Value |  |
|  | Park state |  |  |  |  |  |
|  | Lower SPI Master clock frequency to improve signal integrity due to IO cell driving limits. |  | VendorUniqueBootstrapFSM_ spi_master_clock_divider |  | 10 |  |
|  | Connect FPGA to camera SPI master interface |  |  |  |  |  |
|  | Configure read data in the FPGA's SPI slave |  |  |  | 0xCAFE1234 |  |
|  | Read a register in the camera through the CXP interface |  | ReverseXReg |  |  |  |
|  | Check if read value matches |  | ReverseXReg |  | 0xCAFE1234 |  |
|  | Write a register in the camera through the CXP interface |  | ReverseXReg |  | 0x600D_C0DE |  |
|  | Get the written from the FPGA's SPI slave |  |  |  |  |  |
|  | Check if value matches with the written data |  |  |  | 0x600D_C0DE |  |
|  | Disconnect the FPGA from the camera SPI master interface |  |  |  |  |  |
| Notes |  |  |  |  |  |  |
| Measurements – Acceptance criteria |  |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |
| M0145 | T0017_M0145_camera_spi_master_write_success | # | 1 | 1 | 1 |
| M0146 | T0017_M0146_camera_spi_master_read_success | # | 1 | 1 | 1 |

## T0018: IO SPI Master GPIO Test

Table 35: T0018: IO SPI Master GPIO Test Overview

| Test Description |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| This test will check the GPIO SPI master interface. This test will check the GPIO SPI master interface by trying to read and write a register in FPGA which acts as a generic SPI slave. |  |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |  |
| FPGA with emulated generic SPI slave. |  |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |  |
| No deviations from chapter 2 applies to this test. |  |  |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |  |  |
| This test will be executed in an automated way and in a temperature controlled environment |  |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |  |
| Step | Action | Register parameter |  |  |  | Value |  |
|  | Park state |  |  |  |  |  |  |
|  | Configure the GPIOs to forward the GPIO SPI Master on GPIO 10, 9, 8 and 7. | PROX_IOLineModeAll_Value PROX_VendorUniqueSPIGPIO_spi_enable PROX_IOLineSelector_Value PROX_IOLineSource_Value PROX_IOLineSelector_Value PROX_IOLineSource_Value PROX_IOLineSelector_Value PROX_IOLineSource_Value PROX_IOLineSelector_Value PROX_IOLineSource_Value PROX_VendorUniqueSPIGPIO_ssel |  |  |  | 1023 1 10 32 9 32 8 32 7 32 0 |  |
|  | Configure the GPIO SPI Master to be compatible with the SPI Slave in the FPGA. - f_spi = 125MHz/2*(divider+1) - Transaction size of 64bits: 31 Address, 1 WNR, 32 data. - Slave select 0 -> GPIO 7 | PROX_VendorUniqueSPIGPIO_transaction_size PROX_VendorUniqueSPIGPIO_divider |  |  |  | 63 67 |  |
|  | Connect FPGA to DUT gpio spi master interface |  |  |  |  |  |  |
|  | Configure read data in the FPGA's SPI slave |  |  |  |  | 0xCAFE1234 |  |
|  | Read a register from the FPGA's SPI slave through the GPIO SPI master |  |  |  |  |  |  |
|  | Check if read value matches |  |  |  |  | 0xCAFE1234 |  |
|  | Write a register in the FPGA's SPI slave through the GPIO SPI master |  |  |  |  | 0x600D_C0DE |  |
|  | Get the written from the FPGA's SPI slave |  |  |  |  |  |  |
|  | Check if value matches with the written data |  |  |  |  | 0x600D_C0DE |  |
|  | Disconnect the FPGA from the GPIO SPI master interface |  |  |  |  |  |  |
| Notes |  |  |  |  |  |  |  |
| Measurements – Acceptance criteria |  |  |  |  |  |  |  |
| ID | Name |  | Unit | LTL | Typ |  | UTL |
| M0147 | T0018_M0147_gpio_spi_master_write_success |  | # | 1 | 1 |  | 1 |
| M0148 | T0018_M0148_gpio_spi_master_read_success |  | # | 1 | 1 |  | 1 |

## T0019: Eye Diagram Test

TODO

## T0020: CXP Uplink Receiver Sensitivity Test

Table 36: T0020: CXP Uplink Receiver Sensitivity Test Overview

| Test Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| This test will check the receiver sensitivity on the default setting. In this test the CXP interface will be connected to the function generator which generates a sequence to write a register in the DUT. The amplitude of the sequence will be modified in a binary search to obtain the receiver sensitivity voltage. |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |
| Function generator is connected with DUT instead of the frame grabber. |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |
| A function generator with: - Arbitrary waveform bit rate: 20.833 Mbps - Arbitrary waveform voltage range: 180 mVpp (according to the CXP standard [3]) - Arbitrary waveform voltage accuracy: 100 uV - Arbitrary waveform memory: Enough to hold waveform for the uplink command. |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |
| No deviations from chapter 2 applies to this test. |  |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |  |
| This test will be executed in an automated way and in a temperature controlled environment |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |
| Step | Action | Register parameter | Value |  |  |  |
| 1. | Park state |  |  |  |  |  |
| 2. | Connect function generator to DUT trx |  |  |  |  |  |
| 3. | Configure the function generator to output the register write arbitrary waveform | Arbitrary Waveform | CXP_10b_stuffed_complete_sequence.arb |  |  |  |
| 4 | Configure the function generator to output the to maximum amplitude for the waveform |  | 100 mV |  |  |  |
| 5 | Enable the function generator output and wait 2.5 seconds |  |  |  |  |  |
| 6 | Check if the register is written | IIDC2EntryImager_VendorUniqueControl5 | 0xCAFE1234 |  |  |  |
| 7 | Update the function generator voltage based on the outcome of the read result. When the read is successful, lower the voltage. If not increase the voltage. |  |  |  |  |  |
| 8 | Disable the function generator output. |  |  |  |  |  |
| 9 | Reset the register | IIDC2EntryImager_VendorUniqueControl5 | 0 |  |  |  |
| 10 | Repeat from 5 until the measurement is within the tolerance. | Tolerance | 1 mV |  |  |  |
| 11 | Disconnect the function generator from DUT trx |  |  |  |  |  |
| Notes |  |  |  |  |  |  |
| The parameters evaluated in this test are invariably impacted by the PCB layout and the components selected for this section of the circuit. Solely assessing ASIC performance based on these RF measurements is not feasible. Demonstrating that specifications are met through effective design should be adequate for these evaluations. Cables to be used ([con1] cable type [con2] (art nr)): - 1 meter: [HD-BNC-M] Belden-1855A [BNC-F] (095-850-130M100) Adapter pieces to be used: - 1x [BNC-F][BNC-F] (112446-12G) - 1x [BNC-M][BNC-M] (UG 491B/U – R141703) - 1x Impedance transformer (Z7550-BMBF+) |  |  |  |  |  |  |
| Measurements – Acceptance criteria |  |  |  |  |  |  |
| ID | Name |  | Unit | LTL | Typ | UTL |
| M0149 | T0020_M0149_trim_hyb_gain_0_voltage |  | V |  |  |  |

## T0021: CXP Uplink Receiver Sensitivity Sweep Test

Table 37: T0021: CXP Uplink Receiver Sensitivity Sweep Test Overview

| Test Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| This test will check the receiver sensitivity on each setting for the hybrid gain. In this test the CXP interface will be connected to the function generator which generates a sequence to write a register in the DUT. The amplitude of the sequence will be modified in a binary search to obtain the receiver sensitivity voltage. |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |
| Function generator is connected with DUT instead of the frame grabber. |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |
| See T0020 |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |
| No deviations from chapter 2 applies to this test. |  |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |  |
| This test will be executed in an automated way and in a temperature controlled environment |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |
| Step | Action |  | Register parameter |  | Value |  |
|  | Park state |  |  |  |  |  |
|  | Connect function generator to DUT trx |  |  |  |  |  |
|  | Start of hybrid gain loop |  |  |  |  |  |
| 3.1 | Configure the first (next) trim setting |  | Analog_trim_hyb_gain |  | 0..7 |  |
| 3.2 | Perform the sequence from T0019 |  |  |  |  |  |
| 3.3 | Go to start 3 in case more trim settings remain |  |  |  |  |  |
| 4 | Disconnect function generator from DUT trx |  |  |  |  |  |
| Test parameters and acceptance criteria |  |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ |  | UTL |
| M0157 | T0021_M0157_trim_hyb_gain_0_voltage | V |  |  |  |  |
| M0158 | T0021_M0158_trim_hyb_gain_1_voltage | V |  |  |  |  |
| M0159 | T0021_M0159_trim_hyb_gain_2_voltage | V |  |  |  |  |
| M0160 | T0021_M0160_trim_hyb_gain_3_voltage | V |  |  |  |  |
| M0161 | T0021_M0161_trim_hyb_gain_4_voltage | V |  |  |  |  |
| M0162 | T0021_M0162_trim_hyb_gain_5_voltage | V |  |  |  |  |
| M0163 | T0021_M0163_trim_hyb_gain_6_voltage | V |  |  |  |  |
| M0164 | T0021_M0164_trim_hyb_gain_7_voltage | V |  |  |  |  |

## T0022: Internal Test Patterns Test

Table 38: T0022: Internal Test Patterns Test Overview

| Test Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| This test will check the test pattern generator in the DUT. The test will loop over test patterns and capture them with the frame grabber. |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |
| Frame grabber connected to Serializer ASIC’s substrate CXP connector. |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |
| No deviations from chapter 2 applies to this test. |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |
| This test will be executed in an automated way and in a temperature controlled environment |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |
| Step | Action |  |  | Register parameter | Value |
|  | Park state |  |  |  |  |
|  | Enable color mode of the TPG to be able to capture color pattern. |  |  | Analog_col_mono_tpg | 1 |
|  | Start of video mode loop |  |  |  |  |
| 3.1 | Configure the first (next) video mode |  |  | Video mode table |  |
| 3.2 | Start of test pattern loop |  |  |  |  |
| 3.2.1 | Configure the first (next) test pattern |  |  | Test pattern table |  |
| 3.2.2 | Start of pixel size loop |  |  |  |  |
| 3.2.2.1 | Configure the first (next) pixel size |  |  | PixelSize_Value | 8,10,12 |
| 3.2.2.2 | Initialize connection between frame grabber and DUT. |  |  |  |  |
| 3.2.2.3 | Capture image |  |  |  |  |
| 3.2.2.4 | Check amount of errors in image when compared to the reference image. |  |  |  |  |
| 3.2.2.5 | If image contains errors, save the image to disk. |  |  |  |  |
| 3.2.2.6 | End of pixel size loop, go to 3.2.2 if more pixel sizes remain. |  |  |  |  |
| 3.2.3 | End of test pattern loop, go to 3.2 if more test patterns remain. |  |  |  |  |
| 3.3 | End of video mode loop, go to 3.2 if more video modes remain. |  |  |  |  |
| Notes |  |  |  |  |  |
| Video mode table Video mode; Register parameter; Value. VGA; ImageSize_Width; 640. ImageSize_Height; 480. HD; ImageSize_Width; 1280. ImageSize_Height; 720. VGA; ImageSize_Width; 1920. ImageSize_Height; 1080. Test Pattern table Pattern; Register parameter; value. Black; TestPattern_Value; 1. White; TestPattern_Value; 2. Grey55; TestPattern_Value; 3. GreyAA; TestPattern_Value; 4. GrayHorizontalRamp; TestPattern_Value; 16. GrayVerticalRamp; TestPattern_Value; 17. ColorBar; TestPattern_Value; 48. GreyScaleChart; TestPattern_Value; 49. VendorSpecific0; TestPattern_Value; 120. VendorSpecific1; TestPattern_Value; 121. VendorSpecific2; TestPattern_Value; 122. VendorSpecific3; TestPattern_Value; 123. VendorSpecific4; TestPattern_Value; 124. VendorSpecific5; TestPattern_Value; 125. |
| Measurements – Acceptance criteria |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |
| M0155 | T0022_M0155_pixel_errors | # | 0 | 0 | 0 |

## T0023: Camera Test

Table 39: T0023: Camera Test Overview

| Test Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Send and capture the test patterns of an external camera (test pattern generator in the FPGA) and compare their raw value with a pre saved correct test pattern. |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |
| In this test the FPGA connects to the video interface and emulates a camera. In this test different timings and video modes can be checked. |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |
| Frame grabber connected to Serializer ASIC’s substrate CXP connector and FPGA with test pattern generator. |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |  |
| No deviations from chapter 2 applies to this test. |  |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |  |
| This test will be executed in an automated way and in a temperature controlled environment |  |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |
| Step | Action |  |  | Register parameter | Value |  |
|  | Park state |  |  |  |  |  |
|  | Connect FPGA test pattern generator to DUT |  |  |  |  |  |
|  | Configure FPGA test pattern generator timings to mimic megacam as close as possible for FHD 25 fps. |  |  | f_rise_to_l_rise_delay | 2 |  |
|  |  |  |  | l_fall_to_f_fall_delay | 0 |  |
|  |  |  |  | l_rise_to_l_rise_delay | 1570 |  |
|  |  |  |  | pixel_clock_frequency | 100e6 |  |
|  |  |  |  | frame_rate | 25 |  |
|  | Start of video mode loop |  |  |  |  |  |
| 8.1 | Configure the first (next) video mode in DUT |  |  | Video mode table |  |  |
| 8.2 | Configure the video mode in the FPGA test pattern generator and DUT |  |  |  | Video mode table |  |
| 8.3 | Start of test pattern loop |  |  |  |  |  |
| 8.3.1 | Configure the first (next) test pattern in the FPGA test pattern generator. |  |  |  |  |  |
| 8.3.2 | Start of pixel size loop |  |  |  |  |  |
| 8.3.2.1 | Configure the first (next) pixel size |  |  | PixelSize_Value | 8,10,12 |  |
| 8.3.2.2 | Configure the pixel size in the FPGA test pattern generator and DUT |  |  |  |  |  |
| 8.3.2.3 | Initialize connection between frame grabber and DUT. |  |  |  |  |  |
| 8.3.2.4 | Capture image |  |  |  |  |  |
| 8.3.2.5 |  | Check amount of errors in image |  |  |  |  |
| 8.3.2.6 |  | If image contains errors, save the image to disk. |  |  |  |  |
| 8.3.2.7 | End of pixel size loop, go to 8.3.2 if more pixel sizes remain. |  |  |  |  |  |
| 8.3.3 | End of test pattern loop, go to 8.3 if more test patterns remain. |  |  |  |  |  |
| 8.4 | End of video mode loop, go to 8 if more video modes remain. |  |  |  |  |  |
| Notes |  |  |  |  |  |  |
| This test was written that the FPGA doesn’t emulate a real register map, therefore it’s required that all image related settings are configured and aligned manually between the emulated camera in the FPGA and the settings in Proxima itself. If needed, a full compliant register map could be implemented in the FPGA but this requires more effort. Video mode table Video mode; Register parameter; Value. VGA; ImageSize_Width; 640. ImageSize_Height; 480. Pixel clock frequency; 100 MHz. HD; ImageSize_Width; 1280. ImageSize_Height; 720. Pixel clock frequency; 100 MHz. FHD; ImageSize_Width; 1920. ImageSize_Height; 1080. Pixel clock frequency; 100 MHz. Pattern table Pattern. Picture 1. . |
| Measurements – Acceptance criteria |  |  |  |  |  |  |
| ID |  | Name | Unit | LTL | Typ | UTL |
| M0156 |  | T0023_M0156_pixel_errors | # | 0 | 0 | 0 |

## T0024: OTP Write Test

Table 40: T0024: OTP Write Test Overview

| Test Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Write data into the OTP cells. Verify that the write was successful. |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |
| The pin VDD_OTP will be connected to SMU to generated a supply for the OTP macro. Important to keep the duration of the supply short. |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |
| Voltage source on VDD2V5_OTP with slope control of 5ms from 0V to 2.5V+/-10% with CC of 50mA. |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |
| No deviations from chapter 2 applies to this test. |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |
| This test will only be executed at room temperature once at the end of the trim flow. |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |
| Step | Action |  | Register parameter |  | Value |
|  | Start from park state, this also loads the trim values. |  |  |  |  |
|  | Check if the OTP is not yet programmed. If not pass stop the test. |  | otp_programmed_state |  | 0x0 |
|  | Check if the PLL is locked (PLL needs to be running and locked to successfully write OTP registers) |  | Analog_ct_lock_bits |  | Read 0x02 or 0x03 |
|  | Set the command to go to pre-program state. |  | otp_cmd otp_cmd_valid |  | 0x9 0x1 |
|  | Write programmed state bit to 1 |  | otp_programmed_state |  | 0x1 |
|  | Check if programmed state bit register, is successfully written. |  | otp_programmed_state |  | 0x1 |
|  | If needed, configure additional settings to be stored in OTP on top of the already loaded trim settings. |  | Analog_trim_hyb_gain |  | 0 |
|  | For safety, turn off SMU before connecting to DUT. |  |  |  |  |
|  | Wait 100 ms. |  |  |  |  |
|  | Connect SMU to pin VDD2V5_OTP. |  |  |  |  |
|  | Configure the SMU for the vdd2v5_otp pin |  | Mode Voltage level Current limit |  | Voltage 2.5 V 50 mA |
|  | Set the command to go to program state. |  | otp_cmd |  | 0x5 |
|  | Ramp up the voltage on pin VDD2V5_OTP from 0.0 towards 2.5V in 5ms. |  |  |  |  |
|  | Wait 100 ms. |  |  |  |  |
|  | Trigger the OTP FSM to execute the command (this bit automatically goes back to 0). |  | otp_cmd_valid |  | 0x1 |
|  | Wait 100ms (while OTP is being programmed) |  |  |  |  |
|  | Ramp down the voltage on pin VDD2V5_OTP from 2.5V towards 0V in 5ms |  |  |  |  |
|  | Turn off and disconnect the SMU from DUT. |  |  |  |  |
|  | Read the OTP error flag, expected value is 0x0. Log as ‘otp_error’ |  | otp_error |  | Read |
|  | Power off DUT |  |  |  |  |
|  | Park state |  |  |  |  |
|  | Check programmed state of OTP |  | otp_programmed_state |  |  |
|  | Read all registers of the type “CONFIGURATION_OTP” again and verify that all the changes are correct. Log as ‘otp_check’. |  | otp_check |  |  |
| Notes |  |  |  |  |  |
| Important note on the VDD2V5_OTP voltage: maximum accumulative time for the entire macro exposed under 2.5V+/-10% should be less than 1 second. WARNING: if this test is conducted via CXP instead of SPI communication be aware that the Frame grabber caching option must be off to avoid incorrect verification of the written registers. |  |  |  |  |  |
| Measurements – Acceptance criteria |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |
| M0151 | T0024_M0151_otp_error | # | Pass/fail |  |  |
| M0152 | T0024_M0152_otp_programmed_state | # | Pass/fail |  |  |
| M0153 | T0024_M0153_otp_check | # | Pass/fail |  |  |

## T0025: OTP Data Retention Test

Table 41: T0025: OTP Data Retention Test Overview

| Test Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Read the data from OTP. Verify that no bits are flipped. |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |
| Test Conditions |  |  |  |  |
| No deviations from chapter 2 applies to this test. |  |  |  |  |
| Environmental conditions |  |  |  |  |
| This test will be executed in an automated way and in a temperature controlled environment |  |  |  |  |
| Test Procedure |  |  |  |  |  |
| Step | Action |  | Register parameter |  | Value |
|  | Start from park state |  |  |  |  |
|  | Read the otp_programmed_state bit in the register map. Log the value. |  | otp_programmed_state |  |  |
| Notes |  |  |  |  |  |
| Measurements – Acceptance criteria |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |
| M0154 | T0025_M0154_otp_programmed_state | # | 1 | 1 | 1 |

## T0026: VIS VDD3V3_DCDC test

Table 42: T0026: VIS VDD3V3_DCDC Test Overview

| Test Description |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| This test checks the output voltage on the VIS modules form the DCDC converter. |  |  |  |  |  |  |
| Test Configuration |  |  |  |  |  |  |
| In this test a vision module is plugged in to the base board instead of the socket board. Because of this the following adjustments: - All connections between the power supply and base board need to be removed. - Connect PSU 1 channel 2 to the base board jack labelled 9V - Disconnect the DMM from the SMU Sense Jack - Connect the DMM to VDD3V3_DIG_SENSE |  |  |  |  |  |  |
| Test Equipment Requirements |  |  |  |  |  |  |
| A DMM with: - Voltage measurement range 3.6 V - Voltage measurement accuracy: 10 mV |  |  |  |  |  |  |
| Test Conditions |  |  |  |  |  |
| No deviations from chapter 2 applies to this test. |  |  |  |  |  |
| Environmental conditions |  |  |  |  |  |
| This test will be executed in an automated way and in a temperature controlled environment. Before this test is part of the VIS module tests, some connections between base board and measurement equipment need to be changed prior to running this test. |  |  |  |  |  |
| Test Procedure |  |  |  |  |  |  |
| Step | Action |  | Register parameter |  | Value |  |
|  | Start from park state |  |  |  |  |  |
|  | Connect DMM to VDD3V3_DIG_SENSE |  |  |  |  |  |
|  | Measure VDD3V3_DCDC with DMM |  |  |  |  |  |
|  | Disconnect DMM from VDD3V3_DIG_SENSE |  |  |  |  |  |
| Notes |  |  |  |  |  |  |
| Measurements – Acceptance criteria |  |  |  |  |  |  |
| ID | Name | Unit | LTL | Typ | UTL |  |
| M0180 | T0026_M0180_dcdc_output_voltage | # | 2.97 | 3.3 | 3.63 |  |

# Bibliografie

| [1] | M. Technologies, „Overview of PRODUCTS available in the VISION SERIES,” Magics Technologies, 10 10 2025. [Online]. Available: https://magicstech.sharepoint.com/sites/Public-Product/SitePages/Vision-Series.aspx. [Geopend 10 10 2025]. |
| --- | --- |
| [2] | Magics, MAG-CXP00002-NP CoaXPress Serializer ASIC Datasheet, Geel, 2025/05-rev2.0. |
| [3] | Japan Industrial Imaging Association (JIIA), „CoaXPress Version 2.0 JIIA CXP-001-2019,” 2019. |

List of Acronyms

| Acronym | Definition |
| --- | --- |

Idm@F4E #